3GPP TSG-RAN WG4 Meeting #114 	R4-2502084

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item: 7.23.3.2

Source: MediaTek inc.

Title: Discussion on SBFD Operation

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion on the RRM impact due SBFD operation

In SBFD operation UE remains a half-duplex node. This implies that in SBFD symbols, the UE can perform either DL reception (on DL subbands) or UL transmission (on UL subband), but not both. To avoid collisions between DL receptions and UL transmissions, the network must provide a way for UE to determine the link direction on SBFD symbols, i.e., whether the UE transmit or receive in a SBFD symbol. Therefore, several issues were brought up to the RRM discussion by multiple companies, which are captured in the WF.

Issue 1-2-2: Requirements for CSI-RS based L1 measurement:

- Agreements (from online session) - For CSI-RS colliding with UL transmission - For aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission. - For periodic and semi-persistent CSI-RS resource, RAN4 will send RAN1 LS for clarification if needed. - For CSI-RS puncturing in frequency domain in SBFD symbols, - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs. - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs. - FFS for the DUD configuration. - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions. - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold. - FFS the measurement behavior for DUD configuration. - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement. - For CSI-RS distributed over SBFD and non-SBFD symbols, - Ask RAN1 clarification on whether related RAN1 agreement applies for L1 measurement (L1-RSRP/SINR measurement and RLM/BFD/CBD measurement). Further discuss the wording when drafting the LS.

RAN4 RRM has discussed the issues related to ‘CSI-RS based L1 measurement’ and based on that RAN4 identified three main sub-issues:

- CSI-RS colliding with UL transmission

- CSI-RS puncturing in frequency domain in SBFD symbols

  - Requirements for DU configuration

  - Requirements for DUD configuration

- CSI-RS distributed over SBFD and non-SBFD symbols

For the issues of (i) CSI-RS colliding with UL transmission and (ii) CSI-RS distributed over SBFD and non-SBFD symbols, RAN4 RRM sent an LS to RAN1 to ask for further clarification and hence RAN4 should wait for the LS response. In addition, regarding the issue of ‘CSI-RS puncturing in frequency domain in SBFD symbols’ RAN4 discussion differentiate the scenario of DU configuration compared to DUD configuration. Based on RAN4 discussion, the existing RRM requirement can be extended to cover the scenario of DU configuration, however, DUD configuration requires further study as agreed in the previous meeting. Figure 1 depicts the configuration of DU versus DUD. The FFS points are:

- FFS for the DUD configuration;

- FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions;

- FFS the measurement behavior for DUD configuration.

Figure 1: DUD SBFD configuration vs DU SBFD configuration.

One of the main reason for further study on DUD configuration is that first the UE can’t process both DL subbands at the same time and second the DL subbands are exposed to more interference compared to the legacy or DU scenarios. However, it is not straight forward to do simple calculation to judge on what is the interference level for each DL subbands and whether there is a need for extending the number of PRBs or relax the accuracy requirements. Therefore, to assess whether to extend the minimum number of CSI-RS PRB, RAN4 should conduct simulation to obtain the measurements performance for the DUD configuration. Then RAN4 to define requirements according to the simulation performance.

Observation 1: The RRM requirements for CSI-RS based measurements are defined by the minimum number of PRBs of CSI-RS resources used for the measurements, which is derived using simulation performance.

- For SBFD-aware UE, NW shall configure the UE with the punctured CSI-RS resources by the SBFD subband operation.

- For DUD configuration, RAN4 to conduct simulation to assess whether there is a need to update the CSI-RS based measurements based on the minimum number of CSI-RS resources.

Issue 1-2-3: Requirements for CSI-RS based L3 measurement:

- Agreements (from online session) - Companies to check the existing requirement, and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed.

With the current existing measurement object (MO) configuration, a UE can’t differentiate whether the neighbouring cells are operating with DU/DUD configuration, also, the UE doesn’t know the exact location of two subband in DUD configuration. Therefore, CSI-RS based L3 measurement should be deprioritized.

Observation 2: The existing MO configures a UE with the location of measurement resources using the starting point and length of that measurement’s resources.

- RAN4 shall deprioritize CSI-RS based L3 measurements.

Issue 1-2-4: Requirements for scheduling restriction:

- Agreements (from Wed AH session) - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction. - Further discuss the impact to inter-frequency SSB measurement without gap case.

On the impact to inter-frequency SSB measurement without gap case, the existing scheduling restriction of TDD bands in RRM requirements should be applicable to the SBFD operation.

- The existing scheduling restriction of TDD bands in RRM requirements should be applicable to the SBFD operation.

Issue 1-2-5: Requirements for RACH requirements:

Based on our observation, we didn’t identify any requirements impact due to the SBFD operation, yet, we are fine with other companies suggestion to check whether there is a need to define test case during the performance part.

- there is no impact on RACH requirements due to SBFD operation.

Issue 1-2-6: Requirements for UL PC and or spatial relation update:

It is not clear to us what UL power control is referred to it in here, besides, we don’t see any impact on spatial relation update. Our preference is to close this issue without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals.

- RAN4 RRM should close this issue of UL PC and spatial relation update without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals.

Issue 1-2-7: Requirements for MG and BWP switch:

The wording of SBFD can be for both UL/DL so at the end all what we need is to keep the requirements as UL slots regardless of whether they are SBFD-UL or TDD-UL.

- There is no need to update the term UL slot for MG and BWP switch.

Issue 1-2-8: Requirements for UL resource muting:

It is not clear which existing RRM requirements that could be impacted or not for UL resource muting. Our preference is to close this issue without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals.

- RAN4 RRM should close this issue of UL resource muting without agreement on whether there is impact or not. If companies identify there is impact in the future, then companies can bring further proposals.

# Summary

In this contribution, we have the following proposals:

Observation 1: The RRM requirements for CSI-RS based measurements are defined by the minimum number of PRBs of CSI-RS resources used for the measurements, which is derived using simulation performance.

Observation 2: The existing MO configures a UE with the location of measurement resources using the starting point and length of that measurement’s resources.

Proposal 1: For SBFD-aware UE, NW shall configure the UE with the punctured CSI-RS resources by the SBFD subband operation.

Proposal 2: For DUD configuration, RAN4 to conduct simulation to assess whether there is a need to update the CSI-RS based measurements based on the minimum number of CSI-RS resources.

Proposal 3: RAN4 shall deprioritize CSI-RS based L3 measurements.

Proposal 4: The existing scheduling restriction of TDD bands in RRM requirements should be applicable to the SBFD operation.

Proposal 5: there is no impact on RACH requirements due to SBFD operation.

Proposal 6: RAN4 RRM should close this issue of UL PC and spatial relation update without agreement on whether there is impact or not. If companies identify there is impact in the future then companies can bring further proposals.

Proposal 7: There is no need to update the term UL slot for MG and BWP switch.

Proposal 8: RAN4 RRM should close this issue of UL resource muting without agreement on whether there is impact or not. If companies identify there is impact in the future, then companies can bring further proposals.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2420097, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Orlando, US, Nov 2024.