3GPP TSG-RAN WG4 Meeting # 114						R4-2502094

Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			On system parameter of WUR

Agenda item:		7.26.2.1

Document for:		Approval

# Introduction

In this paper, we present our overview on system parameter for WUR.

# Discussion

In WF [1], there are several requirements which are FFS. For the operation bands, we think there is a need to specify as the WUR bands as there are some bands to be excluded, e.g the SUL and SDL bands. There are also bands associate with certain feature, e.g the V2X bands n47.

Specify the operation bands for WUR.

For channel raster and channel bandwidth, the WUR is not used in the cell search and the configuration of WUR is specified in RAN2 by below agreement that the LP-WUS will be configured in system information for RRC idle and inactive and this means UE can locate the LP-WUS when the SIB is obtained. In mobility scenario, UE needs to re-select the cell using MR and therefore, there is no frequency raster would be configured to WUR separately.

There is no frequency raster would be configured to WUR separately, WUR is not used in cell search and in mobility scenario, cell re-selection sill be done by MR.

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Entry/exit condition

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

-	LP-SS configuration

-	LP-WUS configuration

-	Entry/exit condition for LP-WUS monitoring (FFS if it is always configured)

Issue 1-1-4: views on which requirements should be updated in core spec

Agreements:

  - The part should be updated for LP-WUS/WUR in spec.

    - The additional requirements are not precluded, if needed.

| Sections or requirements | Needed or not? |
| --- | --- |
| Operation bands | FFS |
| UE channel bandwidth | FFS |
| Channel raster | FFS |
| REFSENS | YES |
| Maximum input level | YES |
| ACS | YES |
| ASCS | YES |
| In-band blocking | YES |
| Out-of-band blocking | YES |
| Narrow band blocking | YES |
| Spurious response | FFS |
| Wide band Intermodulation | FFS |
| Spurious emissions | FFS |
| Annex A Measurement channels | YES |

    - Sub-topic 1-2 System parameters

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

  - No new channel raster is introduced dedicated for LP-WUS/WUR.

    - FFS on whether there is need of the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost

# Conclusions

In this contribution, we present our overview on general view of WUR with below proposal:

Proposal-1: Specify the operation bands for WUR.

Observation 1 There is no frequency raster would be configured to WUR separately, WUR is not used in cell search and in mobility scenario, cell re-selection sill be done by MR.

# References

- R4-2420528, WF on UE RF requirements for LP-WUS, Vivo