# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502103

# Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			RedCap UE RF impact on HD-FDD

Agenda item:		7.29.2.1

Document for:		Approval

# Introduction

In this paper, we present our views on the RedCap UE RF remaining issue.

# Discussion

In WF[1], the remaining issue is below:

## Issue 1-1: PC3 output power for (e)RedCap

Agreement:

- FFS whether and how a potential new scheme to increase output power is addressed in specifications

  - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap

    - If confirmed, no further work is needed.

  - Alt 2: Power increase by network controlled optional new signalling (e.g capability)

  - Alt 3: Change in power tolerances

    - E.g. Upper tolerance exception to UE in specific regions via table note.

  - FFS whether solution is common for HD-FDD and FD-FDD

  - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB.

Rel-18 coverage enhancement work item introduced the power boosting feature for PC2 and PC3. It has introduced two capabilities (powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18). The difference of these two capabilities is that whether the spectrum flatness requirements would be relaxed compared to the legacy spectrum flatness requirements. For UE supporting capability of powerBoosting-pi2BPSK-QPSK-r18, the enhanced inner region (reduced RB ranged within the legacy inner region), no need to relax the spectrum flatness while for UE supporting powerBoosting-pi2BPSK-QPSK-Modified-r18, the spectrum flatness is relaxed and tested against the clause 6.4.2.4.2. No matter which capabilities will be supported by Rel-18 NR UE, either legacy RB inner region or enhanced inner region will be applied and outer RB region is excluded in the power boosting feature.  Also the Rel-18 coverage enhancement is generic feature and it is possible for (e) RedCap UE to support based on capabilities reporting.

For UE supporting Rel-18 power boosting capabilities only inner or enhanced inner region apply and outer region is excluded for power boosting.

Rel-18 (e) RedCap UE can support the Rel-18 power boosting feature.

From the above discussion, a Rel-19 NTN (e) RedCap UE can also support the Rel-18 power boosting feature as the feature is generic for NR UE. Moreover, as the power increase due to the reduction of the insertion loss of the Tx filter, such power boosting is different with Rel-18 power boosting scope where a transparent scheme would be applied, e.g FDSS, peak cancellation, clipping + filtering etc.  Though power increase by reducing the insertion loss of filter can be rendered using the Rel-18 power boosting capabilities, it can also be used independently with the Rel-18 power boosting feature enabled with FDSS. Therefore, a (e)RedCap UE could increase its output power above the power level when such UE also enabling the Rel-18 power boosting feature simultaneously by reducing the filter insertion loss and at the same time retaining the same PA and baseband (e.g FDSS or peak cancellation still apply) if the unwanted emission requirements still be met. For example, for a UE supporting Rel-18 power boosting, the boosted power is 1 dB for PC3 and 0.5 dB for PC2. With insertion loss reduction, the power boosting delta relative to the UE nominal power could be stretched to 1+ X dB for PC3 and 0.5 + Y for PC2 , where X dB for PC3 or Y dB for PC2 is due to the insert loss reduction and it could apply to all modulation scheme which is good property compared to the Rel-18 power boosting which only apply to BPSK/QPSK. On top of this, such power increase (X dB for Pc3 and Y dB for PC2) can apply to all RB allocation without limiting to only inner or enhanced inner region, considering the power boosting mechanism could be different for a (e)RedCap UE. A (e)RedCap UE can support power boosting for outer region with a different power boosting capability on top of Rel-18 power boosting capabilities. As power boosting features are optional features, the below options could be possible for a (e) RedCap UE:

- A (e)RedCap UE operating in HD-FDD mode (indicating the halfDuplexFDD-TypeA-RedCap-r17) supports only Rel-18 power boosting capabilities (powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18)

- A (e)RedCap UE operating HD-FDD mode and new power boosting capability supports only the power boosting by boosting the power with X dB for PC3 and Y dB for PC2.

- A (e)RedCap UE operating HD-FDD mode supports both Rel-18 power boosting capability and Rel-19 power boosting X dB for PC3 and Y dB for PC2 due to insertion loss reduction. The total configured output power could be two options below:

  - Either Rel-18 power boosting (1 dB for PC3 and 0.5 dB for PC2) or Rel-19 power boosting (X dB for PC3 and Y dB for PC2) can be configured, not simultaneously.

  - Both Rel-18 power boosting and Rel-19 power boosting can be configured simultaneously and the total output power boosting could be 1 + X dB for PC3 and 0.5 + Y for PC2.

If the option 3 above would be possible, it should further discuss whether a tightened ALCR compared to PC3 ACLR should apply.

Discuss the (e) RedCap power boosting with the three options listed above.

As this is only for HD-FDD mode, no MSD will be relevant when power is boosted so receiver impact is none. However, if the FD-FDD would be also possible for the power boosting (FD-FDD also can reduce insertion loss of the duplexer), the MSD impact of REFSENS also may need to be investigated. However, as power boosting is enabled by network and MSD issue could follow the Rel-18 power boosting frame work, MSD issue can be ignored, assuming the legacy hardware would apply ( no hardware improving will be needed).

A (e)RedCap UE can support the power boosting with X dB for PC3 and Y dB for PC2 with insertion loss reduction and apply to both outer region and inner region, also apply to other modulation than BPSK/QPSK if unwanted emission would be met.

If power boosting due to the insertion loss reduction would apply both FD-FDD and HD-FDD, no need to specify the MSD for FD-FDD mode.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| powerBoosting-pi2BPSK-QPSK-r18 Indicates whether the UE supports power boosting for DFT-s-OFDM pi/2 BPSK and QPSK without modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1 [2]. The power boosting is only enabled when signalled via powerBoostPi2BPSK-r18 for BPSK and powerBoostQPSK-r18 for QPSK. A UE supporting this feature shall also indicate the support of pusch-HalfPi-BPSK and pucch-F3-4-HalfPi-BPSK. This capability can be supported in any or all scenarios below: - Case 1: FR1 single band with single uplink CC configured in the band where power boosting capability is indicated in this band. - Case 2: FR1 DL CA with a single uplink CC configured in a band where power boosting capability is indicated. The power boosting feature can be configured in this FR1 NR band. - Case 3: FR1 inter-band UL CA/DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured only in one of the bands where capability is indicated. - Case 4: FR1+FR2 UL CA, FR1+FR2 DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured in the FR1 NR band. | FS | No | N/A | FR1 only |
| powerBoosting-pi2BPSK-QPSK-Modified-r18 Indicates whether the UE supports power boosting for DFT-s-OFDM pi/2 BPSK and QPSK with modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1 [2]. The power boosting is only enabled when signalled via powerBoostPi2BPSK-r18 for BPSK and powerBoostQPSK-r18 for QPSK. A UE supporting this feature shall also indicate the support of pusch-HalfPi-BPSK and pucch-F3-4-HalfPi-BPSK. This capability can be supported in any or all scenarios below: - Case 1: FR1 single band with single uplink CC configured in the band where power boosting capability is indicated in this band. - Case 2: FR1 DL CA with a single uplink CC configured in a band where power boosting capability is indicated. The power boosting feature can be configured in this FR1 NR band. - Case 3: FR1 inter-band UL CA/DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured only in one of the bands where capability is indicated. - Case 4: FR1+FR2 UL CA, FR1+FR2 DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured in the FR1 NR band. | FS | No | N/A | FR1 only |

# Release independent features for NR UE supporting satellite access operation

## 9.1	Additional NR operating bands for NR NTN in frequency range 1

Requirements for NR UE supporting satellite access operation, for additional NTN operating bands of TS 38.101-5 in Rel-P [8] are introduced via this clause.

Table 9.1-1: Additional NR operating bands for NR NTN in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| NTN operating bands | FDD | Rel-17 | Table F.1.1-1, Table F.1.2-1, Table F.1.3-1 |

Table 9.1-2: NR NTN UE power class in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 3 | FDD | Rel-17 | Table F.1.1-1 |

Table 9.1-3: NR NTN UE channel bandwidth in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- |
| UE channel bandwidth | FDD | Rel-17 | Table F.1.1-1 |

RAN1 agrees to introduce specfication change on the RedCap on HD-FDD operation, such agreement are listed below:

Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary

For any possible enhancement to the TA report, it is for further study, and it was captured as follows:

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

As the scope of the RAN1 scope is HD-FDD, in the release independent discussion, it can be discussed whether the HD-FDD should be release independent from Rel-19 and other duplex mode should be release independent from the first release, e.g FDD and TDD from Rel-17, HD-FDD from Rel-19.

## 9.2 Additional operating bands for Redcap and eRedCap for NR frequency range 1

Requirements for a Redcap and eRedcap UE for additional operating bands within FR1 of TS 38.101-5 in Rel-P [2] are introduced via this clause.

Table 9.2 -1: NR NTN RedCap operating band

| Feature | Duplex-mode | Release independent from |  | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- | --- |
| Redcap operating band | FDD | Rel-17 |  |  |
|  | TDD | Rel-17 |  |  |
|  | HD-FDD | Rel-19 |  |  |

Table 9.2 -2: NR NTN eRedCap operating band

| Feature | Duplex-mode | Release independent from |  | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- | --- |
| eRedcap operating band | FDD | Rel-18 |  |  |
|  | TDD | Rel-18 |  |  |
|  | HD-FDD | Rel-19 |  |  |

Though it could be argued if the 38.307 should be updated when additional new band is added, this indeed cause confusion for RedCap release independent feature update in 38.307 in later release. We suggest RAN4 discuss this and see if the 38.307 should be added in a updated WID.

RAN4 discuss if the 38.307 should be updated for RedCap NTN as release independent feature (operating band).

In this contribution, we present our view on the remaining issue for  Redcap UE RF:

Observation 1 For UE supporting Rel-18 power boosting capabilities only inner or enhanced inner region apply and outer region is excluded for power boosting.

Observation 2 Rel-18 (e) RedCap UE can support the Rel-18 power boosting feature.

Observation 3 A (e)RedCap UE can support the power boosting with X dB for PC3 and Y dB for PC2 with insertion loss reduction and apply to both outer region and inner region, also apply to other modulation than BPSK/QPSK if unwanted emission would be met.

Proposal-1: Discuss the (e) RedCap power boosting with the three options listed above.

Proposal-1: If power boosting due to the insertion loss reduction would apply both FD-FDD and HD-FDD, no need to specify the MSD for FD-FDD mode.

Proposal-2: RAN4 discuss if the 38.307 should be updated for RedCap NTN as release independent feature (operating band).

# References

- R4-2419897, Way Forward for [113][310] NR_NTN_Ph3_UE_RF, Qualcomm Inc