# 3GPP TSG-RAN WG4 Meeting #114	R4-2502134

# Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 		BS RF impact for 7 MHz Channel Bandwidth for n26 and n5

Agenda Item:	5.32.1

Document for:	Discussion

# Introduction

New WID on introduction of 7 MHz Channel Bandwidth for n26 and n5 bands was agreed in [1].

In this contribution we provide initial analysis of the BS RF impact for 7 MHz Channel Bandwidth introduction for n26 and n5 bands.

# Discussion

There is a range of BS RF requirements which do require new entries to enable 7 MHz channel introduction. One example is Total power dynamic range requirement, as below (requirement value will depend on the spectrum utilization, to be decided as part of system parameters):

Table 6.3.3.2-1: Total power dynamic range

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 3 | 11.7 | N/A | N/A |
| 5 | 13.9 | 10.4 | N/A |
| 7 | TBD | N/A | N/A |
| 10 | 17.1 | 13.8 | 10.4 |
| 15 | 18.9 | 15.7 | 12.5 |
| 20 | 20.2 | 17 | 13.8 |
| 25 | 21.2 | 18.1 | 14.9 |
| 30 | 22 | 18.9 | 15.7 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 23.3 | 20.2 | 17 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 24.3 | 21.2 | 18.1 |
| 60 | N/A | 22 | 18.9 |
| 70 | N/A | 22.7 | 19.6 |
| 80 | N/A | 23.3 | 20.2 |
| 90 | N/A | 23.8 | 20.8 |
| 100 | N/A | 24.3 | 21.3 |

The following requirements were identified as requiring dedicated updates for 7 MHz channel: Total power dynamic range, Rx dynamic range, RX IMD, ICS.

Proposal 1: Update the following requirements with 7MHz-dedicated modifications: Total power dynamic range, Rx dynamic range, RX IMD, ICS, OTA dynamic range, OTA RX IMD, OTA ICS.

However, it was noted that number of BS RF requirements may be introduced in more generic way, i.e., instead of adding 7MHz-dedicated entries, we can refer to the table 5.3.2-1, there NS channel bandwidths are defined.

Proposal 2: For the following requirements, refrain from adding 7MHz-decicated updates, and implement updates based on the range of channel bandwidths as defined in table 5.3.2-1: ACLR/CACLR, reference sensitivity, ACS, in-band blocking, OTA reference sensitivity level, OTA ACS, OTA in-band blocking.

Example implementations of Proposal 2 were presented below, for reference:

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Update the following requirements with 7MHz-dedicated modifications: Total power dynamic range, Rx dynamic range, RX IMD, ICS, OTA dynamic range, OTA RX IMD, OTA ICS.

Proposal 2: For the following requirements, refrain from adding 7MHz-decicated updates, and implement updates based on the range of channel bandwidths as defined in table 5.3.2-1: ACLR/CACLR, reference sensitivity, ACS, in-band blocking, OTA reference sensitivity level, OTA ACS, OTA in-band blocking.

# References

[1] 	RP-243323	7 MHz Channel Bandwidth for n26 and n5, RAN#106