3GPP TSG-RAN WG4 Meeting #114	R4-2502148

Athens, Greece, 17-21 February 2025

Source:	Nokia

Title:	On LBCA switching RRM requirements

Agenda item:	7.8.3

Document for:	Discussion

Introduction

This paper discusses RAN4 RRM requirements for low band carrier aggregation via switching, according to the new WID approved in RAN #106 [1] with the following objectives:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

Discussion

According to the WID and motivation papers submitted to the plenary, the main motivation for this work item was to enable use of band combinations in close frequency proximity through carrier switching. In some examples, the networks may have available low band spectrum which is very valuable for rural environments. However, this cannot be used efficiently because certain band combinations pose practical implementation challenges from the UE perspective. For this reason, this work item proposes to enable carrier switching to provide better utilization of carriers which are otherwise underused.

In the objective of the WID, it is clearly specified that the carrier switching is focusing on specific example for the band combination CA_n5A-n29A. In this combination n5A is used as FDD PCell, and n29A is used as DL only (SDL) SCell. Figure 1 shows one example of the carrier switching operation, where a time gap between operating on the different carriers is expected.

Figure 1 Example of low band carrier switching with FDD PCell and SDL SCell

One important aspect to be discussed in RAN4 is how long the switching delay between the two carriers should be defined (dT1 and dT2 in figure 1). This decision will be important in order for RAN1 to be able to define the switching pattern to be used in this feature. As one example, RF specification 38.101-1, defines transmit time mask for the transient period between ON and OFF transitions. In our view, the carrier switching transients which would be responsible for the duration of the switching delay are similar to those transients in TDD systems and resemble the existing RF requirements. Additionally, the WID states that RAN1 is responsible to specifying a switching pattern.

Therefore, we propose that the discussion regarding carrier switching delay takes place in RF agenda item.

RF requirements already include transient time for UL transmissions.

The switching pattern between the FDD PCell and SDL SCell is up to RAN1 to define.

The switching time (RF transient time) for switching between the PCell and SCell is up to RF to define.

The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session.

Another point for discussion is that the WID is regarding how to capture the requirements considering the band combination CA_n5A-n29A. We understand that this was agreed in plenary in order to limit the scope of the WID, and that any additional new band combinations might need further study, if introduced.

From RRM perspective we should discuss whether new potential RRM requirements should be defined for this particular band combination only, or in a generic manner.

WID describes that only CA_n5A-n29A will have carrier switching requirements in this work item, other combinations are to be defined in basket WIDs.

If RAN4 is to handle all other LBCA band combinations than CA_n5A-n29A with a basket WID then all related core requirements, including RRM, should not be impacted. This as the baskets are only intended to handle band combination specific RF requirements (i.e. relaxations) and do not consider RRM requirements. This means that any new RRM requirements for low band carrier switching shall be applicable to all of this type of band combination.

Any new RRM requirements for low band carrier switching defined for switching carriers n5A and n29A shall be applicable to all of this type of band combination.

In addition to n5A and n29A it is not clear whether another CC may be configured simultaneously without switching, as described in Figure 2. In one example, a UE might have at a given moment 3 or more CCs, 2 of which are n5A and n29A operating in switched more. When this happens, RAN4 needs to discuss whether the additional CCs would experience interruptions due to the LBCA switching, and we should discuss the interruption requirements for that case.

Figure 2 Example of 2 carriers operating with low-band carrier switching, where a 3rd carrier is also active without switching

It is not clear from WID description whether additional CCs may be configured

RAN4 to discuss whether to define requirements (such as interruption requirements) for a configuration where, in addition to the LBCA CCs to be used with a switching pattern, there may be an additional CC which can be used without switching.

Next, we want to highlight an important question about SSB availability on the CCs included in LBCA. Since the LBCA switching patterns are yet to be defined, it is important that RAN4 gives feedback to RAN1 regarding conditions which may impact how the patterns may be designed. One aspect that is impacting RRM requirements, and the switching pattern design, would be whether SSB is required to be available for measurements on both the PCell (FDD) and the SCell (SDL) carriers.

Since there are not that many TUs allocated to this work item, we think that the best approach is that the LBCA-switching pattern is designed in a manner that it ensures by design and configuration, that the UE can perform measurements on both LBCA CCs without impact on existing UE RRM measurement requirements.

Define switching patterns, enabling availability of SSBs to the UE on each CC, that guarantees that the switching pattern is not impacting RRM measurements.

Additionally, RAN4 should send LS to RAN1 where RAN4 indicates that switching pattern to be designed should follow the principle that guarantees that the switching pattern is not impacting existing RRM measurements.

Send LS to RAN1 informing that SSB must be available in the DL switching patterns in both SDL and FDD CCs.

In the setup of this WI, UL is available in only in one of the switched carriers, i.e. the FDD carrier should be the PCell with UL and the other carrier is DL only. Considering that RAN4 can agree on the assumption that PCell always have SSB available, there is no reason for updating UE UL transmit timing requirements. Therefore, there is no impact expected on UE UL transmit requirements from LBCA switching.

Existing UL transmit timing requirements and TA accuracy requirements are reused for low band CC switching.

Conclusion

The following observations and proposals were made in this paper:

Observation 1: RF requirements already include transient time for UL transmissions.

Observation 2: The switching pattern between the FDD PCell and SDL SCell is up to RAN1 to define.

Observation 3: The switching time (RF transient time) for switching between the PCell and SCell is up to RF to define.

Proposal 1: The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session.

Observation 4: WID describes that only CA_n5A-n29A will have carrier switching requirements in this work item, other combinations are to be defined in basket WIDs.

Proposal 2: Any new RRM requirements for low band carrier switching defined for switching carriers n5A and n29A shall be applicable to all of this type of band combination.

Observation 5: It is not clear from WID description whether additional CCs may be configured

Proposal 3: RAN4 to discuss whether to define requirements (such as interruption requirements) for a configuration where, in addition to the LBCA CCs to be used with a switching pattern, there may be an additional CC which can be used without switching.

Proposal 4: Define switching patterns, enabling availability of SSBs to the UE on each CC, that guarantees that the switching pattern is not impacting RRM measurements.

Proposal 5: Send LS to RAN1 informing that SSB must be available in the DL switching patterns in both SDL and FDD CCs.

Proposal 6: Existing UL transmit timing requirements and TA accuracy requirements are reused for low band CC switching.

- References

- RP-243317, New WID on low band carrier aggregation via switching, RAN #106, Madrid, Spain, December 9-12, 2024

- Draft LS to RAN1

3GPP TSG-RAN WG4 Meeting #114	R4-250xxxx

Athens, Greece, February 17 – 21, 2025

Title:	LS on switching pattern for low band carrier switching

Response to:

Release:	Rel-19

Work Item:	NR LBCA_Sw (NR LBCA_Sw-Core)

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact person:	Riikka Dimnik

Riikka.dimnik (at) nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 have discussed the need for UE to be able to monitor SSB/SMTC on both low band carriers withing the switching pattern, and have reached the following conclusion:

- Define switching patterns, enabling availability of SSBs to the UE on each CC, that guarantees that the switching pattern is not impacting RRM measurements.

# 2	Actions

To RAN1

ACTION:  RAN4 respectfully asks RAN1 to take the above information in consideration while designing LBCA switching patterns.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN WG4 Meeting #114-bis		07 – 11 April 2025				China

TSG-RAN WG4 Meeting #115		19 – 23 May 2025				Malta, MT