3GPP TSG-RAN WG4 Meeting #114	R4-2502155

Athens, Greece, February 17th – 21st 2025

Agenda Item:	7.23.1

Source:	Ericsson

Title:	SBFD general aspects

Document for:	Approval

# 1	Introduction

In this contribution, we present our views and proposals on general aspects for SBFD capable BS regarding band support for SBFD capable BS, SBFD sub-band configuration, and specification structure. Comprehensive analysis of each SBFD BS RF TX/RX requirement and potential new requirements for SBFD capable BS are detailed in separate submissions [1-3].

# 2	Discussion

2.1	SBFD subband configuration

2.1.1	Clarification on declaration and flexibility

##### Issue 1-3-1: Clarification on declaration and flexibility Agreement: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications

In RAN4#113 meeting, it was agreed that for FR1 WA BS, all declared SBFD subband configurations shall be tested. For other BS classes, companies are encouraged to further study the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations.

In our view, for FR1 LA SBFD BS, where analog filters are not expected to be used, and everything is digital, testing the configurations with the narrowest and widest UL/DL subband sizes should be sufficient for a specific channel bandwidth and a specific SBFD pattern.

For FR1 MR SBFD BS, some company claim that a MR SBFD BS could support hundreds of configurations and there is no need to test all of them. In our view, whether to test all of them or not depends on how the RF front end is deployed for the configurations. For lower power configurations, which only require one RF front end, testing the configurations with the narrowest and widest UL/DL subband sizes might be sufficient for a specific channel bandwidth and a specific SBFD pattern. For high power configurations, where an analog filter is expected at the RF frontend for each specific configuration, all configurations shall be tested. Otherwise, testing only the configurations with the narrowest and widest UL/DL subband sizes, while ignoring intermediate ones, means the analog filters for the intermediate subbands are not tested.

For FR1 MR SBFD BS, a power level threshold could be established: above this threshold, all configurations shall be tested; below it, only the configurations with the narrowest and widest UL/DL subband sizes need to be tested for a specific channel bandwidth and a specific SBFD pattern, assuming analog filters are not needed below this power level.

Considering the BS rated carrier TRP output power for BS type 1-O defined in table 9.3.1-1 in TS 38.104 [4], we propose using 33 dBm either as TRP or total conducted power as the power level threshold.

For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

2.1.2 DL/UL subband configurations

##### Issue 1-3-2: DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS ##### Issue 1-3-6: Other restriction on DL subband size Way Forward: - FFS the following restriction on DL subband size: - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size). - FR2 minimum CBW to support SBFD is 100MHz.

In the RAN4#113 meeting, it was agreed to adopt option 2b with restrictions for SBFD subband configurations. For FR1 WA, we are skeptical about the feasibility of using an analog filter, particularly the concept of an analog filter bank. Even if it is feasible to utilize the analog filter, it will not be more than one. Therefore, our opinion is that for a specific channel BW supported for SBFD, a maximum of one specific UL subband sizes is allowed to be declared for FR1 WA.

During the intense discussions in the RAN4#113 meeting, considering the proponents’ strong opinions on the analog filter bank, we could compromise to allow a maximum of two specific UL subband sizes to be declared for a specific channel BW supported for SBFD for FR1 WA. Given that there are two patterns (DUD and DU/UD) for SBFD, for a specific channel BW, a maximum of two specific UL subband sizes implies that four analog filters are needed.

For FR1 LA, since analog filter is not expected to be used, there is no need to apply the restriction on the maximum number of specific UL subband sizes for a specific channel BW supported for SBFD.

For FR1 MR, the discussion is similar to the one above regarding the RF front end: a power level threshold could be established. Above this threshold, a maximum of two specific UL subband sizes are allowed to be declared for a specific channel BW supported for SBFD. Below this threshold, analog filters are not expected to be used, and there is no need to apply the restriction on the maximum number of specific UL subband sizes for a specific channel BW supported for SBFD.

Given that there are two patterns (DUD and DU/UD) for SBFD, for a specific channel BW, a maximum of two specific UL subband sizes implies that four analog filters per subarray are needed.

For FR1 WA SBFD capable BSs, a maximum of two specific UL subband sizes is allowed to be declared for a specific channel BW supported for SBFD.

For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), there is no need to restrict the maximum number of specific UL subband sizes for a specific channel BW supported for SBFD.

For FR1 MR SBFD capable BSs, when the TRP output power is greater than 33 dBm, a maximum of two specific UL subband sizes are allowed to be declared for a specific channel BW supported for SBFD; when the TRP output power is no greater than 33 dBm, analog filters are not expected to be used, and the maximum number of specific UL subband sizes for a specific channel BW supported for SBFD is not restricted.

2.2	Specification structure

##### Issue 1-2-1: BS RF Specification structure for SBFD requirements Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

In the WF [5] approved during RAN4#112bis, there are two options captured on how to introduce BS RF new requirements for SBFD-capable BS. We prefer to create new specifications for SBFD-capable BS, rather than merging it into TS 38.104. The reasons are as follows.

On one hand, a dedicated SBFD spec ensures clarity. It includes only relevant information related to bands, bandwidth, and SBFD configurations, etc., and avoids unnecessary complexity and provides a concise, focused reference for SBFD-related details, which makes it easier to understand and maintain. Creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements could reduce the number of 3GPP specifications but increase the complexity and reduce the readability of TS 38.104, as what we have on UE side. On the other hand, SBFD-capable BS may require new hardware components, which makes it different from legacy base stations.

Moreover, new requirements, e.g., in-channel adjacent subband leakage ratio and blocking/selectivity, are needed for SBFD, and it is more proper to capture them in a new specification.

In the document attached to this T-doc, an example of new spec skeleton is presented, where the new requirements are highlighted, and dedicated clauses needed for SBFD are marked in blue, which shows that if option 2 is adopted, the size of TS 38.104 will be doubled.

RAN4 create new specifications for SBFD-capable BS.

# Conclusion

In this contribution, views, and proposals on general aspects for SBFD WI are presented.

Observation 1	Given that there are two patterns (DUD and DU/UD) for SBFD, for a specific channel BW, a maximum of two specific UL subband sizes implies that four analog filters per subarray are needed.

Proposal 1	For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

Proposal 2	For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

Proposal 3	Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

Proposal 4	For FR1 WA SBFD capable BSs, a maximum of two specific UL subband sizes is allowed to be declared for a specific channel BW supported for SBFD.

Proposal 5	For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), there is no need to restrict the maximum number of specific UL subband sizes for a specific channel BW supported for SBFD.

Proposal 6	For FR1 MR SBFD capable BSs, when the TRP output power is greater than 33 dBm, a maximum of two specific UL subband sizes are allowed to be declared for a specific channel BW supported for SBFD; when the TRP output power is no greater than 33 dBm, analog filters are not expected to be used, and the maximum number of specific UL subband sizes for a specific channel BW supported for SBFD is not restricted.

Proposal 7	RAN4 create new specifications for SBFD-capable BS.

# References

R4-2502157, Impact on SBFD BS RF TX requirements, Ericsson

R4-2502158, Impact on SBFD BS RF RX requirements, Ericsson

R4-2502156, Potentially new SBFD BS RF requirements, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

R4-2416584, Way forward for [112bis][306] NR_duplex_evo_General