3GPP TSG-RAN WG4 Meeting #114	R4-2502159

Athens, Greece, February 17th – 21st 2025

Agenda Item:	7.13.4.2

Source:	Ericsson

Title:	OTA test scope reduction

Document for:	Approval

# 1	Introduction

According to the work plan outlined in R4-2406088 [1], the CR is scheduled to be presented at the meeting in Feb. 2026, and prior to that, the focus will be on the analysis work. Given that there is no TR assigned for this topic, it is essential that all agreements are comprehensively documented in the WF. This will ensure clarity and facilitate progress tracking throughout the work process.

In the approved Way Forward [2] in RAN4#113 meeting, there are two requirements, as shown below, identified as the most time consuming and should be prioritized, i.e., TX IM and RX out-of-band blocking, on which our view focusing on FR1 BS type 1-O is shared in the following section.

Issue 1-1: test scope reduction for Tx requirement - Recommended WF: - Focus on test scope reduction for Tx IMD requirement - Further discuss the applicable interference freq offset for ACLR, OBUE and spurious emission requirements; - Further discuss the test scope reduction of test configuration. Issue 1-2: Out of band blocking - Recommended WF: - Focus on test scope reduction for Rx OOBB requirement - Focus on the measurement step size for Rx OOBB requirement;

# 2	Discussion

The transmitter intermodulation and receiver out-of-band blocking requirements are among the most time-consuming tests due to their extensive frequency range. These tests involve numerous measurement points and test cases, leading to prolonged testing durations and increased costs. The following analysis provides a detailed examination of the challenges and proposed solutions for these requirements.

2.1	Transmitter Intermodulation (TX IM)

The transmitter intermodulation requirement measures the transmitter’s capability to inhibit signal generation of signals in its non-linear elements due to the presence of wanted and interfering signals. This requirement is crucial for ensuring that the transmitter does not produce unwanted intermodulation products that could interfere with other signals.

For BS type 1-O the transmitter intermodulation level shall not exceed the TRP unwanted emission limits specified for OTA transmitter spurious emission, OTA operating band unwanted emissions and OTA ACLR in the presence of a wanted signal and an interfering signal. Currently, there are 6 interferer frequency offsets tested (3 on each side of the carrier), where, for n=1, 2 and 3, as illustrated in Figure 1. The OTA transmitter intermodulation requirement is not applicable for BS type 2-O.

Considering the various emission limits, the number of interferers, and all applicable test configurations, the extensive testing is both time-consuming and resource intensive. Therefore, it is essential to reassess the number of interferer offsets and test configurations in order to reduce testing efforts while ensuring full requirement coverage.

Figure 1: Illustration of interferers

2.1.1	Interferer frequency offset

To reduce the testing burden of FR1 OTA transmitter intermodulation, an effective approach is to reduce the number of interferer frequency offsets. Meanwhile, it is important to maintain the requirement coverage and ensure the transmitter’s ability is well examined. This can be achieved by testing the worst-case scenario. The following analysis examines the worst-case scenario from various perspectives, including ACLR, OBUE and transmitter spurious emission.

OBUE

The OTA operating band unwanted emission requirement for BS type 1-O is that for each applicable basic limit for BS type 1-H in clause 6.6.4.2, the power of any unwanted emission shall not exceed an OTA limit specified as the basic limit + X, where X = 9 dB.

Unlike ACLR and transmitter spurious emissions, OBUE is more complex and has varying limit levels. For instance, referring to Table 6.6.4.2.1-2 in TS 38.104 [3], we can observe that the basic limits differ across frequency ranges. As a result, it is difficult to determine which interferer frequency offset represents the worst-case scenario.

Table 6.6.4.2.1-2 in TS 38.104 [3]: Wide Area BS operating band unwanted emission limits 
(NR bands above 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 5 MHz | 0.05 MHz £ f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz £ Df < min(10 MHz, Dfmax) | 5.05 MHz £ f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz £ Df £ Dfmax | 10.5 MHz £ f_offset < f_offsetmax | -13 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Df ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be 13 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when Dfmax < 10 MHz. |  |  |  |

For OBUE, it is difficult to determine which interferer frequency offset represents the worst-case scenario due to the varying basic limits across frequency ranges.

Regarding the test case of OBUE for TX IM requirement, no reduction in the number of interferer frequency offset should be made.

ACLR

Regarding ACLR, on one hand, we share similar view as the analysis outlined in R4-2419243 [4] that the closest interferers (f3 and f4 in Figure 1) represent the worst-case scenarios. On the other hand, given that both OBUE and ACLR are in the adjacent channel, another potential improvement would be to measure OBUE and ACLR simultaneously and perform data post-processing to assess each requirement separately.

Regarding ACLR for LTE and NR, the 1xCBW offset is the worst-case scenario.

Given that both OBUE and ACLR are in the adjacent channel, a potential improvement would be to measure OBUE and ACLR simultaneously and perform data post-processing to assess each requirement separately.

Transmitter spurious emission

The OTA transmitter spurious emission limits for FR1 shall apply from 30 MHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table 9.7.1-1 in TS 38.104 [3].

Table 9.7.1-1 in TS 38.104 [3]: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics (MHz) | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 | 40 |
| BS type 2-O | FDL,high – FDL,low ≤ 4000 | 1500 |
|  | 4000 < FDL,high – FDL,low ≤ 14000 | 3500 |

Considering ΔfOBUE for BS type 1-O, as defined in table 9.7.1-1 in TS 38.104 [3], among the six interferers in Figure 1, the closer interferers (f2, f3, f4, f5) are less likely to fall within the scope of spurious emission. In contrast, the more distant interferers (f1 and f6) can be considered the worst-case scenario. While simplification is feasible, it would be better to limit it to specific relevant bands, as some of the lower bands will not be OTA tested anyway.

For transmitter spurious emissions, the 3xCBW offset is the worst-case scenario.

While simplification is feasible, it would be better to limit it to specific relevant bands, as some of the lower bands will not be OTA tested anyway.

2.1.2	Test configuration

To reduce the testing burden of FR1 OTA transmitter intermodulation, another effective approach is to reduce the number of test configurations.

As outlined in R4-2419243 [4], and referencing AAS TS 37.145-2, both ATCR7 and ANTCR7 are required for C and NC capable BS with identical parameters. The decision of whether to use ATCR7 or ANTCR7 requires further consideration. In our view, similar to the approach taken in the analysis of interferer frequency offset, the worst-case scenario should be considered. Since the requirement in question is a transmitter test, the worst-case scenario occurs when the PA is fully loaded. Therefore, regardless of which test configuration is used, the condition of maximum output power – Rated total output power (Prated,t,AC, or Prated,t,TABC) – should be met.

For the reduction of test configurations for TX IM requirement, regardless of the test configuration selected (ATCRx or ANTCRx), the condition of maximum output power – Rated total output power (Prated,t,AC, or Prated,t,TABC) – should be met.

2.2	Receiver Out-of-Band Blocking

The out-of-band blocking requirement measures the receiver’s ability to handle high-level interference from unwanted signals without degrading sensitivity. This requirement ensures that the receiver can operate effectively in the presence of strong interfering signals.

The out-of-band blocking test is conducted across a wide frequency range 30 – 12750 MHz for FR1, with a 1 MHz step size. This small step size results in a large number of measurement points and prolonged testing durations and increased costs.

Therefore, it is essential to find an approach, e.g., increasing the measurement step size, to reduce the excessive testing meanwhile maintain the requirement coverage and ensure the receiver’s ability is well examined, i.e., the increased step size needs to provide sufficient granularity.

In R4-2419211 [5] the proposal suggests reusing the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02. If we follow the FR2 approach, when different BSs declare different minimum channel bandwidths, it will probably result in different BSs being tested with varying step sizes. However, all BSs should be tested consistently using the same step size. Moreover, the minimum channel bandwidth supported by BS is related to digital filtering and digital processing, which means that adjustment of step size is made digitally, while the analog filter is tested with different step sizes.

Another finding is that FR1 offers much more bandwidth than FR2, and BSs in FR1 can easily support narrower bandwidths. If a BS declares support a very narrow bandwidth, the FR2 approach becomes less meaningful for test reduction.

We need further study on how to increase the step size, e.g., a fixed step size larger than 1 MHz, or relating the step size to the size of the band.

If the measurement step size for FR1 OTA out-of-band blocking test is increased from 1 MHz to 1/3 of minimum supported BS channel bandwidth of wanted signal, different BSs may be tested with varying step sizes. However, all BSs should be tested consistently using the same step size.

Minimum channel bandwidth supported by BS is related to digital filtering and digital processing and should not be a factor affecting testing of analog filter.

FFS on how to increase the step size, e.g., a fixed step size larger than 1 MHz, or relating the step size to the size of the band.

# Conclusion

In this contribution, views, and proposals on OTA test scope reduction are presented.

Observation 1	For OBUE, it is difficult to determine which interferer frequency offset represents the worst-case scenario due to the varying basic limits across frequency ranges.

Observation 2	Regarding ACLR for LTE and NR, the 1xCBW offset is the worst-case scenario.

Observation 3	Given that both OBUE and ACLR are in the adjacent channel, a potential improvement would be to measure OBUE and ACLR simultaneously and perform data post-processing to assess each requirement separately.

Observation 4	For transmitter spurious emissions, the 3xCBW offset is the worst-case scenario.

Observation 5	While simplification is feasible, it would be better to limit it to specific relevant bands, as some of the lower bands will not be OTA tested anyway.

Observation 6	If the measurement step size for FR1 OTA out-of-band blocking test is increased from 1 MHz to 1/3 of minimum supported BS channel bandwidth of wanted signal, different BSs may be tested with varying step sizes. However, all BSs should be tested consistently using the same step size.

Observation 7	Minimum channel bandwidth supported by BS is related to digital filtering and digital processing and should not be a factor affecting testing of analog filter.

Proposal 1	Regarding the test case of OBUE for TX IM requirement, no reduction in the number of interferer frequency offset should be made.

Proposal 2	For the reduction of test configurations for TX IM requirement, regardless of the test configuration selected (ATCRx or ANTCRx), the condition of maximum output power – Rated total output power (Prated,t,AC, or Prated,t,TABC) – should be met.

Proposal 3	FFS on how to increase the step size, e.g., a fixed step size larger than 1 MHz, or relating the step size to the size of the band.

# References

R4-2406088, Way Forward for [110bis][311] NR_BS_RF

R4-2419828, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

R4-2419243, Test case reduction for TX IM

R4-2419211, Discussion on the applicability improvement of TRP measurement