# 3GPP TSG-RAN WG4 Meeting # 114                                                          R4-2502172

# Athens, Greece, 17 – 21 February 2025

# Title:	HPUE MSD for SUL

# Agenda Item:	6.8.4

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document proposes to introduce missing 1UL HPUE MSD requirements for SUL combinations based on [1].

# Discussion

## Background

PC2 operation is supported for the following SUL bands: n80, n84, n95, n97 and n98.

The following is a list of SUL combinations in which 1UL PC2 MSD test points should be introduced:

- MSD due to UL harmonic interference:

  - SUL_n77A-n80A, SUL_n78A-n80A, SUL_n77A-n84A, and SUL_n79A-n97A,

- MSD due to Cross-band isolation interference:

  - SUL_n41A-n80A, SUL_n3A-n84A, SUL_n41A-n95A, SUL_n41A-n97A, SUL_n41A-n98A.

The cross-comparison of NR-CA 1UL PC2 MSD test points with valid 1UL PC2 SUL configurations for SUL band combinations demonstrates that we also need to introduce RX harmonic mixing MSD test points for SUL_n41A-n98A for both PC3 and PC2 operation.

Regarding the HPUE MSD simplification initiative ongoing at RAN4:

- It may be possible to avoid introducing new MSD tables for 1UL HPUE SUL operation thanks to the concept of 1UL HPUE MSD simplification look-up tables [2].

- However, there are no agreements on such a simplification process today. New MSD test points should therefore be introduced using the legacy MSD table format.

Observation 1: Several 1UL PC2 MSD test points need to be introduced for MSD due to UL harmonic interference and for MSD due to cross-band isolation interference. Also new PC3 and PC2 test points for MSD due to RX harmonic mixing are needed for SUL_n41A-n98A. These new test points should be introduced using the legacy MSD table format.

Based on this observation, we propose MSD requirements for 1UL PC2 1Tx and 1UL PC2 2Tx for eligible SUL combinations with the following MSD evaluation method:

- Adopt NR CA 1UL PC2 1Tx and PC2 2Tx MSD requirements if an exact NR-CA counterpart MSD test point is specified,

- Otherwise, evaluate the 1UL PC2 MSD using the Table 1 1UL HPUE MSD simplification look-up table [2]:

Table 1: 1UL PC21Tx , PC22Tx , PC1.5 LUT.

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

## 1UL HPUE MSD test points due to UL harmonic interference

For SUL_77A-n80A and SUL_n77A-n84A: there is no PC2 NR CA MSD test point counterpart. The PC2 1Tx and PC2 2Tx MSD requirements are captured in Table 2 using the Table 1 HPUE simplification LUT, where the PC3 MSD entry to the LUT is rounded to the closest integer.

Table 2: Band n77 PC2 1Tx/2Tx MSD requirements for SUL_n77A-n80A and SUL_n77A-n84A.

|  | SUL_n77A-n80A UL2/DL1 SUL_n77A-n84A UL2/DL1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Band n77 10MHz 1st MSD test point |  | Band n77 100MHz 2nd MSD test point |  |
|  | PC21Tx | PC22Tx | PC21Tx | PC22Tx |
| Table 7.3C.2-2 PC3 MSD (dB) | 23.9 |  | 13.8 |  |
| PC21Tx/PC22Tx (dB) | 3.0 | 6.0 | 3.0 | 6.0 |
| PC21Tx/PC22Tx MSD (dB) | 26.9 | 29.9 | 16.8 | 19.8 |

For SUL_n79A-n97A: The Table 1 HPUE simplification LUT cannot be applied directly to the specified PC3 MSD requirement because the 1st PC3 MSD test point needs to be updated to account for the SUL_n79A-n97A BCS4/5 requirements where the new smallest DL CBW is now 10MHz and not 40MHz. Also, the SUL n97 UL RB configuration needs to be corrected for UL2/DL1 from 25RBs to 12RBs.

Observation 2: The current Band n79 first PC3 MSD test point needs to be re-evaluated for 10MHz CBW to account for the BCS4/5 requirements of SUL_n79A-n97A. The SUL n97 UL RB configuration needs to be corrected from 25RBs to 12RBs.

Based on observation 2, the band n79 MSD 10MHz CBW is evaluated at 35.5dB in Table 3 using the following method:

- Evaluate the SUL n97 UL2 interference level that corresponds to the specified Band n79 40MHz PC3 MSD of 29.4dB. We assume 6dB Main/Diversity interference imbalance.

- Thanks to the new rule on UL RB allocation for UL harmonic MSD test points, the same level of interference can be applied to the MRC MSD equations to evaluate Band n79 PC3 10MHz CBW MSD.

Table 3: Band n79 PC3 10MHz MSD requirements for SUL_n79A-n97A.

|  | 40MHz CBW MSD test point |  | 10MHz CBW MSD test point |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Prim. | Div. | Prim. | Div. |  |
| Antenna referred in-channel harmonic interference levels: [primary/diversity] (dBm/MBW) | -54.9 | -60.9 | -54.9 | -60.9 |  |
| Ideal REFSENS: [n79 40/10MHz] (dBm/MBW) | -89.7 |  | -95.8 |  |
| Noise floor (dBm/MBW) | -85.7 | -85.7 | -91.8 | -91.8 |  |
| Total noise level at antenna connector (dBm/MBW) | -54.9 | -60.9 | -54.9 | -60.9 |  |
| REFSENS MRC degradation: [n79 40/10MHz] (dB) | 29.4 |  | 35.5 |  |

The Band n79 1UL PC2 1Tx/2Tx MSD requirements are captured in Table 4 using the Table 1 HPUE simplification LUT.

Table 4: Band n79 PC2 1Tx/2Tx MSD requirements for SUL_n79A-n97A.

|  | SUL_n79A-n97A UL2/DL1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Band n79 10MHz 1st MSD test point |  | Band n79 100MHz 2nd MSD test point |  |
|  | PC21Tx | PC22Tx | PC21Tx | PC22Tx |
| Table 7.3C.2-2 PC3 MSD (dB) | 35.5 |  | 25.3 |  |
| PC21Tx/PC22Tx (dB) | 3.0 | 6.0 | 3.0 | 6.0 |
| PC21Tx/PC22Tx MSD (dB) | 38.5 | 41.5 | 28.3 | 31.3 |

We note that no MSD test point has been specified its NR counterpart “CA_n40A-n79A”.

Observation 3: The Band n79 UL2/DL1 MSD requirements are missing for CA_n40A-n79A.

For SUL_n78A-n80A: The PC2 2Tx MSD requirements of the NR counterpart CA_n3A-n78A do not correspond to those evaluated in Table 2. Since there is no reason to have different PC2 MSD requirements than those of SUL_n77A-n80A, we propose to copy the 1UL PC2 MSD requirements from SUL_n77A-n80A and invite companies to study how to resolve the discrepancy between SUL_n78A-n80A and CA_n3A-n78A.

Observation 4: The 1UL PC2 2tx MSD requirements for CA_n3A-n78A need to be cross-aligned with the proposed test point for SUL_n78A-n80A.

Proposal 1: Adopt the following changes to Table 7.3C.2.

Table 7.3C.2-2: PC3 Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to uplink harmonic issue)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n80 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n81 | n41 | 5 | 15 | 8 | 10 | 13 | NOTE 3 | UL3/DL1 direct-hit |
| n81 | n41 | 5 | 15 | 8 | 100 | 4.5 | NOTE 3 | UL3/DL1 direct-hit |
| n81 | n78 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n81 | n78 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| n81 | n79 | 5 | 15 | 5 | 40 | 6.8 | NOTE 5 | UL5/DL1 direct-hit |
| n81 | n79 | 5 | 15 | 5 | 100 | 3.8 | NOTE 5 | UL5/DL1 direct-hit |
| n82 | n78 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n82 | n78 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| n83 | n78 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n83 | n78 | 5 | 15 | 5 | 100 | 2.9 | NOTE 5 | UL5/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n86 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n86 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n89 | n78 | 5 | 15 | 6 | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n89 | n78 | 5 | 15 | 6 | 100 | 2.9 | NOTE 4 | UL4/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 12 | 10 | 35.5 | NOTE 2 | UL2/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 12 | 100 | 25.3 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 1: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. This DL band may be affected by near-miss interference for which the MSD is not specified. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Proposal 2: Introduce the following new PC2 MSD table due to UL harmonics for SUL.

Table 7.3C.2-2a: PC2 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to uplink harmonic issue).

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n80 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n77 | 5 | 15 | 12 | 100 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 12 | 10 | 26.9 | 29.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 12 | 100 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.9 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 12 | 100 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 25 | 10 | 38.5 | 41.5 | NOTE 2 | UL2/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 25 | 100 | 28.3 | 31.3 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 1: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. This DL band may be affected by near-miss interference for which the MSD is not specified. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |  |

## 1UL HPUE MSD test points due to cross-band isolation

For SUL_n41A-n80A, SUL_n3A-n84A, SUL_n41A-n95A: there is no PC2 NR CA MSD test point counterpart. The PC2 1Tx and PC2 2Tx MSD requirements are captured in Table 5 using the Table 1 HPUE simplification LUT, where the PC3 MSD entry to the LUT is rounded to the closest integer.

Table 5: PC2 1Tx/2Tx MSD requirements for SUL_n41A-n80A, SUL_n3A-n84A, SUL_n41A-n95A.

|  | SUL_n41A-n80A, >ACLR2 |  |  |  | SUL_n3A-n84A, ACLR1 |  | SUL_n41A-n95A, >ACLR2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band n41 10MHz 1st MSD test point |  | Band n41 100MHz 2nd MSD test point |  | Band n3 5MHz MSD test point |  | Band n41 10MHz 1st MSD test point |  | Band n41 100MHz 2nd MSD test point |  |
|  | PC21Tx | PC22Tx | PC21Tx | PC22Tx | PC21Tx | PC22Tx | PC21Tx | PC22Tx | PC21Tx | PC22Tx |
| Table 7.3C.2-4 PC3 MSD (dB) | 0.7 |  | 0.7 |  | 19.7 |  | 3.2 |  | 3.2 |  |
| PC21Tx/PC22Tx (dB) | 0.6 | 0.9 | 0.6 | 0.9 | 3.0 | 6.0 | 1.6 | 2.7 | 1.6 | 2.7 |
| PC21Tx/PC22Tx MSD (dB) | 1.3 | 1.6 | 1.3 | 1.6 | 22.7 | 25.7 | 4.8 | 5.9 | 4.8 | 5.9 |

For SUL_n41A-n97A: The PC2 2Tx MSD requirements of the NR counterpart CA_n40A-n41A 1st MSD test point is exactly identical to the MSD evaluated in Table 6 using the 1UL HPUE MSD simplification LUT. However, for NR-CA only 1 PC2 MSD test point is specified, while for SUL two test points are specified. We therefore propose to evaluate the SUL_n41A-n97A PC2 MSD using the 1UL HPUE LUT Table 1. For SUL_n41A-n98A, RAN4 did not specify PC2 2Tx MSD. We propose MSD requirements for both architectures for SUL_n41A-n98A in Table 6.

Table 6: PC2 1Tx/2Tx MSD requirements for SUL_n41A-n97A, SUL_n41A-n98A.

|  | SUL_n41A-n97A, ACLR2 |  |  |  | SUL_n41A-n98A, ACLR2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Band n41 10MHz 1st MSD test point |  | Band n41 10MHz 1st MSD test point |  | Band n41 10MHz 1st MSD test point |  |
|  | PC21Tx | PC21Tx | PC21Tx | PC21Tx | PC21Tx | PC22Tx |
| Table 7.3C.2-4 PC3 MSD (dB) | 28.1 |  | 10.6 |  | 3.3 |  |
| PC21Tx/PC22Tx (dB) | 3.0 | 6.0 | 3.0 | 6.0 | 1.6 | 2.7 |
| PC21Tx/PC22Tx MSD (dB) | 31.1 | 34.1 | 13.6 | 16.6 | 4.9 | 6.0 |

Proposal 3: Introduce the following new PC2 cross-band isolation MSD table for SUL.

Table 7.3C.2-4a: PC2 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to cross band isolation).

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2501 | 10 | 1.31 1.62 | >ACLR2 |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2546 | 100 | 1.31 1.62 | >ACLR2 |
| n84 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 22.71 25.72 | ACLR1 |
| n95 | n41 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2501 | 10 | 4.81 5.92 | >ACLR2 |
| n95 | n41 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2546 | 100 | 4.81 5.92 | >ACLR2 |
| n97 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 31.11 34.12 | ACLR2 |
| n97 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2546 | 100 | 13.61 16.62 | ACLR2 |
| n98 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 4.91 6.02 | >ACLR2 |
| NOTE 1: Applicable to UE’s supporting PC2 with 1Tx. NOTE 2: Applicable to UE’s supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |

## 1UL HPUE MSD test points due to RX harmonic mixing

For SUL_n41A-n98A, a PC3 and a PC2 UL4/DL3 MSD test point is specified for Band n41 in the counterpart CA_n39A-n41A. We therefore propose to copy the PC3 CA_n39A-n41A MSD requirements and evaluate the PC2 1Tx/2Tx requirements using the Table 1 1UL HPUE simplification LUT as follows:

- For the 1st MSD test point:

  - PC21Tx = 2.5dB, i.e. PC2 1Tx MSD = PC3 MSD + PC21Tx = 9.3 + 2.5 =11.8dB,

  - PC22Tx = 2.5dB, i.e. PC2 2Tx MSD = PC3 MSD + PC22Tx = 9.3 + 5 =14.3dB,

- For the 2nd MSD test point:

  - PC21Tx = 2.5dB, i.e. PC2 1Tx MSD = PC3 MSD + PC21Tx = 2.2 + 1.2 =3.3dB,

  - PC22Tx = 2.5dB, i.e. PC2 2Tx MSD = PC3 MSD + PC22Tx = 2.2 + 2.1 =4.3dB,

Proposal 4: Introduce the following new PC3 and new PC2 RX harmonic mixing MSD table for SUL.

Table 7.3C.2-6: PC3 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to harmonic mixing)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n98 | n41 | 5 | 15 | 6 | 10 | 9.3 | NOTE 2 | UL4/DL3 |
| n98 | n41 | 5 | 15 | 6 | 100 | 2.2 | NOTE 2 | UL4/DL3 |
| NOTE 1: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 7.3C.2-6a: PC2 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to harmonic mixing)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n98 | n41 | 5 | 15 | 6 | 10 | 11.83 14.34 | NOTE 2 | UL4/DL3 |
| n98 | n41 | 5 | 15 | 6 | 100 | 3.33 4.34 | NOTE 2 | UL4/DL3 |
| NOTE 1: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: Applicable to UE’s supporting PC2 with 1Tx. NOTE 4: Applicable to UE’s supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |

# Conclusion

This paper proposes 1UL PC2 MSD requirements for eligible SUL band combinations.

Observation 1: Several 1UL PC2 MSD test points need to be introduced for MSD due to UL harmonic interference and for MSD due to cross-band isolation interference. Also new PC3 and PC2 test points for MSD due to Rx harmonic mixing are needed for SUL_n41A-n98A. These new test points should be introduced using the legacy MSD table format.

Observation 2: The current Band n79 1st PC3 MSD test point needs to be re-evaluated for 10MHz CBW to account for BCS4/5 requirements of SUL_n79A-n97A. The SUL n97 UL RB configuration needs to be corrected from 25RBs to 12RBs.

Observation 3: The Band n79 UL2/DL1 MSD requirements are missing for CA_n40A-n79A.

Observation 4: The 1UL PC2 2tx MSD requirements for CA_n3A-n78A need to be cross-aligned with the proposed test point for SUL_n78A-n80A.

Proposal 1: Adopt the following changes to Table 7.3C.2.

Table 7.3C.2-2: PC3 Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to uplink harmonic issue).

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n80 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n81 | n41 | 5 | 15 | 8 | 10 | 13 | NOTE 3 | UL3/DL1 direct-hit |
| n81 | n41 | 5 | 15 | 8 | 100 | 4.5 | NOTE 3 | UL3/DL1 direct-hit |
| n81 | n78 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n81 | n78 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| n81 | n79 | 5 | 15 | 5 | 40 | 6.8 | NOTE 5 | UL5/DL1 direct-hit |
| n81 | n79 | 5 | 15 | 5 | 100 | 3.8 | NOTE 5 | UL5/DL1 direct-hit |
| n82 | n78 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n82 | n78 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| n83 | n78 | 5 | 15 | 5 | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n83 | n78 | 5 | 15 | 5 | 100 | 2.9 | NOTE 5 | UL5/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n86 | n78 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n86 | n78 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n89 | n78 | 5 | 15 | 6 | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n89 | n78 | 5 | 15 | 6 | 100 | 2.9 | NOTE 4 | UL4/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 12 | 10 | 35.5 | NOTE 2 | UL2/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 12 | 100 | 25.3 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 5 | 15 | 12 | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 5 | 15 | 12 | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 1: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. This DL band may be affected by near-miss interference for which the MSD is not specified. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Proposal 2: Introduce the following new PC2 MSD table due to UL harmonics for SUL.

Table 7.3C.2-2a: PC2 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to uplink harmonic issue).

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD (NOTE 7) | MSD (NOTE 8) | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n80 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n77 | 5 | 15 | 12 | 100 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 12 | 10 | 26.9 | 29.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 12 | 100 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 12 | 10 | 26.9 | 29.9 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 12 | 100 | 16.8 | 19.8 | NOTE 2 | UL2/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 25 | 10 | 38.5 | 41.5 | NOTE 2 | UL2/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 25 | 100 | 28.3 | 31.3 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 1: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. This DL band may be affected by near-miss interference for which the MSD is not specified. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: Applicable to UEs supporting PC2 with 1Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |  |

Proposal 3: Introduce the following new PC2 cross-band isolation MSD table for SUL.

Table 7.3C.2-4a: PC2 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to cross band isolation).

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2501 | 10 | 1.31 1.62 | >ACLR2 |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2546 | 100 | 1.31 1.62 | >ACLR2 |
| n84 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 22.71 25.72 | ACLR1 |
| n95 | n41 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2501 | 10 | 4.81 5.92 | >ACLR2 |
| n95 | n41 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2546 | 100 | 4.81 5.92 | >ACLR2 |
| n97 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 31.11 34.12 | ACLR2 |
| n97 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2546 | 100 | 13.61 16.62 | ACLR2 |
| n98 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 4.91 6.02 | >ACLR2 |
| NOTE 1: Applicable to UE’s supporting PC2 with 1Tx. NOTE 2: Applicable to UE’s supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |  |

Proposal 4: Introduce the following new PC3 and new PC2 Rx harmonic mixing MSD table for SUL.

Table 7.3C.2-6: PC3 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to harmonic mixing)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n98 | n41 | 5 | 15 | 6 | 10 | 9.3 | NOTE 2 | UL4/DL3 |
| n98 | n41 | 5 | 15 | 6 | 100 | 2.2 | NOTE 2 | UL4/DL3 |
| NOTE 1: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 7.3C.2-6a: PC2 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to harmonic mixing)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n98 | n41 | 5 | 15 | 6 | 10 | 11.83 14.34 | NOTE 2 | UL4/DL3 |
| n98 | n41 | 5 | 15 | 6 | 100 | 3.33 4.34 | NOTE 2 | UL4/DL3 |
| NOTE 1: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: Applicable to UE’s supporting PC2 with 1Tx. NOTE 4: Applicable to UE’s supporting PC2 with 2Tx. |  |  |  |  |  |  |  |  |

# References

- R4-2419721 Impact of SUL HPUE on MSD, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

- R4-242xxxx 1UL MSD simplification rules for Rel-19, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc, Nokia, [Qualcomm France]