3GPP TSG-RAN WG4 Meeting #114									                        R4-2502193

Athens, Greece, February 17 – 21, 2025

Agenda item:			7.10.2.2.1

Source:	Ericsson

Title:	On UE RF Tx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Tx requirements for NR-NTN HPUE from the approved WF in RAN4#113 [2].

# Discussion

## General considerations

The following issues have remained for further study:

Issue 2-8: SEM

Way forward:

- Follow previous agreement that re-use existing SEM from PC3 to other PCs as starting point.

- Further discuss if SEM can be relaxed for the best power gains for NTN with co-ex study outcomes.

SEM requirements are specified based on the regulatory requirements and should not depend on the outcome of the co-existence study. In TN UE RF requirements, TS38.101 [3], the SEM requirements do not depend on the Power Class. In TS38.101-5, the SEM requirements follow the corresponding ones from TS38.101-1, and those should also not depend on the Power Class.

Proposal 1: The existing SEM requirements from PC3 should be re-used for other PCs as well, as these requirements are specified based on the regulatory requirements.

Issue 2-5: NR-NTN PC2 UE

Agreement in #112bis meeting: Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE Agreement: - Follow previous agreement Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - ....

- Proposals

  - Option 1: In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices. (P1a/R4-2417747)

  - Option 2: For handheld UE, 1Tx and 2Tx are covered with existing TN 26dBm and 23dBm PAs respectively. (P1/R4-2419521)

Proposal 2: For PC2, both 1Tx and 2Tx architectures can be considered as 23dBm and 26dBm PAs are available for UEs for TN.

Issue 2-6: NR-NTN PC1.5 UE

Agreement in #112bis meeting: Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE Agreement: - Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions: - FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified - FFS whether PC1.5 handheld UE can be specified or not

- Proposals

| PC1.5 UE | 1Tx | 2Tx |
| --- | --- | --- |
| Handheld | To specify: Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015, P1/R4-2419521 FFS: | To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532 Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015 FFS: |
| Non-handheld | To specify: Not to specify: P1/R4-2419668, P3/R4-2419072 FFS: P5/R4-2418200, P1/R4-2419521 | To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532, P2/R4-2419521 Not to specify: P1/R4-2419668, P3/R4-2419072, P1b/R4-2417747 FFS: |

For TN [3], for all FDD bands which support PC1, specification of PC1.5 is “skipped” (actually, no FDD band supports PC1.5). The similar can be done for the NTN as well.

Proposal 3: For non-handheld UEs, for a given operating band PC1.5 does not need to be specified if PC1 is specified.

Issue 2-7: NR-NTN PC1 UE

Agreement in #112bis meeting: Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.

- Proposals

| PC1 | 1Tx | 2Tx | 3Tx | 4Tx |
| --- | --- | --- | --- | --- |
| Handheld | To specify: Not to specify: P1/R4-2419521 FFS: | To specify: Not to specify: P5/R4-2418200, P1/R4-2419521 FFS: | To specify: P1/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200 | To specify: Not to specify: FFS: |
| Non-handheld Only non-handheld: P2/R4-2419015, P1/R4-2419072 | To specify: P1a/R4-2417747, P2/R4-2419521 Not to specify: FFS: | To specify: Not to specify: P5/R4-2418200 FFS: | To specify: P2/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200 | To specify: P4/R4-2419072 Not to specify: FFS: |

In the ongoing Rel-19 WI “NR MIMO Phase 5”, RAN1 has specified only non-coherent TPMI precoders, and full-power transmission modes 1 and 2 are precluded from the WID. The outcome of RAN4#113 is that the full-power mode 0 will not be specified for PC1.5 as well, so only “Rel-15 behaviour” will be available. PC1 is out of the scope (not even discussed). It would be pre-mature to consider PC1 with 3Tx for either handheld or non-handheld UEs in Rel-19.

Proposal 4: For TN, in Rel-19 only PC1.5 is being considered for 3Tx (not yet confirmed to be feasible), so 3Tx for PC1 should not be considered in this Release for NTN (for both handheld and non-handheld UEs).

Since in RAN4#112-bis meeting companies have agreed that PC1.5 with 1Tx is feasible for NTN UEs, as well as PC1 with 1Tx, PC1 with 2Tx should also be feasible since its complexity is equivalent to having two times PC1.5 with 1Tx.

Proposal 5: PC1 with 2Tx should be feasible for non-handheld NTN UEs, since PC1/PC1.5 with 1Tx is feasible.

## SAR related

Regarding the solution which should be specified for SAR compliance, the similar solution can be considered for both NR-NTN and IoT-NTN, where we have made our proposals under AI “7.10.2.3.1 Tx requirements” for “IoT-NTN HPUE RF requirements” [4], since the discussion is taking place there. We will copy our proposals to this contribution as well:

Proposal 6: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 7: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details:

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

# Conclusion

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: The existing SEM requirements from PC3 should be re-used for other PCs as well, as these requirements are specified based on the regulatory requirements.

Proposal 2: For PC2, both 1Tx and 2Tx architectures can be considered as 23dBm and 26dBm PAs are available for UEs for TN.

Proposal 3: For non-handheld UEs, for a given operating band PC1.5 does not need to be specified if PC1 is specified.

Proposal 4: For TN, in Rel-19 only PC1.5 is being considered for 3Tx (not yet confirmed to be feasible), so 3Tx for PC1 should not be considered in this Release for NTN (for both handheld and non-handheld UEs).

Proposal 5: PC1 with 2Tx should be feasible for non-handheld NTN UEs, since PC1/PC1.5 with 1Tx is feasible.

Proposal 6: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value.

Proposal 7: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details:

- If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance:

  - DPC = ‘00’: the criteria to report DPC is not met

  - DPC = ‘01’: ΔPPowerClass = 0 dB

  - DPC = ‘10’: ΔPPowerClass = 3 dB

  - DPC = ‘11’: ΔPPowerClass = 6 dB

- If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c):

  - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications

  - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB

  - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB

  - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2420403, “WF on NR NTN HPUE”, Samsung, RAN4#113.

- 3GPP TS 38.101-1 V19.0.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2502195, “On UE RF Tx requirements for IoT-NTN HPUE”, Ericsson, Sony, RAN4#114.