3GPP TSG-RAN WG4 Meeting # 114	R4-2502200

# Athens, Greece, 17th – 21st February, 2025

Source:			T-Mobile USA

Title:			Spectrum Utilization for the 7 MHz channel bandwidth

Agenda item:		7.9.2

Document for:		Discussion

# Introduction

At RAN#106 a new Work Item to introduce a 7 MHz channel bandwidth was approved [1].

The objectives included the following:

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4]

  - Introduce the channel bandwidth 7MHz

    - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

  - Specify BS RF requirements for UL and DL

This contribution proposes values for spectrum utilization for the 7 MHz channel bandwidth.

# Discussion

## Existing spectrum utilization

The number of RBs for 5 MHz is 25, and the number of RBs for 10 MHz is 52, as can be found in Table 5.3.2-1 in 38.101-2:

The minimum guardband for 5 MHz is 242.5 MHz and for 10 MHz is 312.5 MHz:

## Spectrum utilization options for the 7 MHz channel bandwidth

The potential options for RBs for 7 MHz are 35 or 36:

|  |  |  | Option A | Option B |  |
| --- | --- | --- | --- | --- | --- |
| SCS (kHz) | 3 | 5 | 7 | 7 | 10 |
|  | MHz | MHz | MHz | MHz | MHz |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 35 | 36 | 52 |

The effective channel bandwidth for Option A and Option B in kHz are:

|  |  |  | Option A | Option B |  |
| --- | --- | --- | --- | --- | --- |
| SCS (kHz) | 3 | 5 | 7 | 7 | 10 |
|  | MHz | MHz | MHz | MHz | MHz |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 2700 | 4500 | 6300 | 6480 | 9360 |

The minimum guardband for Option A and Option B are:

|  |  |  | Option A | Option B |  |
| --- | --- | --- | --- | --- | --- |
| SCS (kHz) | 3 | 5 | 7 | 7 | 10 |
|  | MHz | MHz | MHz | MHz | MHz |
| 15 | 142.5 | 242.5 | 342.5 | 252.5 | 312.5 |

As can be seen above, 35 RBs in Option A results in a minimum guardband for 7 MHz that is even greater than the minimum guardband for 10 MHz. On the other hand, for 36 RBs in Option B, the minimum guardband for 7 MHz is only 10 kHz more than the minimum guardband for 5 MHz.

While we would certainly prefer the higher spectral efficiency of 36 RBs, we wouldn’t want the lower guard

Proposal: RAN4 should choose between 35 and 36 RBs for 7 MHz channel bandwidth.

# Summary

Proposal: RAN4 should choose between 35 and 36 RBs for 7 MHz channel bandwidth.

# References

- RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5,” China Telecom, T-Mobile USA