3GPP TSG-RAN WG4 Meeting # 114	R4-2502202

# Athens, Greece, 17th – 21st February, 2025

Source:			T-Mobile USA, Verizon

Title:			NTN S-Band UE-UE coexistence

Agenda item:		6.18.2

Document for:		Discussion

# Introduction

We have previously submitted several discussion papers on the topic of NTN S-Band coexistence, including [1], [2], and [3]. In this paper we look at the impact of various spurious emissions levels from and NTN S-band UE into a Band2/n2/Band25/n25 victim UE.

# Discussion

## UE-UE distances based on 1999/2008 assumptions

If we use the same assumptions from the 2008 LTE UE-UE coexistence paper from Motorola, namely -3dBi antenna gain and 6 d dB per UE of antenna+body loss for sub – GHz, we get the following:

With the baseline spurious emission protection level of -50 dBm, the UE-UE distance is just under 1 meter for 3 dB of degradation. With a relaxed value of -40 dBm/MHz, the distance is 2.7 meters. With a further relaxed values of -30 dBm/MHz, the distance is 8.5 meters. If, only the minimum -13 dBm/MHz that is provided by the SEM for wider channel bandwidths is provided, the UE-UE distance is over 60 meters.

Observation 1: With the 2008 assumptions of – 3dBi antenna gain and 6 dB per UE of head/body loss, the UE-UE distance for 3dB degradation with -13 dBm/MHz spurious emissions is 60 meters.

## UE-UE distances based on ITU 4 dB head/body loss

An important thing to consider is that the 6 dB head/body loss per UE was based on the assumption that a phone was being held between a hand and a head, as shown in [4]:

While that may have been the case in 2008, it is probably not the typical use scenario in 2025.

If we use the 4 dB of head/body loss per UE that the ITU uses, we calculate the following:

As can be seen above, over 95 meters of UE-UE separation is needed if the spurious emission level in the victim band is -13 dBm/MHz.

Observation 2: With the ITU assumptions of – 3dBi antenna gain and 4 dB per UE of head/body loss, the UE-UE distance for 3dB degradation with -13 dBm/MHz spurious emissions is 95 meters.

## UE-UE distances based on ITU 0 dBi antenna and 0 dB head/body loss

In order to maximize the uplink link budget, NTN UEs are likely to do all they can to optimize the antenna gain, and minimize head/body loss. 0 dBi antenna gain is not out of the question, and 0 dB head/body loss is possible with selectable Tx antennas. If we redo the calculations with 0 dBi and 0 dB loss, the EU-UE distance at -13 dBm is 479 meters.

Observation 3: With the more aggressive assumptions of 0 dBi antenna gain and 0 dB per UE of head/body loss, the UE-UE distance for 3dB degradation with -13 dBm/MHz spurious emissions is 479 meters.

## UE-UE distances based on ITU 0 dBi antenna and 0 dB head/body loss and 6 dB below sensitivity

Now, if we further consider that 3 dB vict8im receiver degradation could be considered to be unacceptable, if we look at keeping the interferer power 6 dB below sensitivity for 1 dB degradation, the required separation distance with -13 dBm/MHz is nearly a kilometer!

Observation 4: With the more aggressive assumptions of 0 dBi antenna gain and 0 dB per UE of head/body loss, the UE-UE distance for 1 dB degradation with -13 dBm/MHz spurious emissions is 956 meters.

Observation 5: Clearly -13 dBm/MHz is not sufficient protection for UE-UE coexistence. Depending on the assumptions, the minimum UE-UE distance can vary from 60 meters up to nearly a kilometer.

## SEM for 5, 10, 15 and 20 MHz channels

The protection provided by the SEM alone is a function of the uplink channel bandwidth. The following show the SEM below the S-Band for 5, 10, 15 and 20 MHz with the channel located at the bottom of the S-Band:

Figure 1 5 MHz SEM with Fc=2002.5 MHz

Figure 2 5 MHz SEM with Fc=2007.5 MHz

Figure 3 10 MHz SEM with Fc=2005 MHz

Figure 4 10 MHz SEM with FC=2015 MHz

Figure 5 15 MHz SEM with Fc=2007.5 MHz

Figure 6 20 MHz SEM with Fc=2010 MHz

Observation 6: The SEM only provides -13 dBm/MHz protection a channel bandwidth away from the low edge of the channel, which could be as low as 1980 MHz with a 20 MHz uplink channel.

Observation 7: The SEM would provide -30 dBm/MHz protection for the entire n2/n25 for 5 MHz channels with a center frequency of 2007.5 or higher, or for 10 MH channels with Fc at 2015 MHz with no A-MPR.

## Proposals

Proposal 1: RAN4 should choose a protection level that can guarantee reasonable protection, while not excessively impacting the NTN n252 operations.

Proposal 2: The protection level for n252 into the n25 downlink should be -30 dBm/MHz without NS signalling.

# Summary

Observation 1: With the 2008 assumptions of – 3dBi antenna gain and 6 dB per UE of head/body loss, the UE-UE distance for 3dB degradation with -13 dBm/MHz spurious emissions is 60 meters.

Observation 2: With the ITU assumptions of – 3dBi antenna gain and 4 dB per UE of head/body loss, the UE-UE distance for 3dB degradation with -13 dBm/MHz spurious emissions is 95 meters.

Observation 3: With the more aggressive assumptions of 0 dBi antenna gain and 0 dB per UE of head/body loss, the UE-UE distance for 3dB degradation with -13 dBm/MHz spurious emissions is 479 meters.

Observation 4: With the more aggressive assumptions of 0 dBi antenna gain and 0 dB per UE of head/body loss, the UE-UE distance for 1 dB degradation with -13 dBm/MHz spurious emissions is 956 meters.

Observation 5: Clearly -13 dBm/MHz is not sufficient protection for UE-UE coexistence. Depending on the assumptions, the minimum UE-UE distance can vary from 60 meters up to nearly a kilometer.

Proposal 1: RAN4 should choose a protection level that can guarantee reasonable protection, while not excessively impacting the NTN n252 operations.

Proposal 2: The protection level for n252 into the n25 downlink should be -30 dBm/MHz without NS signalling.

# References

- R4-2413305, “NTN S-Band coexistence,” RAN4#112, T-Mobile USA

- R4-2416507, “NTN S-Band UE-UE coexistence with TN,” RAN4#112bis, T-Mobile USA, AT&T, US Cellular

- R4-2419375, “NTN S-Band UE-UE coexistence,” RAN4#113, T-Mobile USA, US Cellular, Verizon, AT&T and Deutsche Telekom

- R4-080710, “TS36.101: TP for UE Spurious emission limits,” Motorola