3GPP TSG-RAN WG4 Meeting #114                                   	R4-2502205

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:			7.11.6 RRM core requirements

Source:	Eutelsat Group

Title:	UL Timing modifications to enable mobile VSAT served by NGSO

Document for:	Discussion

# Introduction

Following the decision in [1] to define support for mobile VSAT in the Ku bands WI, this paper further develops the areas of the RRM specifications which have been identified from [2].

The Rel-18 WI NR_NTN_enh added support for Fixed and Mobile VSAT above 10 GHz served by GSO (Geosynchronous Orbit) satellites. For NGSO, only Fixed VSAT was specified due to the expectation that Mobile VSAT to fast-moving NGSO (Non-Geosynchronous Orbit) satellites would be more challenging. Now that the Rel-18 WI has completed, it has become clearer what additional work is required to specify Mobile VSAT served by NGSO satellites. This paper outlines these issues with a view to including Mobile VSAT to NGSO support within Rel-19.

# Rel-18 Background on NTN Uplink Timing

The Rel-18 NR_NTN_enh work item augmented the RRM core requirements for UL timing to accommodate satellite geometry and velocity, which are vastly different from the terrestrial baseline.

For TN, the UL timing is defined in TS 38.133 sub-clause 7.1.1 as:

### 7.1.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state or when transmitting PUSCH on CG resources for SDT in RRC_INACTIVE. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG.

Where:

Timing offset between uplink and downlink radio frames at the UE, as defined in clause 4.2 in TS 38.213.

Fixed timing advance offset, as defined in clause 7.1.2 in TS 38.133.

This is a closed loop requirement that assumes the BS is static and the UE velocity is low. Figure 16.14.2.1.1 from [3] illustrates satellite geometry with the two additional open loop uplink timing parameters added.

For NTN, the UL timing includes two additional open loop parameters and is defined in TS 38.133 as:

### 7.1C.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state. The uplink frame transmission takes place NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

The terms are defined in 38.211 as

NTA	Timing advance between downlink and uplink; see clause 4.3.1

NTA,offset	A fixed offset used to calculate the timing advance; see clause 4.3.1

NTA,adjcommon	Network-controlled timing correction; see clause 4.3.1

NTA,adjUE	UE-derived timing correction; see clause 4.3.1

38.211 4.3.1 further defines NTA,adjcommon as:

NTA,adjcommon given by clause 4.2 of [5, TS 38.213] is derived from the higher-layer parameters ta-Common, ta-CommonDrift, and ta-CommonDriftVariant if configured, otherwise NTA,adjcommon=0;

NTA,adjcommon is a semi-static timing offset to account for the delay on the feeder link between the gateway on the ground and the satellite, which is common to all UE served by that satellite

38.211 4.3.1 further defines NTA,adjUE as:

NTA,adjUE given by clause 4.2 of [5, TS 38.213] is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

NTA,adjUE is a UE-specific delay, computed by the UE, which represents the delay on the service link between the satellite and the UE. This is calculated by the UE using the ephemeris information provided by the network in SIB19, the satellite propagator in the UE and the UE’s knowledge of its own position based on GNSS and knowledge of its own velocity between GNSS fixes.

Rel-18 defined core requirements for Fixed VSAT and Mobile VSAT served by GSO satellites. The geometry for GSO mean that the required rate of UL timing adjustment due to satellite and VSAT movement is very small.

Rel-18 also defined core requirements for Fixed VSAT to NGSO. In this geometry, the velocity of the satellite relative to the VSAT is large, with orbital speeds in the range of 7 km/s. This is a much more challenging scenario than GSO. The solution to this continual change in relative path length is mitigated by the UE, which uses the satellite ephemeris in SIB19 along with a propagator in the UE, to predict the satellite position relative to the UE position at the point of transmission. Using this information the UE calculates NTA such that the UL transmission meets the core requirements for Te_NTN given in TS 38.133 table 7.1C.2-3.

# Specifying Mobile VSAT Uplink Timing to NGSO

Rel-18 did not define core requirements for Mobile VSAT served by NGSO. This scenario adds one further variable to the currently specified Fixed VSAT served by NGSO, which is the velocity of the VSAT. Although this is a more difficult scenario, the geometry indicates it is a far closer to the Fixed VSAT served by NGSO case than the already specified Mobile VSAT to GSO case. Most of the relative velocity between the NGSO and the VSAT is due to the orbital speed of the NGSO satellite, which is around 25,000 km/h. By contrast, the maximum speed of a Mobile VSAT is expected to be less than 1,500 km/h for an airborne platform.

The current specifications for Fixed VSAT served by NGSO allow for the UE to take account of the movement of the satellite. From TS 38.133 7.1C.2.1

#### 7.1C.2.1 Gradual timing adjustment When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN. 2) The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last one second, shall be Tp_NTN per second. 3) The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200 ms, shall be Tq_NTN per 200 ms.

It is assumed that the UE component of the reference timing in NTA,adjUE is as per the Fixed VSAT case, i.e. based on static GNSS positioning only, and excludes potential movement of the VSAT. This is consistent with the term “apart from a change of NTA,adjUE due to satellite position update” since for a Fixed VSAT, there will be no dynamic component of the UE position that could contribute to a change in NTA,adjUE.

The situation for mobile VSAT adds the dimension of significant UE movement. In this case the definition of NTA,adjUE which is based partly on the UE position, will include the UE GNSS position (fix) and the possibility that the UE can use knowledge of its own velocity to calculate its actual position between GNSS fixes. If the UE uses its interpolated position to calculate NTA,adjUE while moving at high speed, then it should in normal circumstances be able to keep its timing error below the limit of Te_NTN with the additional support of the closed loop TAC. When a new GNSS fix is achieved, the UE should have anticipated the new position using its propagator such that the new fix will not cause a sudden jump in the calculation of NTA,adjUE other than due to the much smaller errors in the GNSS accuracy.

If on the other hand the UE does not use its interpolated position between GNSS fixes, the closed loop TAC will be unable to keep up with the rapidly changing distance between the UE and satellite causing Te to exceed Te_NTN long before the next GNSS fix which will trigger the gradual timing adjustment procedures to slew rate limit any further changes in NTA.

The interpretation of the gradual timing advance procedures is more nuanced in the case of mobile VSAT since there is now a component of UE position change that did not exist in the Fixed VSAT case.

#### 7.1C.2.1 Gradual timing adjustment When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN. 2) The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last one second, shall be Tp_NTN per second. 3) The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200 ms, shall be Tq_NTN per 200 ms.

It is now unclear how to interpret the term “apart from a change of NTA,adjUE due to satellite position update” since this could be understood to mean the dynamic element of the UE position in NTA,adjUE is being excluded. The purpose of the slew rate limitation is to deal with the situation where the closed loop has managed to maintain Te within Te_NTN transparently but when the next GNSS fix occurs, the UE calculates a new NTA,adjUE ignoring the accumulated TAC thus generating a large and unwanted step in NTA. When this unwanted step occurs, the slew rate limitation pulls Te back to within Te_NTN in a controlled manner such that the closed loop can take over.

In a system where a mobile VSAT does not use an inerpolated position to calculate NTA,adjUE it will trigger the gradual timing adjustment very quickly, and the change in UE position will not be used which is consistent with the literal interpretation of “apart from a change of NTA,adjUE due to satellite position update” . For a mobile VSAT that does use its interpolated positon to calculate  NTA,adjUE and for some reason exceeds Te_NTN and triggers the gradual timing adjustment clause, it is then unclear if the known ongoing change in position of the UE is to be excluded from calculation of the slew rate limits.

If it remains included, then it is likely the Te will quickly return to within the Te_NTN limit but if it is excluded, then the rate of change of Te will suddenly jump. The consequence of this will depend on which side of the limit has been triggered. If Te has become too negative, so not using the UE interpolated position causes Te to rise, then the system wil quickly return to stability. But if Te were to be too large, then not using the UE interpolate position to calculate  NTA,adjUE will cause the system to move in the wrong direction with the accumulated UE position error working against the slew rate limiting. This does not sound like a desirable situation and for that reason it would be beneficial to not take the literal interpretation of “apart from a change of NTA,adjUE due to satellite position update” and allow the UE to also discount from the slew rate calculations, the known change in the UE position between GNSS fixes.

When the gradual timing advance procedure has been triggered, it is also necessary to re-evaluate the defined step sizes for Tq and Tp_NTN. Currently these are 5.5 for FR1-NTN and 2.5 for FR2-NTN.

These figures were adequate for Fixed VSAT served NGSO and GSO, and for mobile VSAT served by GSO. They represent the ability to correct for a UE speed of 1114 km/h and 507 km/h at 30 degrees elevation. For the FR1-NTN case, 1114 km/h is marginal for the anticipated speed of commercial aircraft and ti is noted that for ATG, Tq and Tp_NTN were increased to 9.5 which represents,  1924 km/h.

In the ATG case, it is not required that the moving UE is updating its position and relative timing based on GNSS and a UE propagator, therefore the changing path length will accumulate until Te_NTN is exceeded and the UE is allowed to adjust NTA to bring it back into the limit. This solution has been seen to be successful and therefore would seem appropriate to use for the FR1-NTN mobile VSAT case as a figure of 9.5 is well inside the range of Te_NTN which varies from 22 to 29 meaning the system should have good stability.

For FR2-NTN the situation is not so simple. The current step size of 2.5 represents a maximum speed of 507 km/h which is well short of what is required. A mobile VSAT that implemented interpolated positional updates between GNSS fixes could work, but if Te_NTN were to be exceeded, then the slew rate limiting would be unable to keep up with the rate of change of Te and the call would drop. As I the case fo FR1-NTN, this problem can be mitigated by increasing Tq however, for FR2-NTN, Te_NTN is much smaller, being only 7.5. In order to ensure the control loop remains stable, Tq should ideally not be less than Te_NTN although it could be double provided Tp_NTN were no more than half that. A value of 7.5 for Tq would give a maximum velocity of 1560 km/h, which may be considered sufficient.

# Conclusions

Proposal 1: Allow the Mobile VSAT to use knowledge of its own position to calculate the reference timing

This can be implemented either by implicitly allowing the UE to use knowledge of its own velocity to autonomously update NTA or this could be explicitly defined. Were the UE to autonomously use its own positional knowledge to minimize Te would be transparent to the network and implementation of this capability would only be detectable by the design of a specific test case to exclude this capability.

Alternatively, the ability of the UE to use knowledge of its own position in calculating NTA could be made explicit with the following changes to section 7.1C.2.1:

#### 7.1C.2.1	Gradual timing adjustment

When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. NTA,adjUEmay include interpolated updates between GNSS updates. All adjustments made to the UE uplink timing shall follow these rules:

- The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and UE position, and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN.

- The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and UE position, and NTA,adjcommon during the last one second, shall be Tp_NTN per second.

- 3)	The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and UE position,and NTA,adjcommon during the last 200 ms, shall be Tq_NTN per 200 ms.

With this clarification, the UE can autonomously modify NTA using explicit knowledge of its own position derived from GNSS and the on-board propagator. This is a more sophisticated solution than simply ignoring the known position change of the UE and allowing the timing to exceed Te_NTN requiring a sudden step of Tq.

Proposal 2: Increase the size of Tq to allow for Mobile VSAT velocity (airborne case).

Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq_NTN | Tp_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 9.5*64*Tc | 9.5*64*Tc |
|  | 30 | 9.5*64*Tc | 9.5*64*Tc |
|  | 60 | N/A | N/A |
| FR2-NTN | 60 | 7.5*64*Tc | 7.5*64*Tc |
|  | 120 | 7.5*64*Tc | 7.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

A CR to implment proposals 1 and 2 is provided in [4].

# References

[1] RP-243314, “Revised Ku band WID” (Eutelsat Group) RAN #106 Dec 2024

[2] RP-242986 “Technical aspects for specifying Mobile VSAT served by NGSO” (Eutelsat Group) RAN #106 Dec 2024

[3] TS 38.300 v18.3.0 NR and NG-RAN Overall Description

[4] R4-2502207 “CR to 38.133 to modify uplink timing requirements to enable mobile VSAT served by NGSO by NGSO” (Eutelsat Group) RAN4 #114 Apr 2025