3GPP TSG-RAN4 Meeting #114	R4-2502208

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.870 | CR | 0019 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.870 for an alternate TRP test procedure for XR devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRP_TRS_MIMO_OTA_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR provides content to accommodate alternative TRP test methods for XR devices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing spiral scan as an alternate test time reduction test procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is not appicable for XR UE testing by an alternative test method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Spiral scan test procedure presented in R4-2417756. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

### 9.3.1		    Spiral Scan procedure for TRP Test

According to TS 38.161 Clause A.3.3.2, the measurement is performed with a constant sampling step, e.g., 15 degrees or 30 degrees, in both theta (θ) and phi (ϕ) axes for TRP measurement.

The Spiral scan method can significantly reduce the test time for TRP measurements compared to the standard constant step size approach. This is achieved by utilizing a test setup with the distributed axes system, as specified in Clause 7.2. This procedure depends on the continuous rotation of dual-polarized measurement antenna that rotates along the measurement surface at the same time as the DUT rotates during the collection of data points. Both DUT and measurement antenna rotate continuously, and power measurement can be taken simultaneously on the measurement antenna for both the theta and phi polarizations. The measurement value and the measurement location (theta, phi) shall be recorded at every measurement point.

The Spiral scan measurement procedure adheres to the specified steps outlined in TS 38.161 Clause A.3.3.2 with the exception that it is applicable on the test system setup with the distributed axes for continuous rotation of both theta (θ) and phi (ϕ) axes for TRP measurement. The TRP value is calculated using the TRP integration approaches outlined in Clause A.3.5.1.

The spiral scan method does not require to have measurement point spacing exactly 15 degrees. However, equal spacing of the test points shall be achieved using following criteria:

- The maximum theta-axis rotation between successive spiral turns at the same phi angle shall be 15 degrees for each polarization.

- The maximum distance in phi-axis between adjacent points shall be 15 degrees for each polarization.

- The minimum number of data points shall be 350, oversampled relative to standard 266 measurement points for each polarization.

This alternate test method does not require additional measurement uncertainty calculations.

<<< END OF CHANGES >>>