3GPP TSG-RAN WG4 Meeting #114	R4-2502228

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.26.4.2

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for LP-WUR

Document for:	Discussion

# Introduction

RAN4 has been discussing various scenarios and issues related to RRM requirements for LP WUR/WUS over the past few meetings and achieved many agreements [2]-[6]. In this paper, we present our views on some of the open issues.

# RRM measurements relaxation

Scaling factor(s) is/are >= 4. Further down-select from

- Option 1: 1 fixed scaling factor

- Option 2: multiple values configured by NW

- Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo)

The first issue that we want to address is related to RRM measurement relaxation for serving/neighbor cell for case #3 where RAN4 made the following agreements on this issue during the last meeting:

We agreed to specify the measurement scaling factor to be greater than or equal to 4 with 8 and 16 being the favorite values among companies. Some companies raised the concern with overly long measurement delays when the DRX cycles are longer. Although a single fixed scaling factor is desirable, but we understand that this may not fit well given the range of DRX cycle lengths that we have. So, in our opinion it makes sense to specify multiple scaling factors based on the DRX cycle lengths.

Proposal 1: Specify multiple measurement relaxation factors (F) based on the DRX cycle lengths, e.g., the following values can be used:

- F = 16 for DRX = 0.32s and 0.64s

- F = 8 for DRX = 1.28s and

- F = 4 for DRX = 2.56s

# LP-WUR operating carrier frequency

RAN1 sent as LS [7] asking RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs that may not support LP-WUS reception on all the bands supported by the UE.

It was agreed that from Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency and there is an FFS for the case when MR and LR are working on different carrier frequencies if it is supported by RAN1/2.

We think it’s clear that we don’t see any issue as of now and there’s no need to discuss the FFS case.

Proposal 2: Since RAN4 assumed LR and MR are operating on the same carrier frequency to specify RRM requirements, RAN4 to not discuss the case when MR and LR are working on different carrier frequencies.

# LP-WUR operation in Connected mode

The next issue is related to WUR implementation. Although 3GPP may assume that the WUR is completely detached from the MR, it’s highly likely that UE may share some RF components between the two radios. So, for CONNECTED mode, when the UE uses measurement gaps for neighbour cell measurements, it may tune away from the serving cell. This typically causes interruptions on the UE where the UE cannot transmit or receive any signal other than those intended for RRM measurements. We think such interruptions may also be needed for WUS monitoring when the WUS MO collides with the MG.

Observation 1: In the Connected mode, the UE may not be able to monitor WUS during MGs as it may retune to a different carrier frequency for measurements.

Proposal 3: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs.

# Conclusion

Proposal 1: Specify multiple measurement relaxation factors (F) based on the DRX cycle lengths, e.g., the following values can be used:

- F = 16 for DRX = 0.32s and 0.64s

- F = 8 for DRX = 1.28s and

- F = 4 for DRX = 2.56s

Proposal 2: Since RAN4 assumed LR and MR are operating on the same carrier frequency to specify RRM requirements, RAN4 to not discuss the case when MR and LR are working on different carrier frequencies.

Observation 1: In the Connected mode, the UE may not be able to monitor WUS during MGs as it may retune to a different carrier frequency for measurements.

Proposal 3: RAN4 to introduce interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. The interruption on WUS monitoring follows the current interruption requirements on DL data during MGs.

# References

[1] 	RP-240801, “Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR)”, vivo, RAN#103

[2] R4-2406367, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#110bis

[3] R4-2410296, “WF for RRM requirements for LP-WUSWUR”, vivo, RAN4#111

[4] R4-2413899, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112

[5] R4-2416861, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#112bis

[6] R4-2420100, “WF for RRM core requirements for LP-WUSWUR”, vivo, RAN4#113

[7] R1-2407559, “LS on LP-WUS operation in IDLE/INACTIVE mode”, Apple, RAN1#118