3GPP TSG-RAN WG4 Meeting #114	R4-2502229

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.28.2

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for XR enhancements

Document for:	Discussion

# Introduction

For release 19, RAN4 started working on the new work item on XR enhancements (Phase 3) for NR where the following WID [1] objective is expected to have impact on RRM requirements:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

RAN1 has been discussing mechanisms to enable transmission/reception in gaps/restrictions that are caused by RRM measurements and have reached the following working assumption as conveyed in the LS [2]:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

In this contribution paper, we discuss the impact of the working assumption made by RAN1, on RRM requirements and propose solutions to address them.

# Impact on RRM requirements due to MG deactivation

Firstly, although RAN1 has agreed upon the dynamic indication to enable Tx/Rx in particular gaps, RAN2 may further consider semi-static indication schemes as well, e.g., RRC based MG deactivation pattern. We understand that due to unpredictable nature of the XR data, dynamic indication is better suited from signaling perspective but at the same time, from UE implementation perspective, it may be quite challenging to deactivate a single MG occasion as this needs extensive processing on the modem SW and RF procedures. RRC/MAC-CE based semi-static indications enables the UE to handle MG deactivations in an efficient manner.

Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC or MAC-CE based signaling as well.

Coming back to the DCI based dynamic indication of MG deactivation, few key things to consider in RAN4 for RRM requirement specification are:

- Minimum time offset(s) X between indication to skip and skipped measurement occasion

- Measurement occasions that can be deactivated

- Impact on measurement delay/accuracy requirements

For the first issue, i.e., the minimum offset X between the indication to skip and the measurement occasion to be skipped, RAN4 has agreed to X = 5 ms as a baseline value and X = 3ms based on optional UE capability.

A component of an FG can have Option 1 as a first value and Option 2 as a second value:

- Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI.

- Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Note: UE indicates only one value of this component.

Furthermore, RAN1 discussed the UE behavior for subsequent DCI for the same measurement gap occasion and made the following agreement:

For UEs indicating Option 1 above, UE may simply ignore the MG cancellation indications in all the subsequent DCIs for the same MG occasion. However, for UEs indicating Option 2, our understanding is that the UE will process and honor the subsequent DCIs and if indicated ‘0’, the UE will not cancel the MG occasion. This behavior impacts UE implementation as the procedure to cancel the MG occasion may already have started. Furthermore, RAN1 doesn’t talk about the timeline for such DCI indicating ‘0’. If this indication is received very close the MG occasion, it may not be possible to revert the procedure. We think it’s important to establish the timeline to honor the subsequent DCI indication in Option 2 and this timeline should be no shorter than X (5ms or optional 3ms).

Proposal 2: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI atleast X ms before the MG occasion, where

- X=5ms as baseline and X=3ms as optional UE capability.

- [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

RAN4 already started an initial discussion on the second issue, i.e., the measurement occasions that can be deactivated. There are three main types of measurements considered here

- L3 measurements with gaps

- L3 measurements outside gaps

- L1 measurements

Among these, L3 measurements with gaps are already agreed to be considered for the DCI based deactivation since MGs cause long interruption and may significantly delay the XR traffic. L3 measurements outside gaps and L1 measurements cause scheduling restrictions and it is being discussed whether such measurements can be cancelled by the DCI.

Most of the L1 measurements cause scheduling restrictions on a symbol level (typically SSB symbols and additional one or two symbols). Since these restrictions are very small, deactivation of such measurements may not bring much gains but will complicate the specification work considering various requirements and scenarios. Furthermore, the L1 measurements are performed with the periodicity of the RS. So receiving a separate DCI for each of these occasions cause a significant overhead on the UE processing bringing minimal benefits. With limited TU budget, RAN4 may not have sufficient time to finish all the work. So, considering the marginal gains that comes with L1 measurement deactivation and limited TUs, we think deactivation of L1 measurements can be deprioritized.

On the similar lines, measurement outside gaps are performed within SMTCs and the following scheduling restrictions are defined

- On SSB symbols to be measured and 1 data symbol before and after SSB​

  - FR1 with support of simultaneousRxDataSSB-DiffNumerology ​

  - FR1 without support of simultaneousRxDataSSB-DiffNumerology with deriveSSB_IndexFromCell enabled​

  - FR2 with deriveSSB_IndexFromCell enabled​

- On all symbols within the SMTC occasion​

  - FR1 without support of simultaneousRxDataSSB-DiffNumerology with deriveSSB_IndexFromCell disabled​

  - FR2 with deriveSSB_IndexFromCell disabled​

Similar to the scheduling restrictions for L1 measurements, measurement outside gaps also specify scheduling restrictions on a symbol level for most of the cases and the only case where we have scheduling restrictions on the entire SMTC is when the UE doesn’t support simultaneousRxDataSSB-DiffNumerology and/or deriveSSB_IndexFromCell is disabled​. Again, very limited use-case and almost no benefit in cancelling such measurements.

Proposal 3: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L1 measurements.

Proposal 4: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L3 measurements without gaps.

Moving on to the L3 measurements, most of the RRM measurement requirements specified in TS 38.133, such as PSS/SSS detection and SSB index identification period and measurement period, are specified as multiple of measurement gap repetition period (MGRP). Deactivation of any MG during these periods would result in UE not having sufficient measurement samples to meet the performance requirements related to delay and accuracy. One simple way to handle this would be to extend the delay requirements based on the number of deactivated MGs. A similar methodology was also followed during NR-U, where the delays were extended based on the number of LBT failures. In our opinion, from RRM requirements perspective, MG deactivation for XR can be handled in a similar manner.

Proposal 5: For DCI based dynamic indication, RAN4 to extend the L3 delays by the number of MGs deactivated during the period.

# Conclusion

Proposal 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well.

Proposal 2: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI atleast X ms before the MG occasion, where

- X=5ms as baseline and X=3ms as optional UE capability.

- [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Proposal 3: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L1 measurements.

Proposal 4: Dynamic signaling to enable Tx/Rx doesn’t apply to scheduling restrictions caused by L3 measurements without gaps.

Proposal 5: For DCI based dynamic indication, RAN4 to extend the L3 delays by the number of MGs deactivated during the period.

# References

[1] 	RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia, RAN#103

[2] R4-2407561, “LS on gaps/restrictions that are caused by RRM measurements”, Nokia, RAN1#118

[3] R4-2411007, “LS on UE assistance information”, Nokia, RAN4#112