3GPP TSG-RAN WG4 Meeting #114	R4-2502240

Athens, Greece, February 17 – 21, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SSB adaptation

Agenda item:	7.25.2.2

Document for:	Discussion

Introduction

The discussion on SSB adaptation as part of the third objective “adaptation of common signal/channel transmissions” of the Rel-19 WID on Enhancements of network energy savings for NR [1], was continued at RAN4#113 meeting [2] and the agreements were captured in the WF [3].

In this contribution, we continue the discussion on the RRM impact from SSB adaptation.

Discussion

  - Progress status in RAN4 and RAN1

At RAN4#113 meeting, the following has been agreed on RRM requirements for SSB adaptation [3].

- Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Issue 1-1-2: Scenario clarification- SSB periodicity Agreement: - Postpone the discussion and wait for RAN1 progress. Issue 1-1-3: Scenario clarification- SSB adaptation triggering Agreement: - RAN4 to focus on NW controlled SSB adaptation firstly. Issue 1-1-4: Scenario clarification- SSB adaptation mechanism Agreement: - Postpone the discussion and wait for RAN1 progress.

- Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: - Wait for LS reply on whether there is impact on SMTC.

Open issues for SSB adaptation are contained in [2] and previous RAN4 agreements are listed in Annex 1.

RAN1 agreements are listed in Annex 2.

  - Progress status in RAN4 and RAN1

In this section we consider issues related to scenarios for SSB adaptation.

## SSB adaptation in PCell

The RAN1#118 agreement on support of SSB adaptation in time domain for PCell is depicted below.

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3)

Based on the current agreement, it is proposed to consider RRM impacts from SSB adaptation for PCell, as the use case that CD-SSB is not on the sync raster is considered as corner case. On the other side, for the Rel-17 RedCap feature, it was also agreed that NCD-SSB cannot have a periodicity less than that for CD-SSB. Given that NCD-SSB can also be used for neighbor cell measurements used for direct handover between dedicated BWP of serving cell to that of target cell, it is undesirable to allow for SSB adaptation on NCD-SSB in PCell.

Proposal 1: 	RAN4 to not consider RRM impacts from SSB adaptation for PCell based on current RAN1 agreement.

## SSB adaptation in SCell

The RAN1#118 agreement on support of SSB adaptation for SCell is depicted below.

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Based on the current agreement, it is proposed to consider RRM impacts from SSB adaptation for SCell related to NCD-SSB, both on and off sync raster. For CD-SSB in SCell, RAN1 is still discussing whether to apply SSB adaptation. This may be important for e.g. cell barring purposes. Hence it is proposed that RAN4 waits on further RAN1 progress on the support of SSB adaptation for CD-SSB in SCell.

Proposal 2: 	RAN4 to consider RRM impacts from SSB adaptation for NCD-SSB in SCell based on current RAN1 agreement and to wait on further RAN1 progress on SSB adaptation for CD-SSB in SCell.

## RAN1 Reply LS

It is noted RAN1#118bis concluded that there is no support for SSB adaptation in idle mode. Hence RAN4 can stick to its RAN4#112bis agreement on the scenarios to be considered as a starting point.

RAN1#119 sent Reply LS [4] replying to RAN4’s LS [5] also on SSB adaptation on the relation in time/frequency locations and on the spatial relation between SSB before and after SSB adaptation. The extract of [4] is duplicated below.

Obj 3 Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

RAN1 hence confirmed their current agreements on:

- time location: change of SSB periodicity only is considered as use case for SSB adaptation so far, whilst other parameters are still under discussion,

- frequency location: no change of SSB frequency is considered as use case for SSB adaptation so far, and,

- spatial relation: no change of spatial relation, i.e. the SSB(s) in the burst with same SSB index(es) is (are) transmitted before and after SSB adaptation as use case for SSB adaptation so far.

The above means that SSB adaptation in time domain is supported by Rel-19 NES capable UE for SCell and NCD-SSB and is limited to SSB periodicity adaptation on the same frequency location.

Observation #1: SSB adaptation in time domain is supported by Rel-19 NES capable UE for SCell and NCD-SSB and is limited to SSB periodicity adaptation on the same frequency location.

Proposal 3: 	RAN4 to identify RRM impacts for SSB adaptation in time domain for SCell and NCD-SSB and limited to SSB periodicity adaptation on the same frequency location.

## Application to OD-SSB Case-1

Based on the current RAN1 agreement that SSB periodicity only is changed, hence no change of time offset of SSB bursts, only one SSB periodicity can be active at a time. This includes the case of alternating SSB periodicity, as was discussed for on-demand SSB in RAN1 for Case-1, namely Option 4 of the RAN1#116 agreement:

Agreement (RAN1#116) … Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. …

Regarding Option 4 in the RAN1#116 agreement, based on our understanding, it is intended for Case-1, where the NW triggers on-demand denser SSB transmission with shorter SSB periodicity on NCD-SSB from time instance A, i.e. when NW sends SCell activation command, and later NW starts sending SSB with longer periodicity after time instance B, i.e. when NW receives UE measurement report. The Option 4 can thus be jointly considered with the RAN1#117 agreement about adaptation of SSB burst periodicity in time-domain and can be considered as use case for SSB adaptation given that the frequency location remains the same.

Observation #2: Option 4 of the RAN1#116 agreement (UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.) can be considered as use case for SSB adaptation given NCD-SSB is applied, and the frequency location remains the same.

Proposal 4: 	RAN4 to study RRM impacts due to applying SSB adaptation for OD-SSB Case-1.

On the other hand, requirements for OD-SSB with SSB adaptation should not be interfering or overlap with OD-SSB requirements specified under the OD-SSB objective. This is related, for instance, to transition requirements. In our view transition requirements due to SSB periodicity adaptation for OD-SSB Case-1 should be dealt with under the SSB adaptation objective.

Proposal 5: 	RAN4 to specify requirements for SSB adaptation on OD-SSB Case-1 under the SSB adaptation objective.

## SSB periodicity range

Regarding the range of SSB periodicities, we observe that 160ms is the longest periodicity in current spec. It is noted in the RAN1 study item [6] few companies have investigated NW energy saving potential for SSB periodicities beyond 160ms, such as 640ms and 1280ms, but the majority of companies provided results for NW energy savings for SSB periodicities up to and including 160ms. Furthermore, considering the identified impact of SSB periodicity adaptation to L1 and L3 measurement delays, as depicted further below, RAN4 should maintain the existing specified range of SSB burst periodicity for SSB periodicity adaptation.

Proposal 6: 	RAN4 to maintain the specified range of SSB burst periodicities (5ms to 160ms) for SSB periodicity adaptation.

  - Layer 3 measurement impact

RAN4#112bis achieved the above listed agreement on requirements need updating. Based on this, SSB based L3 measurements in RRC CONNECTED, agreed to be affected by SSB adaptation, are identified to be the following ones:

- NR intra-frequency NR measurements,

- NR inter-frequency NR measurements.

The impact to these measurements is analyzed in this section.

According to TS 38.133, NCD-SSB based L3 measurements can be used for serving cell, intra-frequency and inter-frequency neighbor cell measurements in RRC_CONNECTED state, if the NW indicates it as reference SSB.

For NR intra-frequency measurements in RRC_CONNECTED state, RRM impacts exist for non-serving cell detection delay and the measurement delay for known non-serving cell, as listed below in further detail, referring to existing RRM requirements in TS 38.133.

Intra-frequency cell detection

Requirements for intra-frequency cell detection, i.e. for intra-frequency PSS/SSS detection in RRC_CONNECTED state without measurement gaps, are based on the configured period for the SMTC window. If, due to SSB periodicity adaptation, the SMTC period (highlighted) is adjusted, which is to be confirmed by RAN2, the cell detection delay increases proportionally up to the defined limit in the max calculation as depicted in Table 9.2.5.1-1 for the case of FR1.

Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Similar requirements apply for cell detection delay for UEs indicating support for “no-gap-with-interruption”, hence also here there is an impact due to SSB periodicity adaptation.

For the case of configured measurement gap, the cell detection delay increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.2-1 for the case of FR1. Thus, similar impact is observed as for intra-frequency cell detection without measurement gaps.

It is noted, for the case of deactivated SCell, in legacy requirements, the cell detection delay is proportional to the measCycleSCell parameter as depicted in Table 9.2.5.1-4 for the case of FR1. This parameter ranges between 160ms and 1280ms, hence it is greater or equal to the largest existing SSB periodicity (160ms). Thus, there is no impact from SSB periodicity adaptation to cell detection of deactivated SCells based on use of measCycleSCell.

For cell detection using NCSG, the delay requirements are also defined proportional to SMTC period up to the defined limit in the max calculation as depicted in Table 9.2.7.1-1 for the case of FR1. Thus, there is similar impact from SSB adaptation as for cell detection using measurement gaps.

Intra-frequency Cell identification

Additionally, for the UE to perform a cell identification, a round of measurements would be needed when considering cell identification without index reading. If index reading is needed, then an additional delay for time index detection is added.

Intra-frequency Time index detection

Requirements for intra-frequency time index detection in RRC_CONNECTED state without measurement gaps, are based on the configured SMTC window. If, due to SSB periodicity adaptation, the SMTC period (highlighted) is adjusted, which is to be confirmed by RAN2, the time index detection delay increases proportionally up to the defined limit in the max calculation as depicted in Table 9.2.5.1-3 for the case of FR1.

Table 9.2.5.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

For the case of configured measurement gap, the time index detection delay increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.2-3 for the case of FR1. Thus, similar impact due to SSB periodicity adaptation is observed as for intra-frequency time index detection without measurement gaps.

It is noted, for the case of deactivated SCell without gaps, in legacy requirements, the time index detection delay is proportional to the measCycleSCell parameter as well, as depicted in Table 9.2.5.1-6 for the case of FR1. Again, there is no impact from SSB periodicity adaptation to time index detection of deactivated SCells based on use of measCycleSCell. For the case of deactivated SCell measurement based on NCSG, in legacy requirements, the time index detection delay is also proportional to the measCycleSCell value, as depicted in Table 9.2.7.1-6 for the case of FR1, and hence there is no impact from SSB periodicity adaptation to time index detection of deactivated SCells also in this case.

Intra-frequency measurements

Requirements for intra-frequency measurements in RRC_CONNECTED state without measurement gaps are based on the configured SMTC window. If, due to SSB periodicity adaptation, the SMTC period (highlighted) is adjusted, which is to be confirmed by RAN2, the intra-frequency measurement period increases proportionally up to the defined limit as depicted in Table 9.2.5.2-1 for the case of FR1.

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Similar requirements apply for intra-frequency measurement period for UEs indicating support for “no-gap-with-interruption”, hence also here there is an impact due to SSB periodicity adaptation.

For the case of configured measurement gap, the intra-frequency measurement period increases proportionally to the SMTC period up to the defined limit as depicted in Table 9.2.6.3-1 for the case of FR1, hence also here there is an impact due to SSB periodicity adaptation.

For the case of deactivated SCell, in legacy requirements the intra-frequency measurement period is proportional to the measCycleSCell value up to a defined limit as depicted in Table 9.2.5.2-3 for the case of FR1, hence here there is no impact due to SSB periodicity adaptation.

Requirements for intra-frequency measurements period in RRC_CONNECTED state using NCSG, are also based on the configured SMTC window. If, due to SSB periodicity adaptation, the SMTC period (highlighted) is adjusted, which is to be confirmed by RAN2, the intra-frequency measurement period increases proportionally up to the defined limit as depicted in Table 9.2.7.2-1 for the case of FR1, hence there is an impact due to SSB periodicity adaptation.

Inter-frequency cell detection

For NR inter-frequency cell detection in RRC_CONNECTED state, similar RRM impacts exist for non-serving cell detection delay, as listed above for intra-frequency measurements. Legacy requirements are specified under clause 9.3 of TS 38.133.

Inter-frequency measurements

For NR inter-frequency measurement in RRC_CONNECTED state, similar RRM impacts exist for the measurement period for non-serving cell, as listed above for intra-frequency measurements. Legacy requirements are specified under clause 9.3 of TS 38.133.

Thus, RAN4 needs to consider the above depicted L3 measurement impact.

Proposal 7: 	RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and NR intra-frequency time index delay, as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell.

  - Layer 1 measurement impact

RAN4#112bis achieved the agreement on requirements needing updating as listed in Annex 1. Based on this, SSB based L1 measurements in RRC CONNECTED, agreed to be affected by SSB adaptation, are identified to be the following ones:

- L1-RSRP,

- L1-SINR,

- BFD,

- CBD.

The impact to these measurements is analyzed in this section.

It is noted the above list excludes RLM requirements for good reasons as depicted here below.

SSB based radio link monitoring

SSB based radio link monitoring is performed both for PCell and PSCell serving cells.

According to the previous RAN4 #112bis agreement (see Annex1), not to consider SSB adaptation in PCell (and implicitly PSCell), if not agreed in RAN1, no impact to SSB based radio link monitoring in TS 38.133 clause 8.1.2.2 is observed. Given no consensus so far on PCell scenarios, this agreement still holds for the considered SCell scenarios.

Proposal 8: 	RAN4 to not consider the L1 measurement impact from SSB periodicity adaptation to RLM.

SSB based beam failure detection

SSB based beam failure detection is performed for all of PCell, PSCell and SCells serving cells. Hence it depicts a valid scenario for SSB periodicity adaptation in the activated SCell.

According to the minimum requirements specified in clause 8.5.2.2 for SSB based beam failure detection, the evaluation period TEvaluate_BFD_SSB for the active serving cell (including activated SCell) depends on periodicity of SSB (TSSB) in the set , configured for BFD, except for DRX cycles > 320ms, as shown in Table 8.5.2.2-1 for serving cell in FR1. FR2 in Table 8.5.2.2-2 has similar requirements for the evaluation period being multiplied with the scaling factor N for configured number of TRP beams.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 ´ P) ´ TSSB) |
| DRX cycle ≤ 320ms | Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)) |
| DRX cycle > 320ms | Ceil(5 ´ P) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

For relaxed measurement criteria, clause 8.5.2.4 specifies the evaluation period TEvaluate_BFD_SSB for the active serving cell (including active SCell) depending on periodicity of SSB in the set , configured for BFD, as shown in Table 8.5.2.4-1 for serving cell in FR1. FR2 in Table 8.5.2.4-2 has similar requirements, taking into account the scaling factor N.

Table 8.5.2.4-1: Evaluation period TEvaluate_BFD_SSB_Relax for FR1

| Configuration | TEvaluate_BFD_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX, TSSB) ≤ 80 ms | Max(50 ´ K3, Ceil(7.5 ´ K1 ´ P) ´ Max(TDRX,TSSB)) |
| 80ms＜Max(TDRX, TSSB) ≤ 160 ms | Max(50, Ceil(7.5 ´ P) ´ Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length and no longer than 80ms. Note 2: K1 is the relaxation factor. K1 = 2 for 40ms＜Max(TDRX, TSSB) ≤ 80 ms, K1 = 4 for Max(TDRX, TSSB) ≤ 40 ms Note 3: K3 is the relaxation factor for the lower bound. K3 = K1, if 1 < K1 ≤ 2; K3 = 1 otherwise. |  |

Similar requirements exist for TRP specific SSB beam failure detection in clause 8.18.2.

SSB based candidate beam detection

SSB based candidate beam detection is performed for all of PCell, PSCell and SCells serving cells. Hence it depicts a valid scenario for SSB periodicity adaptation in the activated SCell.

According to clause 8.5.5.2 for SSB based candidate beam detection requirements, the evaluation period TEvaluate_CBD_SSB for the active serving cell (including active SCell) depends on periodicity of SSB (TSSB) in the set , configured for CBD, except for DRX cycles > 320ms, as shown in Table 8.5.5.2-1 for serving cell in FR1. FR2 in Table 8.5.5.2-2 has similar requirements, taking into account the scaling factor N.

Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(3 ´ P ´ PCBD) ´ TSSB) |
| DRX cycle > 320ms | Ceil(3 ´ P ´ PCBD) ´ TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

Similar requirements exist for TRP specific SSB based candidate beam detection in clause 8.18.5.

SSB based L1-RSRP measurements

According to clause 9.5.4.1 for SSB based L1-RSRP reporting requirements, SSB based L1-RSRP measurement period depends on periodicity of SSB (TSSB), as shown in Table 9.5.4.1-1 for serving cell in FR1. FR2 in Table 9.5.4.1-2 and group-based beam reporting in FR2 in Table 9.5.4.1-2A have similar requirements, taking into account the scaling factor N.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

For other requirements, e.g. high-speed channels, the same dependency is observed.

Other L1-RSRP measurement dependent procedures

Other L1-RSRP measurement dependent procedures are also impacted from SSB periodicity adaptation, as listed below.

- Measurement restriction for CSI-RS and SSB for L1-RSRP measurement (clause 9.5.5)

- The UE is required to perform SSB and CSI-RS measurements with measurement restrictions. If CSI-RS and SSB are in the same symbol, there is an overlap of SSB and CSI-RS reference symbols.

- Scheduling availability of UE during L1-RSRP measurement (clause 9.5.6)

- The UE is performing L1-RSRP measurement on serving cell and receiving PDCCH/PDSCH from serving cell and/or cell(s) with different PCI.

Although SSB periodicity adaptation will impact both measurement restriction and scheduling availability performance due to the SSB frequency, there is no formal specification impact.

SSB based L1-SINR measurements

According to clause 9.8.4.2 for L1-SINR reporting requirements with SSB based CMR and dedicated IMR being configured, SSB based L1-SINR measurement period depends on periodicity of SSB (TSSB), as shown in Table 9.8.4.2-1 for serving cell in FR1. FR2 in Table 9.8.4.2-2 has similar requirements, taking into account the scaling factor N.

Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

For other requirements, e.g. high-speed channels, the same dependency is observed.

Other L1-SINR measurement dependent procedures

Other L1-SINR measurement dependent procedures are also impacted from SSB periodicity adaptation, as listed below.

- Measurement restriction for SSB based L1-SINR measurement (clause 9.8.5)

  - in case of overlap of SSB and CSI-RS reference symbols

- Scheduling availability of UE during L1-SINR measurement (clause 9.8.6)

  - For measurements with different SCS than PDSCH / PDCCH in FR1 or for measurements in FR2

As for L1-RSRP measurement, although SSB periodicity adaptation will impact both measurement restriction and scheduling availability performance for L1-SINR due to the SSB frequency, there is no formal specification impact.

Proposal 9: 	RAN4 to consider the L1 measurement impact from SSB periodicity adaptation to measurement delay for BFD, CBD, L1-RSRP and L1-SINR for active SCells.

  - Transition requirements

## L1 requirements

RAN4#113 achieved agreements related to L1 transition requirements as summarized in section 2.1 and discussed the following open issue.

Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements.

Based on the analysis in section 2.4, following L1 requirements are applicable for adjustment of L1 evaluation / measurement periods according to the above agreement for issue 1-2-1:

- L1-RSRP,

- L1-SINR,

- BFD,

- CBD.

Proposal 10: 	RAN4 to consider transition requirements for L1-RSRP, L1-SINR, BFD and CBD requirements.

## L3 requirements

RAN4#113 achieved the following agreement [3] and discussed the recommended way forward in [2] related to L3 transition requirements.

- Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: - Wait for LS reply on whether there is impact on SMTC. Recommended WF: - Wait for LS reply on whether there is impact on SMTC. - Discuss whether to define requirements at transition for L3 measurement base on LS reply.

In fact, at RAN4#112bis, an LS was sent to RAN2 [7] asking whether it is foreseen to define a new SMTC configuration for the purpose of SSB adaptation. RAN2 is in charge of responding on this matter – the discussion still ongoing in RAN2. Thus, L3 requirements impact related to transition periods due to SSB adaptation can only be assessed, once RAN2 has clarified the impact to the SMTC configuration. So, we agree with the recommended WF.

On the other side, L3 transition requirements need also to be specified for OD-SSB Case-1 using SSB periodicity adaptation. Thus, RAN4 needs to consider also this scenario, no matter if RRC or MAC/CE or DCI based signaling is used indicating the new SSB periodicity.

Proposal 11: 	RAN4 to consider L3 transition requirements due to SSB periodicity adaptation for single always-on SSB and for OD-SSB Case-1, respectively.

  - SCell activation requirements

RAN4#113 discussed the impact to SCell activation due to SSB adaptation and a recommended way forward was provided [2].

- Sub-topic 1-4 SSB adaptation – Other requirements impacts Issue 1-4-1: SCell activation requirement for SSB adaptation - Proposals - Option 1: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching (CATT, Ericsson, Vivo, Samsung, OPPO) - Option 1: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. (CATT) - Option 1a: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. (Ericsson) - Option 1b: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. (Vivo) - Option 1c: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception. (Samsung) - Option 1d: RAN4 not to define L3 measurement based on SMTC periodicity before and during SCell activation if OD-SSB is triggered before the complete of SCell activation. (Oppo) - Option 2a: (CTC) - After SSB adaptation, for L1 measurement during SCell activation, UE shall perform SCell activation based on new SSB configurations, e.g. new SSB periodicity (CTC) - After SSB adaptation, for L3 measurement during SCell activation, wait for the conclusions on SMTC impact from other WGs. (CTC) - Option 2b: (Huawei) - UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation. Deprioritize the requirements when SSB adaptation happens during SCell activation. - Detailed triggering on SSB adaptation depends on further RAN1 discussion. - Option 2c: RAN4 to study the SCell activation and deactivation delay impact. (Samsung) - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements - Recommended WF: - No requirements when SSB adaptation happens during SCell activation. - FFS the requirements impact when SSB adaptation happen before or at the same time as SCell activation considering.

For a deactivated SCell, the measurement period is a multiple of the parameter measCycleSCell, as depicted in section 2.3. When an SCell is going to be activated, the network may decide to lower the SSB periodicity to allow faster L3 measurements, which the UE reports to the gNB, for deciding whether to send the SCell activation command. The SSB adaptation is indicated by network prior to SCell activation. During SCell activation, no SSB periodicity adaptation is foreseen. The SSB periodicity adaptation can be again performed after SCell activation is complete. Thus, no RRM impact during SCell activation is considered due to SSB adaptation.

Proposal 12: 	RAN4 to not consider SSB periodicity adaptation during SCell activation.

  - TCI state switching requirements

RAN4#113 discussed the impact to TCI state switching due to SSB adaptation and a recommended way forward was provided [2].

Issue 1-4-2: TCI state switching requirements for SSB adaptation - Proposals - Option 1: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. (CATT) - Option 1a: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. (Ericsson) - Option 1b: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. (Vivo) - Option 2: RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for active SCells for BFD and CBD, and to TCI state switching delay. (Nokia) - Recommended WF: - No requirements when SSB adaptation happens during TCI state switching.

L1 measurement impact to TCI state switching delay due to change of SSB periodicity is described below.

- Active downlink TCI state switching delay (e.g. clause 8.10.3)

  - The main impact due to SSB adaptation to DL TCI state switch delay is identified due to the parameter Tfirst-SSB being the time to first SSB transmission after MAC-CE / RRC based TCI state switch command has been received and decoded by the UE. This time will be extended or shortened due to longer or shorter SSB periodicity, respectively, provided the RS is SSB.

- Active uplink TCI state switching delay (e.g. clause 8.16.3)

  - The main impact from SSB adaptation to UL TCI switch delay is for unknown target TCI state due to L1-RSRP measurement. In particular, the UE shall be able to transmit uplink signal with the target TCI state within the TCI state switching delay, which depends for FR2 on the L1-RSRP measurement time for Rx beam refinement, where L1-RSRP may be based on SSB.

Based on RAN1’s reply [4], that there is no change to spatial relation for the same SSB index and no change to actually transmitted SSBs, it is assumed that there is no need for TCI state switching during SSB adaptation. Hence no L1 measurement impact from SSB adaptation to TCI state switching delay needs to be considered in RAN4.

Proposal 13: 	RAN4 to not consider an impact from SSB periodicity adaptation to TCI state switching delay.

  - SSB adaptation delay requirements

RAN4#113 did not further discuss the impact to other requirements due to SSB adaptation as provided in the recommended way forward [2].

Issue 1-4-3: Other requirements impacts by SSB adaptation - Proposals - Option 1: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity. (Nokia) - Option 2: RAN4 to discuss the transition period requirements by SSB periodicity adaptation. (Samsung) - FFS: Requirements on interrupts at state transitions - Recommended WF: - Discuss above proposal

In fact, as SSB adaptation represents a network-controlled procedure, as agreed under issue 1-1-3, see section 2.1, the UE needs to process the signaling message, depending on the signaling method, which will be defined by RAN1 and RAN2 (e.g., based on DCI, MAC-CE or RRC). Hence some processing time needs to be defined, until the UE is ready to receive SSBs with the adjusted SSB burst periodicity, which can be identified as SSB adaptation delay.

Proposal 14: 	RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1/RAN2, to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay.

- Conclusion

In this contribution, as part of the objective on “adaptation of common signal/channel transmissions” of the Rel-19 WID [1] to further enhance network energy savings for NR, we discussed scenarios for investigating the RRM impact from SSB periodicity adaptation in time domain based on RAN1/2 progress and RAN4#112 agreements and presented our views on the potential RRM impact.

Following observations are made.

Observation #1: SSB adaptation in time domain is supported by Rel-19 NES capable UE for SCell and NCD-SSB and is limited to SSB periodicity adaptation on the same frequency.

Observation #2: Option 4 of the RAN1#116 agreement (UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.) can be considered as use case for SSB adaptation given NCD-SSB is applied, and the frequency location remains the same.

And following proposals are made.

Scenarios for SSB adaptation

Proposal 1: 	RAN4 to not consider RRM impacts from SSB adaptation for PCell based on current RAN1 agreement.

Proposal 2: 	RAN4 to consider RRM impacts from SSB adaptation for NCD-SSB in SCell based on current RAN1 agreement and to wait on further RAN1 progress on SSB adaptation for CD-SSB in SCell.

Proposal 3: 	RAN4 to identify RRM impacts for SSB adaptation in time domain for SCell and NCD-SSB and limited to SSB periodicity adaptation on the same frequency location.

Proposal 4: 	RAN4 to study RRM impacts due to applying SSB adaptation for OD-SSB Case-1.

Proposal 5: 	RAN4 to specify requirements for SSB adaptation on OD-SSB Case-1 under the SSB adaptation objective.

Proposal 6: 	RAN4 to maintain the specified range of SSB burst periodicities (5ms to 160ms) for SSB periodicity adaptation.

Layer 3 measurement impact

Proposal 7: 	RAN4 to consider the L3 measurement impact from SSB periodicity adaptation to NR intra-/inter-frequency cell detection delay and NR intra-frequency time index delay, as well as to NR intra-/inter-frequency measurement period, i.e. the impact to configured SMTC period in case of active SCell.

Layer 1 measurement impact

Proposal 8: 	RAN4 to not consider the L1 measurement impact from SSB periodicity adaptation to RLM.

Proposal 9: 	RAN4 to consider the L1 measurement impact from SSB periodicity adaptation to measurement delay for BFD, CBD, L1-RSRP and L1-SINR for active SCells.

Transition requirements

Proposal 10: 	RAN4 to consider transition requirements for L1-RSRP, L1-SINR, BFD and CBD requirements.

Proposal 11: 	RAN4 to consider L3 transition requirements due to SSB periodicity adaptation for single always-on SSB and for OD-SSB Case-1, respectively.

SCell activation requirements

Proposal 12: 	RAN4 to not consider SSB periodicity adaptation during SCell activation.

TCI switching requirements

Proposal 13: 	RAN4 to not consider an impact from SSB periodicity adaptation to TCI state switching delay.

SSB adaptation delay requirements

Proposal 14: 	RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity, depending on the signaling method specified in RAN1/RAN2, to be ready to receive the SSBs with the adjusted SSB burst periodicity, identified as the SSB adaptation delay.

- References

[1] 	RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson

[2]	R4-2418283, “Topic summary for [113][225] Netw_Energy_NR_enh_Part2”, Moderator (Huawei)

[3]	R4-2420242, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei

[4]	R1-2410917, “Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell“, RAN1, RAN1#119

[5]	R4-2416913, “LS on SSB relation in On-demand SSB and SSB adaptation on Scell”, RAN4, RAN4#112bis

[6]	TR 38.864, “Study on network energy savings for NR”, V18.1.0

[7]	R4-2416911, “LS on SSB adaptation”, RAN4, RAN4#112bis

- Annex 1: Previous RAN4 agreements on SSB adaptation

RAN4 has discussed the SSB adaptation enhancement in their last meetings. At RAN4#112bis meeting, the following has been agreed on RRM requirements for SSB adaptation.

### Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

At RAN4#112 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 3-1: SSB Adaptation requirements Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

- Annex 2: RAN1 agreements on SSB adaptation

RAN1 has discussed the SSB adaptation enhancement in their last meetings. At RAN1#119 meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#119) Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode).

At RAN1#118bis, the following conclusion was reached:

Agreements (RAN1#118bis) [Adaptation of SSB in time domain] Conclusion​ There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19​. Proposal 2.1.4-rev1​ For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).​ - No spec impact

At RAN1 #118 meeting, the following agreements were reached:

Agreements (RAN1#118) [Adaptation of SSB in time domain] Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

At RAN1#117 meeting, the following agreement was reached:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB