# 3GPP TSG-RAN WG4 Meeting #114	R4-2502243

Athens, Greece, 17th – 21st February 2025

Agenda Item:	7.2.3

Source:	Ericsson

Title:	On open issues for 14800 - 15350 MHz

Document for:	Discussion

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz. RAN WG4 has already replied for the 4400-4800 MHz frequency range [2], for the 7125-8400 MHz frequency range [3] and for the 14800-15350 MHz frequency range [4] with the recommended IMT technology related parameters.

This contribution discusses the other open issues of 14800 to 15350 MHz range to be captured in TR 38.922 and shares a draft LS response to questions from ITU-R.

# 2	Discussion

In the RAN4#113 meeting, there was an agreement to specify only out-of-band blocking radiated characteristics in the LS. However, it was left for future discussions where there is a need to also specify out-of-band conducted characteristics.

It has been noted in the LS [4] that the AAS BS type ((i.e. Hybrid AAS vs. OTA AAS requirements set; FR1 vs. FR2 frequency assignment) has not yet been specified for 14800 to 15350 MHz frequency range. To align with the LS response, we suggest not to specify the out-of-band conductive characteristics at this stage and if there is a need specify the requirements when the bands are defined in the WI phase.

RAN4 to specify the out-of-band conductive characteristics when 3GPP defines the band in the WI phase and mention in TR – “The out-of-band conductive requirements should be further studied in the WI phase.”

To address the other issue for in-band blocking which was mentioned as “To be specified” in the LS [4], we provide simulations to evaluate the blocking power levels.

Figure 2-1 shows the in-band blocking wideband power received at the BS receiver from the adjacent channel UL, considering both the cases when the beamforming gain is included and when only the subarray gain is considered. At the 99.99%tile, we observe the potential blocking requirement, for a BS array with 3072 AE and 4x1 subarrays, and 200 MHz channel bandwidth, would be in the range of -41 and -49 dBm. If the CDF point to consider was 99%, the requirement would be relaxed around -63 dBm and -72 dBm respectively.

Figure 2-1 UL In-band blocking wideband power

Table 2-1 In-band blocking power levels

| In-band blocking power level |  |  |
| --- | --- | --- |
|  | When including Rx BF gain + subarray gain | When including subarray gain |
| 99.99%tile | - 41 dBm | -49 dBm |
| 99%tile | - 72 dBm | -63 dBm |

Based on our simulation results, we observe in-band blocking power levels of -49 dBm at 99.99%tile depending only subarray gain is included and -41 dBm at 99.99%tile when both Rx BF gain and subarray gain are included.

In RAN4, it is not clear what methodology is being used by the companies to derive the in-band blocking signal power levels whether at the TAB connector or RIB and needs further discussion within RAN4. However, these requirements are not used by ITU-R for sharing studies and more definitive work can be performed when 3GPP defines the band in the WI phase.

RAN4 to clarify on what methodology to be used to derive the in-band blocking power levels, and at this point mention in TR – “The in-band blocking requirements should be further studied in the WI phase when the bands are defined”.

In the LS response for 14800-15350 MHz frequency range to ITU-R [4], an editorial error was identified concerning the OTA limits for IMT BS spectral mask in Table 1A. Based on the previous agreements in Way Forward for 15 GHz BS parameters [5] and the approval of the Way Forward in the Chairman notes during 3GPP RAN4#112bis meeting, a draft LS response has been added in the Annex.

RAN4 to agree to send a LS response to ITU-R with the correct OTA limits for IMT BS spectral mask, as shared in the Annex.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Based on our simulation results, we observe in-band blocking power levels of -49 dBm at 99.99%tile depending only subarray gain is included and -41 dBm at 99.99%tile when both Rx BF gain and subarray gain are included.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to specify the out-of-band conductive characteristics when 3GPP defines the band in the WI phase and mention in TR – “The out-of-band conductive requirements should be further studied in the WI phase.”

Proposal 2	RAN4 to clarify on what methodology to be used to derive the in-band blocking power levels, and at this point mention in TR – “The in-band blocking requirements should be further studied in the WI phase when the bands are defined”.

Proposal 3	RAN4 to agree to send a LS response to ITU-R with the correct OTA limits for IMT BS spectral mask, as shared in the Annex.

# References

R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

R4-2410576: “LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4#111, LSout

R4-2414449: “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4#112, LSout

R4-2420385: “LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4#112, LSout

R4-241707: “WF for 15 GHz BS parameters”, 3GPP RAN4#112bis

# Annex: Draft LS

TSG-RAN Working Group 4 (Radio) meeting #114	R4-250xxxx

Athens, Greece, 17th – 21st February 2025

Title:	Draft LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

Response to:	LS R4-2400333 on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

Release:	Rel-19

Work Item:	FS_NR_IMT_4400_7125_14800MHz

Source:	TSG RAN WG4

To:	ITU-R WP 5D

Cc:	RAN

Contact person:	Shubham Bhargava

shubham.bhargava@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN WG4 initially received the incoming LS from ITU-R Working Party 5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 (Att. 7.4 to 5D/134) and would like to thank for the opportunity to give input on this topic.

RAN WG4 has already replied for the 4400-4800 MHz frequency range (R4-2410576), for the 7125-8400 MHz frequency range (R4-2414449) and for the 14800-15350 MHz frequency range (R4-2420385). A further LS was received from ITU-R WP5D to get confirmation from TSG-RAN Working Group 4 (Radio) that the spectral mask provided in Table 1A in earlier LS (R4-2420385) is the recommended OTA limits for IMT base station spectral mask for all relevant frequency offset.

TSG-RAN Working Group 4 (Radio) thanks ITU-R Working Party 5D for giving us the opportunity to further clarify on the stated IMT technology related parameter for 14800 to 15350 MHz frequency range.

# 2	Actions

To ITU-R WP5D

ACTION: 	3GPP RAN WG4 asks ITU-R WP 5D to consider the following clarification provided for 14800 to 15350 MHz in the below Annex.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #114bis		Wuhan, China				07-11 April 2025

TSG-RAN4 Meeting #115		St. Julians, Malta				19-23 May 2025

TSG-RAN4 Meeting #116		TBC, India					25-29 August 2025

# ANNEX

The spectral mask of AAS IMT base station provided in the previous LS response for frequency range 14800 to 15350 MHz was an editorial error. As per the agreements in Way Forward for 15 GHz BS parameters (R4-241707) and the approval of the Way Forward in the Chairman notes during 3GPP RAN4#112bis meeting, the recommended OTA limits for IMT base station spectral mask for all relevant frequency offset has been reproduced in Table 1 below.

TABLE 1

AAS BS Spectral mask (Operating band unwanted emissions limits) for 14800 - 15350 MHz operation (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 50MHz | -7dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz £ Df < min(100 MHz, Dfmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | -13 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |