3GPP TSG-RAN4 Meeting #114	R4-2502248

# Athens, GR, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0154 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Core) Maintenance CR on 38.101-5 - Removing NTN 256QAM from Rel-17 FR1-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 256QAM is a very unrealistic scenario for NTN never agreed previously in any RAN4 meetings in the context of NTN (in Rel-17, Rel-18 or even on-going Rel-19). Moreover, FR1-NTN S/L bands were introduced for smartphone applications and NB-IoT NTN, This mistake has been further propagated through the specification and Rel-18 defined even requirements on 256QAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cleaning up the specification by removing 256QAM, only for the Annexes A.2.2.5 and A.2.2.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unrealistic scenario, 256QAM should be removed from requirements and related tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.2.5, A.2.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change for TS 38.101-5>>

### A.2.2.5	DFT-s-OFDM 256QAM

The reference channel for DFT-s-OFDM 256QAM is not defined in this release of specification.

## <<End of Change>>

## <<Start of Change for TS 38.101-5>>

### A.2.2.9	CP-OFDM 256QAM

The reference channel for CP-OFDM 256QAM is not defined in this release of specification.

## <<End of Change>>