3GPP TSG-RAN WG4 Meeting # 114	R4-2502253

# Athens, GR, 17th – 21th February, 2025

Source:	LG Electronics

Title:	Discussion on RF requirement for Type 1-C Ambient-IoT BS

Agenda item:	7.24.2.1

Document for:	Discussion

Introduction

In 3GPP RAN4 #113, WF[1] was approved. In this paper, we provide our views on RF requirement for Type 1-C Ambient-IoT BS.

- Issue 2-4: TX Agreement: - For R2D transmission requirements, LP-WUS requirements can be considered as reference. - Whether requirements for A-IoT device RX RF potentially needed or not is summarized in the following table: RF Requirement for A-IoT BS- TX requirements; Potentially needed or not. Base station output power; YES. Output power dynamic; RE power control dynamic range; TBD in WI phase. Total power dynamic range; TBD in WI phase. Power boosting; NO. Some requirements like Transmit ON/OFF power; Transmitter OFF power; TBD in WI phase. Transmitter transient period; TBD in WI phase. Transmit signal quality; Frequency error; YES. Some requirements like EVM (Modulation quality); YES. TAE; NA. Unwanted emissions; Occupied bandwidth; YES. ACLR; YES. IBE; NO. Operating band unwanted emissions; YES. Transmitter spurious emissions; YES. Transmitter intermodulation; TBD in WI phase. - Issue 2-5: RX Agreement: - Capture something like the following in the general section for BS: For A-IOT BS receiver requirements, some requirements can be different for inside and outside topology respectively - Whether requirements potentially needed or not is summarized in the following table: A-IoT BS- RX RF Requirement; Potentially needed or not. Reference sensitivity level; YES. Dynamic range; TBD in WI phase. Adjacent Channel Selectivity; YES. In-band blocking; general blocking; YES. narrowband blocking; YES. Out-of-band blocking; YES. Receiver spurious emission; YES. Receiver intermodulation; General intermodulation; Wait for RAN1 conclusion.. Narrowband intermodulation; YES. In-channel selectivity; TBD in WI phase.

Discussion

Transmitted output power

The requirements currently specified in 38.104 are as follows.

[38.104, Transmitted output power]

In the WID, BS type 1-C, and the meaning of the type can be divided into the following.

|  | BS Type 1-O | BS Type 2-O | BS Type 1-C | BS Type 1-H |
| --- | --- | --- | --- | --- |
| Frequency range | FR1(Sub-6GHz) | FR2(mmwave) | FR1(Sub-6GHz) | FR1( sub-6GHz) |
| Beamforming(AAS) | X | X | O | O |
| Output criteria | Conducted power | Conducted Power | EIRP | EIRP |

BS type 1-C should operate on FR1 and be equipped with beamforming, and the output standard will be set as EIRP. And the BS class is divided above, which can be understood as values proportional to coverage by the size of the cell. These values should be adjusted in the size of the power according to the coverage. Because too large power can cause interference with the proximity cell. In the case of the local area, it is an indoor environment, but it is a narrower area, so it can be driven with low power. As shown above, the decision is made based on optimization in a small cell environment, interference prevention and network quality maintenance, and existing LTE small cell standards and continuity.

In general, the required output power is also determined based on coverage. According to TR 38.848, the goal is to have 10-50m on the indicator basis. Also, since only device1 is currently considered, the reference sensitivity of device1 has a value between -30 and -40dBm. This allows you to calculate the output power required by the BS.

The conditions are as follows.

1. indoor condition(indoor, 10~50m distance)

2. Device reference sensitivity: -40dBm ~-30dBm

3. Example band: 900MHz(n8, FR1)

4. Antenna gain of BS: 5dBi

5. Antenna gain of UE: 0dBi

ITU indoor path loss formula is following:

PL=20logf+20logd+32.4

According to the above formula, when it's 10m. PL=20log0.9+20log10+32.4=51.48dB. when it’s 50m PL=20log0.9+20log50+32.4=65.46dB. The transmission power of the base station is calculated as follows.

Pr=Pout-PL+Gt+Gr
Pout=Pr+PL-Gt-Gr

What each parameter means here is that Pr is the reception sensitivity, PL is the path loss, Gtis the base station antenna gain, and Gr is the terminal antenna gain.

When the terminal reception sensitivity is -40 dBm when covering 10m, the transmission power is Pout=Pr+PL-Gt-Gr=-40+51.48-5-0=6.48dBm and When the terminal reception sensitivity is -30 dBm, the transmit power is Pout=-30+51.48-5-0=16.48dBm.

When it is said to cover 50m and has a receiver reference sensitivity of -40dBm and -30dBm, it is 20.46dBm and 30.46dBm, respectively, when calculated with the above equation.

| Distance | Reference sensitivity(dBm) | Needed BS output power(dBm) |
| --- | --- | --- |
| 10 | -40dBm | 6.48dBm |
| 10 | -30dBm | 16.48dBm |
| 50 | -40dBm | 20.46dBm |
| 50 | -30dBm | 30.46dBm |

As a result, the BS output power is 30.46 dBm when the worst case is 50 m and the reception sensitivity is -30 dBm. Therefore, the Ambient IoT BS 1-C type output power will be 30.46 dBm minimum. Therefore, the transmitted output power of Ambient IoT BS in the indoor environment can be defined as follows.

Prated_c_IoT ≤38dBm

or

Prated_c_IoT ≤31dBm

38 dBm is the output power value that BS 1-C type was used in the medium range in the existing NR. 38 dBm may be also considered in terms of standard continuity.

Proposal1: The transmitted output power of Ambient IoT BS in the indoor environment can be defined as follows.

Prated_c_IoT ≤38dBm

or

Prated_c_IoT ≤31dBm

Frequency error

The requirements currently specified in 38.104 are as follows.

[38.104, Frequency error]

The frequency error is relatively small for the wide area BS, and the medium range BS and the local area BS are smaller than the wide area BS. The reason is that the wide area BS is a macro cell base station that covers a wide area and connects with multiple UEs. If the frequency error is large, problems such as handover failure and increased interference between neighboring base stations occur, which requires higher frequency accuracy. These values are referred to in LTE and can be compatible with existing networks by maintaining similar error criteria in NR. In the case of Ambient IoT BS, since NR in-band operation is used, it is necessary to define the degree of frequency error. The frequency error of NR BS already seems strict enough. NR BS frequency error can be reused

Proposal 2: NR BS frequency error can be reused

EVM

Since the modulation method and signaling are changing, new requirements for EVM will likely be needed for A-IoT.

Proposal 3: New requirements for EVM is needed for A-IoT.

Occupied bandwidth

This value represents the minimum continuous bandwidth, which includes 99% of the total integrated power. We set the frequency range to Occupied bandwidth, which contains 99% of this power. Although this requirement uses the specific channel BW in real environment, the actual transmission signal may differ depending on filtering, PA nonlinearity, and modulation methods. Therefore, we can see that the occluded bandwidth is a comprehensive indicator that includes interference with neighboring bands and optimization of spectral efficiency. This value is also a requirement associated with the value of ACLR, which should be assigned so that the occupied BW is not too large or too small. It is required as a requirement, not something that should have any particular value.

Proposal 3: Occupied bandwidth of NR BS can be reused

- 2. Conclusion

In this contribution, we want to share some views on Ambient IoT BS for RF requirement and proposals are made as following:

Proposal1: The transmitted output power of Ambient IoT BS in the indoor environment can be defined as follows.

Prated_c_IoT ≤38dBm

or

Prated_c_IoT ≤31dBm

Proposal 2: NR BS frequency error can be reused

Proposal 3: New requirements for EVM is needed for A-IoT.

Proposal 4: Occupied bandwidth of NR BS can be reused

- 3. Reference

[1] R4-2420470, “WF on RF requirements for A-IoT”