3GPP TSG-RAN WG4 Meeting #114	                                                                              R4-2502273

Athens, Greece, 17th – 21st February, 2025

Source:	China Telecom

Title:		Ad-hoc meeting minutes for [114][323] NR_demod_Ph5_Part1_General_BS

Agenda Item:	7.18.4

Document for:	Approval

# BS MMSE-IRC requirements

Table. Summary of SNR performance gain of 8Rx for 15kHz SCS 5MHz for CP-OFDM

| MCS for target cell | Propagation condition for target cell | DIP set | Samsung | Nokia | CMCC | CTC | E/// | HW | DCM | AVE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | TDLC300-100 | HomNet set1: (-1.11. -10.91) | 6.35 | 4.6 | 3.17 | 3.92 |  |  |  | 4.51 |
|  | TDLB100-400 |  | 6.21 |  | 3.48 | 3.64 |  | 3.5 |  | 4.21 |
| 20 | TDLA30-10 |  | 5.92 | 4.4 | 3.4 | 3.78 | 3.63 | 3.2 | 3.69 | 4.0 |
|  | TDLB100-400 |  | 5.63 |  | 3.27 | 3.32 | 3.27 |  | 3.37 | 3.77 |
| 16 | TDLC300-100 | HetNet set2: (-0.43. -13.78) | 10.29 | 7.6 | 6.46 | 8.49 |  |  |  | 8.21 |
|  | TDLA30-10 |  | 9.72 |  | 6.57 | 7.91 |  |  |  | 8.07 |
| 20 |  |  | 9.77 | 7.4 | 6.35 | 7.30 | 6.49 | 6.1 | 8.01 | 7.35 |

Issue 1-2-2: MCS

- Proposals on tested MCS:

  - For 2Rx:

- Option 1: MCS 2 (Tejas, CATT, Ericsson, ZTE)

- Option 2: MCS 16 (China Telecom, CMCC, Huawei, Nokia, Samsung)

  - For 4Rx:

- Option 1: MCS 16 (China Telecom, Tejas, CMCC, CATT, Ericsson, ZTE, Huawei, Nokia, Samsung)

  - For 8Rx:

- Option 1: MCS 20 (Tejas, CATT, Ericsson, ZTE, Samsung)

- Option 2: MCS 16 (China Telecom, CMCC, Huawei, Nokia)

- On whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration:

  - Proposal 1: The existing test applicability rule for different Rx number can be reused (Ericsson, ZTE, Samsung)

Discussion:

Moderator: if any company has strong preference to the above options? If no, can we go with option 1?

Ericsson: Better to test higher MCS for 8Rx (MCS 20)

Samsung: ok with option 1. Avoid introducing applicability rule to save efforts.

Huawei: Support option1.

ZTE: Support option 2. We want to test different MCS for different Rx.

Ericsson: Why we need additional applicability rule?

Nokia: Support option 1. MCS 16 has bit higher SNR gain.

Samsung: R15 only define same MCS for all Rx.

Moderator: Can we go with opt1?

Tentative agreement

MCS 16 for 2Rx (if introduced), 4Rx and 8Rx

Issue 1-2-3: Propagation condition

- Proposals for the serving cell:

  - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet (China Telecom, CMCC, Ericsson, ZTE, CATT, Nokia as compromise)

- Option 1a: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Samsung, Huawei)

  - Option 2: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 (Tejas, CMCC, CATT, Huawei, Nokia, Samsung)

  - Option 3: TDLC300-100 for all cases (Samsung, Nokia, Huawei)

Discussion:

Moderator: Can we go with option 1 given that we have already agree sane MCS for all Rx.

Samsung: Prefer TDLC for all scenarios.

Nokia, Huawei: Agree Samsung.

Ericsson: TDLC channel model is not aligned with the practical scenario for HetNEt.

CTC: Agree with E///.

ZTE: Agree E///.

CMCC: Simulation results shows option 1 has higher gain. Agree with E///.

Ericsson: We have already use TDLC for interference.

Nokia: Option 3 is aligned with R15.

Ericsson: Why we need to align with R15?

Nokia: In R15, we decide propagation condition based on MCS.

Moderator, CMCC: if the concern id about dolpper, can we use TDLA30-10 HetNet and TDLC300-100 for HomNet?

Ericsson: If we use same channel for target and interference, why we consider different scenario.

CATT: Can accept option 1.

Moderator:

TDLA30-10 or TDLC300-100 for HetNet

Ericsson: We need to check further on TDLC300-100 for HomNet with respect to our simulation results. So, could you please consider it as FFS for now?

Recommende WF:

TDLA30-10 for HetNet and TDLC300-100 for HomNet

Channel bandwidth and waveform

- Proposals on the CHBW for requirement definition for CP-OFDM:

  - Option 1 (China Telecom, CMCC)

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

  - Option 2 Considering typical CBW (Samsung)

- For 15kHz SCS, cover 5MHz, 10MHz

- For 30kHz SCS, cover 10MHz, 40MHz

  - Option 3: minimum CBW (Tejas, CATT, Ericsson, ZTE, Huawei, Nokia)

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

  - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS (Qualcomm, Nokia)

- Proposals for DFT-s-OFDM:

  - Option 1 (China Telecom, Tejas, CATT, Samsung if defined, [CMCC, Qualcomm])

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

  - Option 2: Not consider DFT-s-OFDM (Tejas, Ericsson, ZTE, Huawei, Nokia, Samsung)

Discussion:

Nokia: why we have 3 CHBW for 30kHz? Prefer only 2 for each SCS.

Huawei: Agree Nokia.

CATT: It should be enough to test only 1 CHBW for each SCS to make things simple.

CMCC: To Nokia and HW: on Monday we did not combine waveform and CHBW. The recommended WF has been our compromise. Min + Max + typical CHBW. To CATT, the spirit of MCS discussion is not only to make things simple.

CTC: We agree with CMCC, MCS discussion and CHBW are different.

CATT: We can compromise to one additional CHBW which is Max.

Ericsson: We can also compromise to Min + Max

Samsung: Ok with Min + Max.

ZTE: As compromise Ok with Min + Max.

CMCC: We concern on removing 40MHz for 30kHz SCS.

Samsung: We have proposed 40MHz for 30kHz SCS.

Tentative agreement:

  - For CP-ODFM:

- For 15kHz SCS, cover 5MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 100MHz

  - Not to cover DFT-s-OFDM

Issue 1-1-1: Number of interferers for 2Rx

- Moderator’s summary of companies’ simulation results (Take 15kHz SCS 5MHz CHBW as example):

Table. Range of SNR performance gain for 2Rx with CP-OFDM

|  | CTC | Tejas | CMCC | E/// | HW | Nokia | DCM | Samsung |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HomNet with 1 interferer | 2.50 ~ 3.08 | 1.03~1.49 | 1.42 - 1.8 | 1.8 |  | 1.2~1.8 |  | 2.48~3.19 |
| HomNet with 2 interferers | 2.54 ~ 2.72 | 1.73~3.16 | 1.68 - 2.23 | 1.88 | 1.9~2.3 | 1.2~1.9 | 1.45~3.18 | 2.58~3.38 |
| HetNet with 1 interferer | 5.04 ~ 5.47 | 1.88~3.19 | 3.22 - 3.62 | 3.47~3.74 |  | 2.2~3.7 |  | 4.89~5.62 |
| HetNet with 2 interferers | 5.05 ~ 5.25 | 2.64~4.57 | 3.38 - 4.41 | 3.57~3.97 | 3.7~3.9 | 2.3~3.8 | 1.81~6.07 | 4.96~5.79 |

- Proposals:

  - Option 1: Define BS MMSE-IRC requirements for 2Rx (China Telecom, Tejas, CMCC, Ericsson, ZTE, Huawei, Samsung)

- Option 1A: 1 interferer (Ericsson, CTC slightly prefer, CMCC for DFT-s-OFDM)

- Option 1B: 2 interferers (Tejas, ZTE, Samsung, Huawei, CTC, CMCC as compromise)

- Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet (CMCC for CP-OFDM)

  - Option 2: Do not need to specify requirements for 2Rx (Nokia)

Discussion:

Nokia: Can accept introducing 2Rx for HetNet but prefer 1 interferer.

Ericsson: We prefer 1 interferer for both HomNet and HetNet deployment since we do not observe much performance difference.

- Recommended WF

  - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx.

  - For the number of interferers for 2Rx under HetNet, further decide between the following:

- Option 1: 2 interferers

- Option 2: 1 interferer

  - For HomNet scenario, further decide among the following:

- Option 1: 2 interferers

- Option 2: 1 interferer

- Option 3: Not to consider 2Rx requirements for HomNet scenario.