3GPP TSG-RAN WG4 Meeting #114                                                               R4-2502307

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.23.4

Source:	Samsung

Title:	Ad-hoc meeting minutes for [114][306] NR_duplex_evo_General

Document for:	Approval

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

## Open issues summary

### Sub-topic 1-1: SBFD applicability for NR TDD band and CBW

##### Issue 1-1-1: FR2-1 CBW and SCS support for SBFD

- Channel BW support for FR2:

  - Option 1 (Samsung): In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

  - Option 2 (Nokia): FR2 minimum CBW to support SBFD is 100MHz for DU/UD case and 200MHz for DUD case.

    - The minimum bandwidth requirement for FR2 DUD case can be revisited depending on future RAN1 agreements on SSB transmission on SBFD symbols.

  - Option 3 (vivo, ZTE): such restriction is not necessary.

- SCS support for FR2-1:

  - Proposal 1 (CMCC): Do not set restriction on use of 60kHz SCS.

- Moderator Recommendation:

  - Discuss on the above proposals firstly.

- Ad-hoc discussion:

  - Nokia: prefer 200MHz for DUD case, and prefer more checking on “In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz”.

- Agreement:

  - In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

    - RAN4 to further study on 100MHz for DUD case to identify if any issue.

##### Issue 1-1-2: Specification implementation of operating bands support for SBFD

- [Moderator] Based on the discussion in RAN4#113 as below. Accordingly, companies have provided proposals on this issue.

Agreement: - The following TP is adopted for clause 5.2 in TS 38.104 - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2. - FFS a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

- For band Table 5.2-1, whether a Note is needed:

  - Option 1 (Samsung, vivo, Qualcomm): A note is needed band table 5.2-1.

  - Option 2 (ZTE): not needed.

- If Option-1 is followed, text proposal to section 5.2 for SBFD operation:

  - Text proposal from Samsung (Similar TP proposed by Qualcomm, here we use Samsung’s TP as example):

=================== Start of Text Proposal ===================

5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n1 | 1920 – 1980 | 2110 – 2170 | FDD |  |
| n2 | 1850 – 1910 | 1930 – 1990 | FDD |  |
| n3 | 1710 – 1785 | 1805 – 1880 | FDD |  |
| n5 | 824 – 849 | 869 – 894 | FDD |  |
| n7 | 2500 – 2570 | 2620 – 2690 | FDD |  |
| n8 | 880 – 915 | 925 – 960 | FDD |  |
| n12 | 699 – 716 | 729 – 746 | FDD |  |
| n13 | 777 – 787 | 746 – 756 | FDD |  |
| n14 | 788 – 798 | 758 – 768 | FDD |  |
| n18 | 815 – 830 | 860 – 875 | FDD |  |
| n20 | 832 – 862 | 791 – 821 | FDD |  |
| n24 | 1626.5 – 1660.5 | 1525 – 1559 | FDD | Note 7 |
| n25 | 1850 – 1915 | 1930 – 1995 | FDD |  |
| n26 | 814 – 849 | 859 – 894 | FDD |  |
| n28 | 703 – 748 | 758 – 803 | FDD |  |
| n29 | N/A | 717 – 728 | SDL |  |
| n30 | 2305 – 2315 | 2350 – 2360 | FDD |  |
| n31 | 452.5 – 457.5 | 462.5 – 467.5 | FDD |  |
| n34 | 2010 – 2025 | 2010 – 2025 | TDD | Note 12 |
| n38 | 2570 – 2620 | 2570 – 2620 | TDD |  |
| n39 | 1880 – 1920 | 1880 – 1920 | TDD |  |
| n40 | 2300 – 2400 | 2300 – 2400 | TDD |  |
| n41 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n46 | 5150 – 5925 | 5150 – 5925 | TDD | Note 3, Note 12 |
| n48 | 3550 – 3700 | 3550 – 3700 | TDD |  |
| n50 | 1432 – 1517 | 1432 – 1517 | TDD |  |
| n51 | 1427 – 1432 | 1427 – 1432 | TDD | Note 12 |
| n53 | 2483.5 – 2495 | 2483.5 – 2495 | TDD | Note 12 |
| n54 | 1670 – 1675 | 1670 – 1675 | TDD | Note 12 |
| n65 | 1920 – 2010 | 2110 – 2200 | FDD |  |
| n66 | 1710 – 1780 | 2110 – 2200 | FDD |  |
| n67 | N/A | 738 – 758 | SDL |  |
| n70 | 1695 – 1710 | 1995 – 2020 | FDD |  |
| n71 | 663 – 698 | 617 – 652 | FDD |  |
| n72 | 451 – 456 | 461 – 466 | FDD |  |
| n74 | 1427 – 1470 | 1475 – 1518 | FDD |  |
| n75 | N/A | 1432 – 1517 | SDL |  |
| n76 | N/A | 1427 – 1432 | SDL |  |
| n77 | 3300 – 4200 | 3300 – 4200 | TDD | Note 11 |
| n78 | 3300 – 3800 | 3300 – 3800 | TDD |  |
| n79 | 4400 – 5000 | 4400 – 5000 | TDD |  |
| n80 | 1710 – 1785 | N/A | SUL |  |
| n81 | 880 – 915 | N/A | SUL |  |
| n82 | 832 – 862 | N/A | SUL |  |
| n83 | 703 – 748 | N/A | SUL |  |
| n84 | 1920 – 1980 | N/A | SUL |  |
| n85 | 698 – 716 | 728 – 746 | FDD |  |
| n86 | 1710 – 1780 | N/A | SUL |  |
| n89 | 824 – 849 | N/A | SUL |  |
| n90 | 2496 – 2690 | 2496 – 2690 | TDD |  |
| n91 | 832 – 862 | 1427 – 1432 | FDD | Note 2 |
| n92 | 832 – 862 | 1432 – 1517 | FDD | Note 2 |
| n93 | 880 – 915 | 1427 – 1432 | FDD | Note 2 |
| n94 | 880 – 915 | 1432 – 1517 | FDD | Note 2 |
| n95 | 2010 – 2025 | N/A | SUL | Note 1 |
| n96 | 5925 – 7125 | 5925 – 7125 | TDD | Note 3, Note 4, Note 12 |
| n97 | 2300 – 2400 | N/A | SUL | Note 5 |
| n98 | 1880 – 1920 | N/A | SUL | Note 5 |
| n99 | 1626.5 – 1660.5 | N/A | SUL | Note 6 |
| n100 | 874.4 – 880 | 919.4 – 925 | FDD | Note 10 |
| n101 | 1900 – 1910 | 1900 – 1910 | TDD | Note 10, Note 12 |
| n102 | 5925 – 6425 | 5925 – 6425 | TDD | Note 3, Note 4, Note 12 |
| n104 | 6425 – 7125 | 6425 – 7125 | TDD | Note 8 |
| n105 | 663 – 703 | 612 – 652 | FDD |  |
| n106 | 896 – 901 | 935 – 940 | FDD |  |
| n109 | 703 – 733 | 1432 – 1517 | FDD | Note 2 |
| NOTE 1: This band is applicable in China only. NOTE 2: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 3: This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [20]. NOTE 4: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. NOTE 5: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 6: UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48 [25]. NOTE 7: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order 20-51 [24]. NOTE 8: This band is applicable only in countries/regions designating this band for IMT licensed operation subject to country-specific conditions. NOTE 9: Operating band n200 is a reserved value. NOTE 10: This band is applicable to CEPT countries subject to ECC Decision (20)02 [21], for the RMR application. NOTE 11: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. NOTE 12: This TDD band is not applicable to SBFD operation. |  |  |  |  |

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high (MHz) | Duplex mode |
| --- | --- | --- |
| n257 | 26500 – 29500 | TDD |
| n258 | 24250 – 27500 | TDD |
| n259 | 39500 – 43500 | TDD |
| n260 | 37000 – 40000 | TDD |
| n261 | 27500 – 28350 | TDD |
| n262 | 47200 – 48200 | TDD |
| n263 | 57000 – 71000 | TDD |

=================== End of Text Proposal ===================

- Moderator Recommendation:

  - Agree on Proposal 1 and endorse text proposal.

##### Issue 1-1-3: Specification implementation of BS CBW support for SBFD

- In RAN4#113, for the specification implementation of BS channel bandwidth support, the following agreements have been reached:

Agreement: - The following note is introduced in Table 5.3.2-1 and Table 5.3.5-1 in TS 38.104: - In the bands applicable to SBFD operation, the channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise.

- Text proposal to Table 5.3.2-1 and Table 5.3.2-2 for SBFD operation:

  - Text Proposal from Samsung:

=================== Start of Text Proposal ===================

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| Note: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |
| Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz. |  |  |  |  |

=================== End of Text Proposal ===================

- Text proposal to Table 5.3.5-1 and Table 5.3.5-2 for SBFD operation:

  - Proposal 2 (Samsung): Same as the Notes in previous proposal, RAN4 shall adopt the same Notes for Table 5.3.5-1 and Table 5.3.5-2 respectively. Take the table for FR2-1 as example:

=================== Start of Text Proposal ===================

Table 5.3.5-2: BS channel bandwidths and SCS per operating band in FR2-1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n257 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n258 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n259 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n260 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n261 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| n262 | 60 | 501 | 100 | 200 |  |
|  | 120 | 501 | 100 | 200 | 400 |
| Note 1: For this bandwidth, it is not applicable for SBFD operation. |  |  |  |  |  |

=================== End of Text Proposal ===================

- Moderator Recommendation:

  - Discussion on Proposal 1 firstly.

##### Issue 1-1-4: Declaration and flexibility for SBFD operation

- In RAN4#113, the following are agreed:

Agreement: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications

- Proposals on the BS classes in the above declaration rule:

  - Proposal 1 (Nokia): The restrictions agreed for FR1 WA BS are also applicable for FR1 MR BS.

  - Proposal 2 (ZTE): Regarding the declaration and flexibility for SBFD operation, we propose to define the conformance testing as following way:

    - FR1 WA BS capable of SBFD operation, BS need to declare its supported SBFD configuration and test all of these SBFD configuration;

    - For other BS class capable of SBFD operation, BS need to declare its supported SBFD configuration and only test the worst case which follows the legacy principle (e.g. typical transmission bandwidth (5MHz or 20MHz) or minimum supported DL sub-band configuration)

  - Proposal/Observation from Qualcomm:

    - Observation 1: Restrictions on in-field operation of certain configurations is not within the scope of RAN4 specs, but could be captured via regional regulations (e.g., Harmonized Standards in Europe).

    - Observation 2: It is challenging for RAN4 to agree on a reference RF solution for interference handling in SBFD-capable BS to be used to test SBFD configurations for medium range base stations.

    - Observation 3: In TR 38.858, no challenges were reported from implementation point of view on the feasibility of SBFD-capable local area BS.

    - Proposal 3: If it is feasible to capture RAN4 specs, RAN4 to consider the below only for wide area base stations

      - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared and tested.

    - Proposal 4: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation.

    - Proposal 5: RAN4 to consider fixed number of downlink subband configuration supported for FR1 WA BS. FFS what exact number to be agreed.

  - Proposals from Ericsson:

    - Proposal 6: For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

    - Proposal 7: For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

    - Proposal 8: Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

- Moderator Recommendation:

  - Discussion on the above proposal and observation firstly.

- Ad-hoc discussion:

  - Nokia: see Ericsson’s proposal on power threshold as a compromise for MR.

  - Ericsson: 33dBm is proposed for the threshold, and we check the necessity of analog filter and based on that to propose this.

  - QC: 33dBm is for LA?

  - ZTE: 33dBm for OTA TRP, it should be LA BS.

- Agreement:

  - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested.

    - This restriction shall not be applied to FR1 LA BS

    - FFS FR1 MR BS

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

- [Moderator] Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#113, the following agreement is provided:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

In which option 2b is given in RAN4#112:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

##### Issue 1-2-1: Restriction on the number of UL subband sizes supported

- [Moderator] For the restriction-2 (i.e., restriction on the number of UL subband sizes supported), the following questions to be further discussed:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 specific UL subband sizes allowed: CMCC, Huawei, Samsung, ZTE

  - Maximum 2 specific UL subband sizes allowed: Nokia, Ericsson

- FFS different sizes for specific UL subband for DUD and DU/UD.

  - The same size for DUD and DU/UD: Samsung

  - DUD and DU/UD can have different UL subband size: CATT, Huawei

- FFS the restriction is also applied to FR1 MR BS

  - No (i.e., only apply to FR1 WA BS): Huawei, Samsung, CMCC, vivo, CATT, Qualcomm

  - Yes (also appliable to FR1 MR BS): Nokia, Ericsson.

    - Ericsson: depends on the TRP output power is greater than 33 dBm or not.

- Moderator Recommendation:

  - Still need more discussion on above options.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

- [Moderator] For the restriction-3 (i.e., Restriction on the number of DL subband sizes supported), the following questions to be further discussed:

- FFS the restriction is also applied to FR1 MR BS

  - No (i.e., only apply to FR1 WA BS): Huawei, Samsung, CMCC, Tejas Network, vivo, CATT, Qualcomm

  - Yes (also appliable to FR1 MR BS): Nokia

- Moderator Recommendation:

  - Conclude “the restriction is NOT applied to FR1 MR BS” based on majority view.

##### Issue 1-2-3: Restriction on guardband size for SBFD

- [Moderator] Based on RAN4#112bis agreement, the following options for restriction on guardband size are identified, while no further agreement achieved in RAN4#113:

Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

- Companies’ preference:

  - Option 3a: Samsung

- Moderator Recommendation:

  - Consider Option 2b is agreed for DL/UL subband configurations support for SBFD, it could be reasonable no further restriction is needed for guardband restriction by following the logic of giving the gNB implementation flexibility to determine DL subband. So Option 3a can be concluded.

##### Issue 1-2-4: Other restriction on DL subband size

- [Moderator] Based on last meeting’s discussion, the following WF is approved for further discussion on the following restrictions:

Way Forward: - FFS the following restriction on DL subband size: - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

- The necessity of the additional restriction on DL subband size, i.e., for SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size):

  - Option 1 (vivo, ZTE): Not needed.

  - Option 2 (Nokia): needed.

- Moderator Recommendation:

  - Discussion on the above proposals firstly.

### Sub-topic 1-3: Requirement Applicability for SBFD

##### Issue 1-3-1: Requirement set applicability table for SBFD-capable BS

- Proposal for the requirement set applicability table for SBFD-capable BS :

  - Proposal 1: The applicability of RAN4 requirement for SBFD capable BS shall be introduced, with the following table used as baseline for further discussion:

- Table x.x: Requirement set applicability for SBFD-capable BS

| Requirement | Requirement Set for non-SBFD symbols |  |  | Requirement Set for SBFD symbols |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BS type 1-H | BS type 1-O | BS type 2-O | BS type 1-H | BS type 1-O | BS type 2-O |
| BS output power | 6.2 |  |  | TBD |  |  |
| Output power dynamics | 6.3 |  |  | TBD |  |  |
| Transmit ON/OFF power | 6.4 |  |  | TBD |  |  |
| Transmitted signal quality | 6.5 |  |  | TBD |  |  |
| Occupied bandwidth | 6.6.2 |  |  | TBD |  |  |
| ACLR | 6.6.3 |  |  | TBD |  |  |
| Operating band unwanted emissions | 6.6.4 |  |  | TBD |  |  |
| Transmitter spurious emissions | 6.6.5 |  |  | TBD |  |  |
| Transmitter intermodulation | 6.7 | NA | NA | TBD | NA | NA |
| Reference sensitivity level | 7.2 |  |  | TBD |  |  |
| Dynamic range | 7.3 |  |  | TBD |  |  |
| In-band selectivity and blocking | 7.4 |  |  | TBD |  |  |
| Out-of-band blocking | 7.5 |  |  | TBD |  |  |
| Receiver spurious emissions | 7.6 |  |  | TBD |  |  |
| Receiver intermodulation | 7.7 |  |  | TBD |  |  |
| In-channel selectivity | 7.8 |  |  | TBD |  |  |
| Performance requirements | 8 |  |  | TBD |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 | TBD | TBD | TBD |
| OTA base station output power |  | 9.3 | 9.3 |  | TBD | TBD |
| OTA output power dynamics |  | 9.4 | 9.4 |  | TBD | TBD |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |  | TBD | TBD |
| OTA transmitted signal quality |  | 9.6 | 9.6 |  | TBD | TBD |
| OTA occupied bandwidth |  | 9.7.2 | 9.7.2 |  | TBD | TBD |
| OTA ACLR | NA | 9.7.3 | 9.7.3 | NA | TBD | TBD |
| OTA out-of-band emission |  | 9.7.4 | 9.7.4 |  | TBD | TBD |
| OTA transmitter spurious emission |  | 9.7.5 | 9.7.5 |  | TBD | TBD |
| OTA transmitter intermodulation |  | 9.8 | NA |  | TBD | NA |
| OTA sensitivity | 10.2 | 10.2 | NA | TBD | TBD | NA |
| OTA reference sensitivity level |  | 10.3 | 10.3 |  | TBD | TBD |
| OTA dynamic range |  | 10.4 | NA |  | TBD | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |  | TBD | TBD |
| OTA out-of-band blocking | NA | 10.6 | 10.6 | NA | TBD | TBD |
| OTA receiver spurious emission |  | 10.7 | 10.7 |  | TBD | TBD |
| OTA receiver intermodulation |  | 10.8 | 10.8 |  | TBD | TBD |
| OTA in-channel selectivity |  | 10.9 | 10.9 |  | TBD | TBD |
| Radiated performance requirements |  | 11 | 11 |  | TBD | TBD |

- Moderator Recommendation:

  - Need to discuss on the proposal firstly.

- Ad-hoc meeting:

  - Nokia: need to check.

  - Ericsson: “Requirement Set for non-SBFD symbols or SBFD symbols” is beyond the initial intention of the applicability table.

  - Samsung: we may need that based on existing agreement.

  - ZTE: same as Samsung.

  - Ericsson: for non-SBFD symbols, we can just have a general description to refer to existing applicability table.

  - CATT: three parts can be separated: no impacted requirement, impacted requirement and new requirements defined not in the existing applicability table.

### Sub-topic 1-4: Fair coexistence between SBFD and TDD networks

##### Issue 1-4-1: Limiting Maximum UE Transmit power in SBFD symbols/slots.

- [Moderator] In this meeting, it is proposed to consider some mechanism on limiting maximum UE transmit power in SBFD symbols/slots to limit UE-to-UE CLI in the adjacent legacy TDD networks, thus making the transmit power can be capped by different maximum values for SBFD symbols/slots and non-SBFD symbols/slots.

Transmit power=minPCMAX, Ptx,  &non-SBFD symbols/slotsminPCMAX,SBFD, Ptx,  &SBFD symbols/slots

- Proposals from Charter:

  - Proposal 1: Reduce the maximum UE transmit power in the SBFD symbols/slots to limit UE-to-UE CLI in the adjacent legacy TDD networks. gNB will convey the XdB value in a message to the UE which will be used to back off the maximum UE transmit power during SBFD symbols/slots.

  - Proposal 2: This message could be defined in 38.101-1, 38.101-2 and 38.101-3.

- Moderator Recommendation:

  - As the new topic proposed, the group need to discuss on the proposals firstly.

- Ad-hoc meeting:

  - Charter: try to mitigate UL CLI from SBFD-UE to TDD DL. Find a way to apply the power cap on some of the UE in the cell edge which has more impact on neighboring TDD.

  - Chair: is that a RAN4 issue to define Pcmax?

  - QC: when/how the gNB decide the delta?

  - CATT: XdB is a single common one for all UE or a configured one (per UE configured)?

    - Charter: should be per-UE configured.

  - KTL: why this parameter, but not the others for PC?

    - Charter: it is the cap for the total power.

  - Fraunhofer: a different cap could be needed, but not necessarily a smaller value.

  - ZTE: For UE-UE CLI, we have not yet identified big issue. Additional power cap can further decrease the benefits of UL coverage by SBFD.

  - Huawei: same as ZTE. Existing power control can work, not quite sure we need the RRC level one.

    - CableLabs: In the study item, the CLI problem has been identified.

    - ZTE: if such problem identified, why we need such scenario for SBFD deployment.

  - CATT: If two TDD networks in adjacent channel is not synchronized, CLI should be there. Even for synchronized networks, how about the symbol level synchronization for flexible symbol, and they should exist already for this issue.

    - CableLabs: for TDD-TDD, it should be synchronized.

- Agreement:

  - N/A

##### Issue 1-4-2: Using threshold(s) to limit the scheduling of cell-edge UEs in SBFD symbols/slots

- [Moderator] In this meeting, it is proposed to use a threshold or two thresholds to limit the scheduling of cell-edge UEs in SBFD symbols/slots

- Proposals from Charter for using one threshold:

  - Proposal 1: Set a UL Power threshold, where only UEs with an estimated Tx power level below the threshold will have full access to the SBFD resources. All other UEs will not be scheduled in the SBFD symbols/slots (but still have full access to TDD symbols).

  - Proposal 2: Alternatively, set a PHR threshold, where only UEs with a PHR level above the threshold will have full access to SBFD resources. All other UEs will not be scheduled in the SBFD symbols/slots (but still have full access to TDD symbols).

- Proposal from Charter for using 2 thresholds:

  - Proposal 3: Alternatively, define 2 PHR Thresholds for managing SBFD UL resources for the served UEs. The thresholds can be found based on historical measurements/reports fed back by the served UEs. These 2 Thresholds will define 3 groups of UE based on their reported PHR levels. A connected UE will be scheduled to the UL SBFD resources based on its specific PHR group level.

- Moderator Recommendation:

  - As the new topic proposed, the group need to discuss on the proposals firstly.

### Sub-topic 1-5: Other general issues for SBFD requirements

##### Issue 1-5-1: BS RF Specification structure for SBFD requirements

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved. Accordingly, BS RF specification structure for SBFD requirements shall be further discussed in this meeting.

<From RAN4#112> - Way forward: - RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS: - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

- [Moderator] Furthermore, in RAN4#113, the following agreement is achieved:

<From RAN4#113> Agreement: - Draft CR’s endorsed against 38.104 does not imply that a new specification could not be agreed.

- Options of how to introduce BS RF new requirements for SBFD-capable BS:

  - Option 2 (Samsung, vivo, CATT, [QC, Huawei, ZTE]): Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

    - Option 2b (Samsung): Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements

  - Option 3 (Ericsson/Nokia): A new specification for SBFD BS RF requirements

- Moderator Recommendation:

  - Based on majority view, agree on Option 2 firstly.

##### Issue 1-5-2: CLI handling impact on RF requirement

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved.

<From RAN4#112> - Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Based on previous meeting’s discussion, there is no agreement to either revisit or confirming no revisit the “FFS” bullet.

- Further proposals are provided:

  - Proposal 1 (Nokia): RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Moderator Recommendation:

  - Suggest to deprioritize the discussion on this issue since it is not easily to be concluded, and this will also not directly make any difference for detailed RF requirements discussion till now.

# Topic #2: PUSCH with resource muting for CLI handling

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Open issues summary

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

- Following proposal/observations provided:

  - Observation 1 (Ericsson): MPR remains unaffected by the introduction of UL resource muting.

  - Observation 2 (Huawei): For UL resource muting, there is no transmit quality or MPR requirement impact can be expected for PUSCH as comb-2 pattern is already supported since LTE.

  - Observation 3 (Qualcomm): Resource muting changes the waveform characteristics and could have an impact on MPR.

  - Observation 4 (Qualcomm): With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.)  might need to be revisited.

  - Observation 5 (Qualcomm): Resource muting has minimal impact on the transmit waveform quality.

  - Proposal 1 (Samsung): The following scenarios for transmit signal quality/MPR impact evaluation can be considered:

    - DFT-s-OFDM waveform:

      - FR1: MPR evaluation at least for power class 3 UE

      - FR2: discussion on the necessity of MPR evaluation

    - CP-OFDM waveform:

      - FR1: MPR evaluation at least for power class 3 UE, with (1) contiguous allocation and (2) almost contiguous allocation

      - FR2: discussion on the necessity of MPR evaluation

- Moderator Recommendation:

  - Collect companies’ view on the above observations firstly.

# Topic #3: Potentially new BS RF requirements for SBFD

Main technical topic overview. The structure can be done based on sub-agenda basis.

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 3-1-1: Location of transient period between SBFD and non-SBFD

- [Moderator] In RAN4#113, the following WF is achieved for “Location of transient period between SBFD and non-SBFD”:

#### Issue 3-1-2: Location of transient period between SBFD and non-SBFD Way Forward: - FFS whether the transmitter transient period shall be located within the SBFD symbol or not: - FFS it is an implementation issue and no RAN4 standard impact

- Observations and proposals from Samsung:

  - Observation 1: For the identified four cases of transitions to/from SBFD symbols, the following timing behaviors are expected:

|  | Expected timing behavior for DL/UL subbands respectively |  | Location of transient period (if any) |
| --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. | N/A |
|  | UL subband | TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period. | Located in TA_offset in SBFD symbol(s) |
| Case-B (SBFD symbol to UL symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbol(s) reserved in SBFD symbol(s) |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all | N/A |
| Case-C (DL symbol to SBFD symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. | N/A |
|  | UL subband | UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbol(s) reserved in non-SBFD DL symbol(s) |
| Case-D (UL symbol to SBFD symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset | Located in TA_offset in non-SBFD UL symbol(s) |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all. | N/A |

  - Proposal 1: In practical SBFD deployment, the location of transient periods should be located:

(1) Case-A: in TA_offset in SBFD symbol(s);

(2) Case-B: in guard symbol(s) reserved in SBFD symbol(s);

(3) Case-C: in guard symbol(s) reserved in non-SBFD DL symbol(s);

(4) Case-D: in TA_offset in non-SBFD UL symbol(s).

  - Proposal 2: For the cases in which the transient period is located in guard symbols reserved in non-SBFD DL symbol(s), i.e., Case-C, the explicit guard should be reserved to allow UE transition from receiving to transmitting, in which the transient period, two-way propagation delay shall be accommodated for UL subband.

  - Proposal 3: In RAN4 requirement, no need to specify the detailed location of the transmitter transient period (i.e., within the SBFD symbol or not), because in practice the length and location of TA_offset and guard symbols are provided by gNB configuration.

- Observations and proposals from Tejas Network:

  - Observation 1: The location of the transmitter transient period is primarily an implementation-specific issue, influenced by factors like hardware capabilities, system timing mechanisms, and self-interference management techniques. Variability across vendors and deployment scenarios makes a standardized approach impractical.

  - Observation 2: The transient period’s location does not directly impact system-level performance if key performance indicators (ACLR, REFSENS, in-band blocking, etc.) are maintained. Current mechanisms such as guard symbols and TA_offset offer sufficient flexibility to accommodate transient periods in different deployment scenarios without requiring standardization.

  - Proposal 1: Recognize the placement of the transmitter transient period as an implementation-specific decision, with no direct impact on RAN4 standards.

  - Proposal 2: Prioritize defining and enforcing system-level performance requirements (e.g., ACLR, REFSENS, in-band blocking) to ensure proper operation during transient periods, independent of their location.

- Options for WF on the location of transient period between SBFD and non-SBFD:

  - Option 1 (Tejas Nework/Samsung): implementation issue and no direct RAN4 standard impact.

  - Option 2 (CMCC/CATT/Huawei): Transmitter transient period shall be located within the SBFD symbol.

- Moderator Recommendation:

  - RAN4 can firstly discuss options. Since this issue has been discussed for many meeting cycles, if no conclusion reachable, the group shall consider to send LS to RAN1 to ask for clarification (see next issue).

##### Issue 3-1-2: LS to RAN1 to clarify the location of transient period between SBFD and non-SBFD

- [Moderator] If the above Issue 3-1-2 can’t be resolved, it is suggested to send LS to RAN1 to clarify the location of transient period between SBFD and non-SBFD.

- Drafted LS to RAN1 (Samsung):

On RAN4 requirements for SBFD transient period at gNB, RAN4 has made the following agreements Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations. Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases.. Particularly for the location of transient period, RAN4 has identified four cases of transitions to/from SBFD symbols as illustrated in Figure 1: Figure 1: Illustration of four cases of transitions to/from SBFD symbols Accordingly, the expected timing behaviours for four cases are provided in the below table, and illustrated in Figure 2: Expected timing behavior for DL/UL subbands respectively; Location of transient period (if any). Case-A (SBFD symbol to DL symbol); DL subband(s); No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol.; N/A. UL subband; TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period.; Located in TA_offset in SBFD symbol(s). Case-B (SBFD symbol to UL symbol); DL subband(s); UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset.; Located in guard symbol(s) reserved in SBFD symbol(s). UL subband; TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all; N/A. Case-C (DL symbol to SBFD symbol); DL subband(s); No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol.; N/A. UL subband; UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset.; Located in guard symbol(s) reserved in non-SBFD DL symbol(s). Case-D (UL symbol to SBFD symbol); DL subband(s); UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset; Located in TA_offset in non-SBFD UL symbol(s). UL subband; TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all.; N/A. Figure 2: Detailed illustration for transient periods for SBFD UE and legacy TDD UE for four cases Note that RAN4 is still discussing the RAN4 requirement on the transient period, especially how to capture the above understanding on the location of transient period in RAN4 requirement for SBFD capable gNB. ## 2 Actions To RAN1 ACTION: RAN4 respectfully asks RAN1 to confirm the above RAN4 understanding, and provide further information on the location of transient period if any.

- Moderator Recommendation:

  - If RAN4 can’t get agreement on Issue 3-1-2, the group consider the way of sending LS to RAN1 to ask for clarification.

##### Issue 3-1-3: Transient period requirement applicability on SBFD subbands

- [Moderator] In RAN4#112 and #112bis, the following agreement and WF are achieved for the requirement for transient period between SBFD and non-SBFD:

<Agreement in RAN4#112> Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. <Agreement in RAN4#112bis> Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498. <Agreement in RAN4#113> Issue 3-1-1: Transient period requirement applicability on frequency/antenna Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases.

- Observation/Proposal from Samsung:

  - Observation 1: The transient period for SBFD operation is different from TDD operation in terms of:

    - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and

    - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements.

- Proposal on the specification implementation:

  - Proposal 1 (Samsung/Nokia): The text proposal is updated for the clause of transmitter transient period for SBFD:

=================== Start of Text Proposal ===================

6.4.2B	Transmitter transient period for SBFD

6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

- Moderator Recommendation:

  - Discuss whether the above text proposal can be endorsed.

### Sub-topic 3-2: In-channel adjacent subband requirements

##### Issue 3-2-1: In-channel adjacent subband leakage ratio (ICASLR): General

- [Moderator] In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband leakage ratio:

Agreement: - New requirement for in-channel adjacent subband leakage ratio should be defined - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline.

- Proposals on assumption:

  - Proposal 1 (CMCC): Identify the assumption of adjacent network performance before defining sub-band Tx leakage.

  - Observation 1 (Qualcomm): RAN4 has not specified before in-band emission requirements for legacy TDD BSs as it is implementation dependent and different solutions might vary among vendors.

- Proposals on whether ACLR-like requirement or not:

  - Option 1 (CMCC/Samsung/CATT/Ericsson/Huawei/Qualcomm): ICASLR requirement is defined with existing ACLR requirement as baseline.

  - Option 1a (Ericsson):

    - Reuse the concept of the existing ACLR (absolute PSD value) when specifying SBFD in-channel adjacent sub-band leakage requirement.

    - Specify -13 dBm/MHz as the requirement level for the SBFD in-channel adjacent sub-band leakage.

  - Option 1b (Nokia):

    - Existing ACLR requirement should be used as baseline for the in-channel adjacent subband leakage ratio depending on the ratio between the bandwidths of the DL and the UL subbands.

  - Option 2 (ZTE):

    - if companies have strong preference to define the Tx in-band leakage requirement, then prefer to define it similar as UE in-band emission general requirement which could be applicable for any DL sub-band transmission bandwidth configuration and also UL sub-band transmission bandwidth configuration.

- Moderator Recommendation:

  - Discuss on the above options firstly.

  - Based on the majority view, Option 1 can be agreed firstly.

##### Issue 3-2-2: In-channel adjacent subband leakage ratio (ICASLR): Details for DU case

- Proposals on DU case configuration for ICASLR requirement

  - Option 1 (Samsung):

| BS DL subband bandwidth BWSBFD,DL (MHz) | BS UL subband bandwidth BWSBFD,UL (UL) | Filter on UL subband and corresponding filter bandwidth | In-Channel Adjacent Subband Leakage Ratio (ICASLR) limit |
| --- | --- | --- | --- |
| BWSBFD,DL as configured by gNB | BWSBFD,UL as configured by gNB | Square (BWSBFD,UL) | TBD |

- Moderator Recommendation:

  - This issue can be further discussed if ACLR-like requirement is confirmed.

##### Issue 3-2-3: In-channel adjacent subband leakage ratio (ICASLR): Details for DUD case

- Proposals on DUD case:

  - Proposal 1 (Samsung): For SBFD DUD case, the in-channel adjacent subband leakage ratio (ICASLR) requirement shall be defined by considering the existing ACLR requirement in non-contiguous spectrum or multiple bands as baseline.

  - Proposal 2 (Huawei): Whether to use the power sum of two DL subbands for DUD case should be further discussed.

Moderator Recommendation:

  - This issue can be further discussed if ACLR-like requirement is confirmed.

##### Issue 3-2-4: In-channel adjacent subband blocking (ICASB) requirement

- [Moderator] In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband blocking requirement, while no need to introduce new requirement for in-channel adjacent subband selectivity:

Agreement: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration.

- Considerations are provided for defining ICASB requirement:

  - Proposal 1 (CATT): The in-channel adjacent sub-band blocking requirement should be defined based on feasibility consideration.

  - Observations 1 (Ericsson): The interfering signals of In-channel selectivity (ICS) for SBFD come from UE and are placed on the UL subband, while the interfering signal of in-channel adjacent subband blocking comes from other sectors and is placed on the DL subband.

  - Observation 2 (Ericsson): In-channel selectivity (ICS) cannot cover in-channel adjacent subband blocking.

  - Observation 3 (Huawei): Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

  - Proposal 2 (Qualcomm): RAN4 consider the impact of inter-CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement.

  - Proposal 3 (Nokia): Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes.

  - Proposal 4 (Nokia): Reasonable expected gNB-to-gNB isolation should be the worst-case scenario to ensure functionality in the field.

  - Observation 4 (ZTE): increasing Iot level due to BS2BS CLI problem in the DL sub-band region if SBFD BS is capable of receiving the whole carrier instead of UL sub-band only and without considering the BS CLI coordination within the carrier is larger than legacy 25dBc for FR1 ICS requirement and also 14dBcfor FR2 ICS requirement except for FR2 WA scenario.

  - Observation 5 (ZTE): BS co-channel coordination, e.g. beam nulling or beam coordination could alleviate the increasing IoT level for SBFD BS reception.

  - Proposal 5 (ZTE): not to define in-channel adjacent subband blocking requirements if received BS2BS CLI within the carrier manged lower than in-band blocking power level.

- Proposals on specify ICASB requirement:

  - Option 1 (Samsung): For in-channel adjacent subband blocking (ICASB), the requirement shall be defined similar to BS in-band blocking requirement.

  - Option 2 (Ericsson): For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

- Moderator Recommendation:

  - Discuss on the above proposals/options firstly.

  - Since the assumptions for in-band interference can be different, try to see the assumption given in Ericsson’s Option 2 firstly