# 3GPP TSG-RAN WG4 Meeting #114	R4-2502578

# Athens, GR, 17th – 21st February, 2025

Agenda item:			7.25.3

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes for Netw_Energy_NR_enh WI

Document for:	Information

# Introduction

This document is the Ad-hoc minutes for Netw_Energy_NR_enh WI with the following topics covered.

Note: The following terminologies from RAN1 are re-used.

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

- Scenario #3A refers to “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

- Scenario #3B refers to “When SCell activation is completed and SCell is activated” or “After SCell activation is completed and SCell is activated”

Note: The following abbreviations are used in the summary.

- AO-SSB: Always-on SSB

- OD-SSB: On-demand SSB

- FMW: Fast Measurement Window

# Topic #1: On-demand SSB(OD-SSB) requirements (AI 7.25.2.1)

# OD-SSB based deactivated SCell measurement procedure (Case 1)

- Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

  - The OD-SSB based deactivated SCell measurement procedure is shown as follow.

  - The agreements in last meeting are as follow.

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed

- Sub-issue 1-1 – The length of FMW

### Moderator Proposal 1-1

  - The time window is defined with uncertain time plus actual measurement duration.

    - The actual measurement duration from UE performing OD-SSB based measurement in a fast mode until the time point = min(Treport, Tminimum).

      - Treport is the latest valid OD-SSB based L3 measurement reporting occasion

      - Tminimum equals OD-SSB meas. period as follow.

        - A detected cell: OD-SSB based measurement period

        - A newly detectable cell: OD-SSB based cell identification period

    - No measurement requirement applies in fast mode if NW deactivates the OD-SSB earlier than Tminimum

  - Once the UE has sent a valid measurement report, the UE should be allowed to ignore the OD-SSB periodicity and fall back to legacy measurement mode.

  - FFS UE to indicate the number of OD-SSB samples to NW

  - FFS if the side condition is not fulfilled, the UE continues performing measurements in ‘fast’ mode during the time window.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Revised | Treport is the time when UE sends a valid measurement report for measurement based on the OD-SSB. |

Tentative agreement:

- The time window is defined with Tuncertain plus actual measurement duration.

  - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows.

  - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

- Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs?

### Moderator Proposal 1-2

  - RAN4 to define a time interval T2 for OD-SSB reactivation.

    - UE is not required to perform “fast measurement” more frequently than the time interval T2.

    - The activation and deactivation of OD-SSB is fully up to NW.

    - The detail value can be FFS

      - Option 1: max(5*measCycleSCell,  5*DRX cycles)

      - Option 2: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity)

      - Option 3: cell detectable time [X]s

      - Option 4: based on the exiting measurement period when MeasCycle =160 ms as reference

    - The condition of reactivated OD-SSB

      - The reactivated OD-SSB has the same frequency, same spatial relation with the deactivated OD-SSB before.

      - Other condition is FFS

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Revised | - UE is not required to perform “fast measurement” during T2. Question to the group: do we need to discuss “The condition of reactivated OD-SSB”? |

Tentative agreement:

RAN4 to define measurement requirement based on T2.

- When NW re-activates the OD-SSB within the T2,

  - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement.

  - Option 2: UE is required to perform “fast measurement” without cell search.

    - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

Option 3: UE is required to perform FMW after T2.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW

  - The UE is assumed to meet fast cell identification and measurement.

- The start point of T2 is based on

  - Option 1: the time point Tend

  - Option 2: the latter point of L3 measurement reporting for FMW and Tend

  - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend

  - Option 4: the time point of OD-SSB deactivation

  - Other options are not precluded

Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3.

- The duration is

  - Option 1: 5*max(measCycleSCell,  DRX cycles) [– OD-SSB periodicity]

  - Option 2: 5* measCycle when measCycle =160 ms

  - Other options are not precluded

- The activation and deactivation of OD-SSB is fully up to NW.

- Sub-issue 1-3 – What’s the UE’s behaviour when OD-SSB further activation/reconfiguration after T2?

### Moderator Proposal 1-3

- When NW re-activates/re-configures the OD-SSB transmission within T2, UE follows legacy deactivated SCell measurement requirement.

  - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition.

    - The detail condition is FFS.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW

  - The UE is assumed to perform both cell identification and measurement.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Revised | - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to perform both cell identification and measurement. Our understanding is that UE measurement behavior is already covered by the previous issue 1-2. |
| Moderator | Closed | Merge with issue 1-2 |

Close the issue. Merge with issue 1-2

# OD-SSB based deactivated SCell measurement(Case 2)

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

### Moderator Proposal 2-1

- Option 1: The same approach in Case 1 can be reused.

  - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

- Option 2:

  - The same approach in Case 1 can be reused if the serving cell is unknown.

  - Otherwise, UE follows legacy requirements based on measCycleSCell.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Support option 2 | If the cell is already known, UE shall not enter “fast mode”. This is not only for UE power saving, but also for mobility performance on other carriers. As proposed by several companies, OD-SSB measurement may have higher priority than other carriers, e.g. UE may suspend measurement on other carriers. |
| LGE | Support | Support option 1, prefer to have same approach as Case 1 |
| Xiaomi | Support Option 1 | We can wait case 1’s discussion |
| vivo | Support Option 2 | For option 1, it is not clear whether UE is still in the coverage of OD-SSB-based SCell after the FMW window closed. UE may or may not provide report to network. In case UE moves outside the coverage window, and moves in again, it is beneficial be perform fast measurement based on OD-SSB. |

Tentative agreement:

When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2),

- Option 1: The same approach in Case 1 can be reused.

  - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

- Option 2:

  - If the serving cell is unknown, the same approach in Case 1 can be reused.

  - If the serving cell is known, UE follows legacy requirements based on measCycleSCell.

    - The known condition reuses the principle of legacy known condition.

    - This scenario applies at least when UE sends the meas. reporting.

  - FFS the case if the L3 meas. report cannot be triggered.

- The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation

- Background

From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible.

  - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different frequencies.

  - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

RAN4 already agreed to support scenario 1.

In last meeting, RAN1 sent the reply LS to RAN4 as follow.

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

### Moderator Proposal 2-2

- RAN4 to support scenario 1 and scenario 4 based on latest RAN1 LS.

- For other scenarios: wait RAN1’s further LS response.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Support | We are also fine if companies want to start from only scenario 1 first. |
| Samsung | Support |  |
| LGE | Support |  |
| Xiaomi | Support |  |

Tentative agreement:

RAN4 to start from the following scenarios:

- Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

- Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities.

  - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity

- The scenarios can be updated based on other WGs’ progress.

Issue 2-3: Combine OD-SSB and AO-SSB in measurement

### Moderator Proposal 2-3

- Option 1: The measurement requirement is defined based on combining the AO-SSB and OD-SSB provided that

  - only applied in the FMW which includes both OD-SSB and AO-SSB, and

  - the periodicity of the union of AO-SSB and OD-SSB equals any defined SSB periodicity(10ms, 20ms, 40ms, 80ms, 160ms) and

  - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

- Option 2: The measurement requirement is defined based on combining the AO-SSB and OD-SSB if

  - they are same periodicity and different offsets on the same frequency with the same spatial relation.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Not support | Feasibility depends on which scenario(s) in issue 2-2 that we need to support. But in general we don’t support combination from RAN4 requirements point of view. |
| Samsung | Not support |  |
| LGE | Support | According to LS from RAN1, same periodicity is included in the scenario. We think AO-SSB and OD-SSB can be combined when they are same periodicity and different offsets on the same frequency with the same spatial relation. |
| Xiaomi | Not support | From the reporting results themselves, the measurement shall be transparent to OD-SSB or AO-SSB. But from RAN4 requirements specification, the other transition time between AO-SSB and OD-SSB shall be considred. This will increase the specification complexity |
| Huawei | Comments on following offline discussion | There are proposals in RAN1 about combine AO and OD-SSB to create shorter periodicity. Thus, we suggest to remove/FFS following bullet. - The effective measurement periodicity should meet any defined SSB periodicity and |

Tentative agreement:

- In scenario 4, the measurement requirement is defined based on the effective measurement periodicity.

  - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and

  - The effective measurement periodicity is half of OD-SSB periodicity

  - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

# OD-SSB based deactivated SCell measurement requirement in FMW (Case 1)

Sub-issue 3-1: Lower bound for measurement in FMW

- Background

  - The agreement in last meeting is as follow.

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

### Moderator Proposal 3-1

- Option 1: RAN4 not to define lower bound to measurement in FMW.

- Option 2: Define a lower bound with [10ms].

  - The lower bound is only applied to measurement in FMW.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Revised option 2 | We proposed 40ms in the first place. We can compromise to 20ms. |
| LGE | Support |  |
| Xiaomi | Revised Option 2 | longer value is preferable. |
| Huawei | Option 1 | The length of the FMW is short. No significant power consumption issues. If the we define a lower bound (e.g. 40 ms), it somehow means gNB can only transmit SSB larger than this value. |

Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers

### Moderator Proposal 3-2

When NW configures single OD-SSB carrier with multiple legacy SSB carriers,

- Option 1

  - UE to prioritize measurement on OD-SSB carrier within FMW

  - Longer measurement delay of other frequency layers with legacy SSBs are expected

- Option 2:

  - Follow the same CSSF as legacy

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Option 1/1-2 |  |
| LGE | Support |  |
| vivo | Support option 1 | Option 1 is beneficial for network energy saving, which is in the WID scope. We can try to minimize the unknown case of a UE if necessary. |
| Huawei | Support | Longer delay are expected. Does it means there is no requirements for other layers measurement? From NW point of views, it is good that we have solution that UE can prioritize the measurement on OD-SSB carrier since all OD-SSB can be used. But we should be cautious that if we assume all other layers have no requirements or delay is longer. It is possible that other frequency layers are also important. If NW is for sure that there is not other neighbour cells to be measured it is fine. We suggest to support the prioritization but based on NW indication. If NW does not other frequency layers can be impacted, then UE just treat this OD-SSB carrier equivalently. |

Moderator Proposal 3-2

When NW configures single OD-SSB carrier with multiple legacy SSB carriers,

- UE to prioritize measurement on OD-SSB carrier within FMW

- FFS: Longer measurement delay of other frequency layers with legacy SSBs are expected

HW: One issue: other frequency meas. may miss mobility. NW should control it.

Vivo: Don’t need NW indication.

QC: What’s the use case. Not agree any indication.

Nokia: The CSSF should be only based on OD-SSB carriers.

E///: No need NW indication.

Xiaomi: Support prioritize the OD-SSB

ZTE: Support. No need NW indication.

HW: have concern on 2nd bullet.

MTK: fine 1st bullet

OPPO: We should agree them together.

Tentative agreement:

When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW

Option 1: The CSSF only counts OD-SSB layers

Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers

### Moderator Proposal 3-3

- When NW only configures multiple OD-SSB carriers,

  - if the FMWs for multiple activated OD-SSB carriers are non-overlapping, CSSF = 1 for each active OD-SSB carrier.

  - if the FMWs for multiple activated OD-SSB carriers are overlapping, FFS

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Revised | First bullet is ok. CSSF needs to be considered in the 2nd bullet, not FFS. If this cannot be agreed, we can exclude this scenario in RAN4 requirements. |
| vivo | Support | Apple’s revision is also OK to us. |
| Huawei | FFS | For the first bullet, even the FMW is not overlapped, but UE is still required to measure OD-SSB after FMW, then the CSSF can be 1. We are fine to consider this case, but we need more time to think about the details on implementation impact. |

Offline discussion:

- When NW only configures multiple OD-SSB carriers,

  - if the FMWs for multiple activated OD-SSB carriers are non-overlapping, CSSF = 1 for each active OD-SSB carrier.

  - if the FMWs for multiple activated OD-SSB carriers are overlapping, CSSF equals the number of active OD-SSB carriers.

CMCC: OK. Suggest merge cases.

HW:  some issues if the 1st carrier still transmits OD-SSB in slow mode.

Xiaomi: We need to discuss the scenarios. Only single OD-SSB carrier.

Nokia: Principle is fine. Focus on single OD-SSB. Complex if multiple carriers.

E///: Principle is fine.

OPPO: Focus on single OD-SSB carrier.

QC: Focus on single OD-SSB carrier.

ZTE: Possible to trigger multiple OD-SSB carriers.

Vivo: It’s difficult to define onverlapping and non-overlapping based on FMW.

MTK: Open to discussion. Suggest to remove 2nd case.

Nokia: It’s too complex. It may some time overlapping some time non-overlapping.

OPPO:

Apple:

ZTE:

HW: Multiple OD-SSB carriers may be important for NES.

Tentative agreement:

RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement.

  - FFS multiple OD-SSB carriers

# OD-SSB general issues

Issue 4-1: L1 measurement before SCell activation

### Moderator Proposal 4-1

- RAN4 not support OD-SSB based L1 measurements for deactivated Scell.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Support |  |
| Huawei | Support |  |
| Samsung | Support |  |
| LGE | Support |  |
| Xiaomi | Support |  |
| Qualcomm | support | Would be a major change in RAN2 since currently no L1-RSRP report is defined for deactivated SCell. Cannot be done in Rel-19 anymore in our opinion |
| vivo | Support |  |
| CMCC | Support |  |
| CATT | Support |  |

Issue 4-2: L1 measurement after SCell activation

- RAN4 to support OD-SSB L1 measurement during and after SCell activation.

  - The current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | support |  |
| Huawei | Support |  |
| Samsung | Support the main bullet | - Not fully agree the current requirement can be reused with the clarification of TSSB is the SSB periodicity of OD-SSB especially for the following configuration: DRX cycle ≤ 320ms - Suggest to consider Measurement priority with relation to DRX first |
| LGE | Support |  |
| vivo | Support |  |
| CMCC | Support |  |
| CATT | Support |  |

### Moderator Proposal 4-2

- RAN4 to support OD-SSB L1 measurement during and after SCell activation.

  - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB

  - FFS the requirement when DRX is configured.

Issue 4-3: OD-SSB accuracy

### Moderator Proposal 4-3

- The legacy measurement accuracy is applied for OD-SSB based measurement.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | support |  |
| Huawei | Support |  |
| Samsung | Support |  |
| LGE | Support |  |
| Qualcomm | support | There is no need to tighten the requirements |
| vivo | Support |  |
| CMCC | Support |  |
| CATT | Support |  |

Issue 4-4: Additional Processing time for OD-SSB reception - scenario

- Background

In last RAN4 meeting, RAN4 achieved agreements as follow.

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

### Moderator Proposal 4-4

- Option 1

Additional UE processing/preparation time for OD-SSB reception:

  - Application scenarios

    - Case 1

    - Case 2 when OD-SSB and AO-SSB are within different frequencies

    - Deactivated SCell L3 measurement

  - Not applied

    - Deactivated SCell measurement

      - Case 1 when OD-SSB has been activated recently

      - Case 2 when OD-SSB and AO-SSB are within same frequencies

    - OD-SSB SCell activation

- Option 2: Additional UE processing/preparation time for OD-SSB reception is unnecessary.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Not support | We support to have the additional UE preparation time by default. It is unclear to us how “Case 1 when OD-SSB has been activated recently” can help. |
| Huawei | Support option 1 | At least for SCell activation case, the additional processing time is not needed. |
| Samsung |  | Also feel confused what does “additional UE processing/preparation time” means here From our understanding, it can refer to at least: - Additional processing time due to searcher limitation - Additional time for RF/BB preparation and RF retuning for the different frequency - UE to receive the complete SSB burst to do AGC or T/F tracking - Others Different bullets corresponding to different additional UE processing/preparation time |
| LGE | Revise | Add “If Time instance A is located within the first possible OD-SSB burst” for application scenarios |
| Qualcomm | Revision of options | Additional time is needed for the UE to be able to process OD-SSB. The same would apply for SSB adaptation in case periodicity is changed by MAC CE. The preparation time applies for deactivated and activated SCell |
| vivo | Not support | Same view as apple. Prefer not to add further applicability of the processing time, which complicates the requirements unnecessarily. |
| CMCC | Revise | Revise the Option 1 as below: - Option 1 Additional UE processing/preparation time for OD-SSB reception: - Application scenarios - Deactivated SCell L3 measurement - Case 1 - Case 2 when OD-SSB and AO-SSB are within different frequencies - Not applied - Deactivated SCell measurement - Case 1 when OD-SSB has been activated recently - Case 2 when OD-SSB and AO-SSB are within same frequencies - OD-SSB SCell activation |
| MTK | Revise | Concern on option 1. still apply for deactivated SCell |

Issue 4-5: Additional Processing time for OD-SSB reception - value

- Background

In last RAN4 meeting, RAN4 achieved agreements as follow.

Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

### Moderator Proposal 4-5

- Option 1: RAN4 to define single UE processing time for OD-SSB reception as [1ms].

- Option 2: RAN4 to define UE processing time based on capability. The candidate values can base on the following aspects.

  - Per-FR

  - With or without AO-SSB

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Revised | We proposed [5ms] for the additional preparation time. |
| Huawei | FFS | It depends on the applicable scenario (above issue) to define additional processing time. |
| Samsung |  | Same view as issue 4-4 |
| LGE | Revise | Issue 4-4 Add as follows: - Option 2: RAN4 to define UE processing time based on capability. The candidate values can base on the following aspects. - Per-FR - With or without AO-SSB - Location of Time instance A |
| Qualcomm | Revision may be needed | If we only have only processing timeline it needs to be based on worst case scenario. Alternatively, we can also define a reference scenario for option 1 for which the timeline holds. For example, FR1, same frequency of always-on and OD-SSB. If we do this is must be understood that for other scenarios the timeline does not hold |
| CMCC | Support |  |

# OD-SSB based SCell activation

Issue 5-1: The known cell condition

### Moderator Proposal 5-1

- RAN4 to further clarify the SSB measured for known cell condition and the SSB used in SCell activation procedure as follow.

  - In Case 2 FR2, the Scell is believed as a known cell if

    - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before the reception,

      - UE has sent a valid L3-RSRP measurement report with SSB index before SCell activation

      - The measured/reported SSB has the same spatial relation and in the same frequency as the SSB used in Scell activation

      - The SSB used in SCell activation has the same SSB index as the SSB measured can be detectable

      - FFS the same transmit power

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Samsung |  | Need more discussion |
| vivo | FFS | Related to TCI configuration, which is not yet clear from RAN1 agreements. Not sure whether there is impact from OD-SSB on the QCL-C configuration of TRS and BM-RS. |
| CMCC | Revised | Need the clarification about whether this issue only for FR2? |
| CATT |  | Need more discussion |

Issue 5-2: The known cell condition – period of known condition

- The time period in OD-SSB SCell activation known condition reuses the time period in legacy known/unknown cell condition, such as max(5*measCycleSCell,  5*DRX cycles) in FR1.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | support |  |
| LGE | Support |  |
| vivo | support |  |
| CMCC | Support | And what for FR2? Issue 5-1. Little confusing here |

### Moderator Proposal 5-2

The time period in OD-SSB SCell activation known condition reuses the time period in legacy known/unknown cell condition.

  - max(5*measCycleSCell,  5*DRX cycles) in FR1;

  - 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2.

Issue 5-3: SCell activation requirement – uncertainty time

- RAN4 to introduce the SSB transmission uncertainty duration TFirstOD-SSB, TFirstOD-SSB_MAX which is the time to the end of the first actual transmitted complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX.

  - FFS TFirstOD-SSB_MAX is based on max SSB periodicity.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Revised | “after the additional preparation time in 4-5” Besides, we need to consider max SSB periodicity for intra-band. |
| Samsung |  | Need further discussion |
| LGE | Revised | Aligned with comments from Apple |
| vivo | Revised | We need to consider max SSB/SMTC periodicity for intra-band. |

### Moderator Proposal 5-3

- RAN4 to introduce the SSB transmission uncertainty duration TFirstOD-SSB, TFirstOD-SSB_MAX which is the time to the end of the first actual transmitted complete OD-SSB burst after the additional UE preparation time, if defined similar as legacy TFirstSSB, TFirstSSB_MAX.

  - FFS TFirstOD-SSB_MAX is based on max SSB periodicity.

# OD-SSB SCell activation scenarios

Issue 6-1: OD-SSB based direct SCell activation

- Background

RAN1 #118 Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

### Moderator Proposal 6-1

- RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | support |  |
| Samsung | Support |  |
| LGE | Support |  |
| Qualcomm | Maybe support | We’re not against it. 2nd priority after multiple SCell |
| vivo | OK |  |
| CMCC | Support |  |
| CATT | Support |  |

Issue 6-2: OD-SSB based multiple SCells activation

### Moderator Proposal 6-2

- FFS RAN4 to define OD-SSB based multiple SCells activation requirements.

  - RAN4 to limit the scope at least as all SCells being activated are OD-SSBs’ SCells

    - FFS whether to further limit the scope

  - It doesn’t impact the WI’s completion.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | No support | Prefer not to consider this scenario at current stage. At least we need to conclude Sub-issue 3-3 first. |
| Samsung | Slightly support | If not to limit the scope as all SCells being activated are SCells supporting NES, we have concerns on whether more workload would be brought |
| Qualcomm | support | Multiple SCell activation is mandatory and should have highest priority for OD-SSB (next to single Scell activation) |
| CMCC | Open |  |
| CATT | Slightly not support |  |

Issue 6-3: OD-SSB based fast SCell Activation

### Moderator Proposal 6-3

- RAN4 to only update the known cell condition based on OD-SSB for Rel-17 fast SCell activation.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

Issue 6-4: Deactivated PUCCH SCell activation

### Moderator Proposal 6-4

- RAN4 to define OD-SSB based deactivated PUCCH SCell activation requirement.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

Issue 6-5: Rel-18 SCell activation with the L3 reporting during activation

- RAN4 to define OD-SSB based Rel-18 SCell activation with the L3 reporting requirement.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Samsung | Open | Depending on different cases (case1 and case 2), and when UE receives the OD-SSB activation indication (no later than SCell activation command), we are not sure whether/how the two features can be combined together easily |
| MTK | FFS | Need clarification on “with the L3 reporting”, does that mean only define requirement for known cell? |
| Huawei | Support |  |

### Moderator Proposal 6-5

- FFS RAN4 to define OD-SSB based SCell activation for UE supporting l3-MeasUnknownSCellActivation-r18.

  - The details can be discussed in the CR directly.

  - It doesn’t impact the WI’s completion.

Issue 6-6: Interruption

- RAN4 to investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated Scell.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | Support | This is an important aspect. When network activates OD-SSB, UE needs to switch on RF chain to perform measurement. Proposal from our side: Upon OD-SSB activation, UE shall be allowed to cause two interruptions (one is before and the other one is after the fast measurement window). After the fast window, interruption is same as legacy. |
| Samsung | Support |  |
| LGE | Support |  |
| Xiaomi | FFS | Whether there is interruption when OD-SSB activated is upon the UE’s behavior. In our view, UE could perform the OD-SSB based measurement without any gap or RF retuning. In that case, there is not any interruption to other serving cells. |
| Qualcomm | Support | We need to analyze this in our opinion |
| CMCC | Support |  |
| CATT | Support |  |
| Huawei | Support | FFS |

### Moderator Proposal 6-6

- RAN4 to investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated SCell.

Issue 6-7: SSB-less Scell and OD-SSB Scell differentiation

- When the UE supports both OD-SSB and SSB-less Scell activation for intra-band contiguous CA or inter-band CA,

  - if NW indicates the OD-SSB transmission during the Scell activation, UE shall follow the OD-SSB based Scell activation procedure;

  - if the conditions meet SSB-less Scell activation conditions, UE shall follow the SSB-less Scell activation requirement;

  - otherwise, no requirement is applied

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Apple | support |  |
| LGE | Revise | The priority of the first sub- bullet and second sub-bullet is unclear. |
| Vivo | Revised | Same comment as LG. Our understanding: - if the conditions meet SSB-less Scell activation conditions, UE shall follow the SSB-less Scell activation requirement; - Otherwise, - if NW indicates the OD-SSB transmission during the Scell activation, UE shall follow the OD-SSB based Scell activation procedure; - otherwise, no requirement is applied |
| CMCC | Revise | Same comment as LGE |
| CATT | Revise |  |
| Huawei | FFS | We need to think more about this combination case. |

### Moderator Proposal 6-7

- When the UE supports both OD-SSB and SSB-less SCell activation for intra-band contiguous CA or inter-band CA,

  - if NW indicates the OD-SSB transmission during the SCell activation, UE shall follow the OD-SSB based SCell activation procedure;

  - Otherwise,

    - if SSB-less SCell activation conditions are met, UE shall follow the SSB-less SCell activation requirement;

    - otherwise, no requirement is applied

# Other OD-SSB measurement issues

Issue 7-1: Cell identification

### Moderator Proposal 7-1

The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios.

      - Case 1: No always-on SSB on the cell

        - Option 1: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the OD-SSB is transmitted again with the same spatial reception parameter.

          - FFS whether UE needs to send a valid measurement report

          - FFS with same transmission power.

(Note: this option has already covered in issue 1-2-3)

        - Option 2: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

        - Option 3: Extend referenceCell to OD-SSB SCell

      - Case 2: Always-on SSB on the cell

        - Option 1: Cell identification can be always skipped in scenario 2

        - Option 2: follow SCell activation known condition

        - Other option is not precluded

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Huawei | Support | For case#1, we can start from option1/2 For case#2, we need FFS (current option 1/2 we think is not precise). |
| LGE | Support |  |
| Xiaomi | FFS |  |
| vivo | Support |  |
| CMCC | Keep FFS |  |

Issue 7-2: OD-SSB based Neighbour cell measurement

### Moderator Proposal 7-2

    - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell

      - No RAN4 spec. change is expected.

    - UE does not need to differentiate the OD-SSB neighbour cell with legacy SSB neighbour cell.

    - UE follows legacy deactivated SCC measurement requirement.

      - The agreement can be revisited with other WG’s further progress.

    - Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell

      - No RAN4 spec. change is expected.

    - UE follows legacy deactivated SCC measurement requirement.

    - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell

      - No RAN4 spec. change is expected.

    - UE does not need to differentiate the OD-SSB neighbour cell with legacy SSB neighbour cell.

    - UE follows legacy deactivated SCC measurement requirement.

      - The agreement can be revisited with other WG’s further progress.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Huawei | Support and further clarification | We are generally fine with the recommended WF. We think the high-level principles are: - UE is not aware of whether there is a OD-SSB neighbour cell or whether OD-SSB is activated or deactivated. (i.e. UE just follow configured SMTC for neighbour cell measurement) - Neighbour cell measurement is not triggered by whether serving Cell’s OD-SSB activation/deactivation. The situation is now like: - neighbour cell measurement is based on SMTC - serving Cell measurement is based on activated OD-SSB. We would like to discuss whether to consider some solution to ease UE implementation complexity (i.e. different measurement behaviour in the same frequency). E.g. when the OD-SSB serving cell quality is good, UE may only needs to measure the serving cell. |
| Xiaomi | FFS | In current WID, OD-SSB to supporte SCEll operation only. Thus, the measurement on the neighbour with ODSSB is out of scope. |
| vivo | OK to scenario 1a. For 1b, and 1c, wait for RAN2 | For scenario 1b and 1c, at least for case 1 we may have the same issue as R18 SSB-less SCell. Prefer to wait R18 conclusion from RAN2 first. |
| CMCC | FFS | OK with Scenario 1a,1b and 1c, but the details can be FFS |

Issue 7-3: OD-SSB based L3 reporting

    - UE follows legacy L3 measurement reporting for OD-SSB measurement.

    - RAN4 not to introduce an OD-SSB based L3 measurement reporting.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Huawei | Not support | The common understanding is gNB is expecting a timely report upon activating OD-SSB with fast measurement window. By legacy case, what we can imagine is gNB to artificially configured a very low threshold to trigger the event and reconfigure it after report to avoid UL overhead and wrongly triggered event. The situation will be more severe for case#2. We think we can have similar approach as introduced in Rel-18 RRM “report on SCell activation”, and the workload is manageable in RAN2 since framework can be reused. |
| Qualcomm | support | We do not see why we need a specific OD-SSB report |
| Vivo | Not support | We think triggering one report from UE is quite beneficial to let gNB know whether UE is still in the coverage of the SCell, and whether FMW window is needed when gNB wants to trigger another OD-SSB transmission. Therefore, it is too early to conclude based on the moderator’s proposal. |

### Moderator Proposal 7-3

- RAN4 to discuss whether to introduce an OD-SSB based L3 measurement reporting.

| Company | Support or not (Revised) | Comment |
| --- | --- | --- |
| Huawei | support | The common understanding is gNB is expecting a timely report upon activating OD-SSB with fast measurement window. By legacy case, what we can imagine is gNB to artificially configured a very low threshold to trigger the event and reconfigure it after report to avoid UL overhead and wrongly triggered event. The situation will be more severe for case#2. We think we can have similar approach as introduced in Rel-18 RRM “report on SCell activation”, and the workload is manageable in RAN2 since framework can be reused. |

# Topic #2: SSB adaptation requirements (AI 7.25.2.2)

# SSB adaptation

Sub-issue 2-1 – SSB adaptation principle

### Moderator Proposal 8-1

Deactivated SCell measurement (Scenario 2 in OD-SSB):

- For SSB adaption, reuse the same conclusion from OD-SSB case#2 discussion to define RRM requirements.

- At least for the case when the SSB periodicity is adapted to be larger than before

- Can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

For SCell activation (Scenario 2A in OD-SSB)

- For SSB adaptation, reuse the same conclusion from OD-SSB case#2 discussion to define RRM requirements.

- Can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

Tentative agreements:

Deactivated SCell measurement:

- Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements.

  - At least for the case when the SSB periodicity is adapted to be shorter than before

- Option 2: Same as legacy

- Other options are not precluded.

The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes together with SCell activation command.

The requirement can be updated based on further progress in other WG

Sub-issue 2-2 –Applicable L1 requirements for SSB adaptation

- Proposals

  - Option 1: L1-RSRP (Apple, LGE, CMCC, Ericsson, Nokia, ZTE, Samsung)

    - Option 1a: For serving Cell (Apple, CMCC)

  - Option 2: L1-SINR (Apple, LGE, CMCC, Nokia, ZTE, Samsung)

    - Option 2a: For serving Cell (CMCC)

  - Option 3:	BFD (Apple, LGE, CMCC, Huawei, Nokia, ZTE)

    - No: Vivo

  - Option 4: CBD (Apple, LGE, Huawei, Ericsson, Nokia, ZTE)

    - No: Vivo

  - Option 5: L1 measurement during SCell activation (LGE)

  - Option 6: TCI state switching (ZTE)

  - Option 7: RLM

    - No: Nokia

### Moderator Proposal 8-2

- No impact of SSB adaptation:

  - RLM

  - BFD

- Define requirements if SSB adaption happens:

  - L1-RSRP for serving cell

  - L1-SINR

  - CBD

- Whether and how to define requirements at transition is discussed in other issues.

Issue 1-2-3: Transition requirements – details

- Proposals

  - Option 1: RAN4 should analyze whether Tfirst_SSB + meas_period(P2) can serve as transition period for all requirements. RAN4 should also discuss whether a general definition of the transition time is sufficient for ich relaxed requirements apply without defining new test cases.  (QC)

  - Option 2: L1 measurement requirement at transition agreed could not applicable for L1 measurement when M=1 and P=1. (Xiaomi)

  - Option 3: Applicability: SSB adaptation happens during one measurement period for L3 cell-level measurements and L1-RSRP measurement based on more SSB samples (i.e., M=3)  (Samsung)

  - Option 4: For L1/L3 measurement under SSB adaption, the start point of measurement period shall be clarified. (Xiaomi)

- Recommended WF:

  - Discuss the starting/ending point of transition period.

    - (Moderator): The transition period starts from when SSB adaption occurs, and the transition period is fined as:

      - Option 1: Tfirst_SSB + maximum of the measurement/evaluation period. (Agreements in last meeting)

      - Option 2: Tfirst_SSB + measurement/evaluation period of SSB after adaptation

      - FFS processing time for Tfirst_SSB

  - Discuss whether to differentiate requirements at transition w.r.t value of M and P.

### Moderator Proposal 8-3

- For L1-RSRP/L1-SINR/[CBD]

  - From the timepoint when SSB adaptation occurs, the measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB.

  - Tfirst_SSB is uncertainty from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.