3GPP TSG-RAN WG4 Meeting # 114													R4-2502580

Athens, Greece, 17/02/2025 to 21/02/2025

Agenda item:			7.28.3

Source:	Moderator (Nokia)

Title:	Ad hoc minutes for [114][228] NR_XR_Ph3

Document for:	Information

# Agreed workplan (from R4-2414027 for information)

| RAN4 meeting | Tasks |
| --- | --- |
| RAN4 #112 - August 2024 | - Approval of workplan - Initial discussion on scenarios for measurement skipping based on RAN1 conclusions. |
| RAN4 #112bis - October2024 | - Conclusions on scenarios for measurement skipping based on RAN1 conclusions. - Discussion on the need for UE assistance information regarding measurements occasions needed. |
| RAN4 #113 - November 2024 | - Discussion on RRM requirements with measurement skipping. - Discussion on the necessary UE assistance information. |
| RAN4 #114 - February 2025 | - Conclusion on RRM requirements with measurement skipping. - Discussion on the need for UE assistance information regarding measurement skipping. - Agreement of CR work split. |
| RAN4 #114bis - April 2025 | - Conclusion on the necessary UE assistance information.. - First Draft CRs for RRM core requirements. - Draft CRs for 38.133 expected. - First big Draft CR. |
| RAN4 #115 - May 2025 | - Discussion on remaining open issues for RRM core |
| RAN4 #116 - August 2025 | - Discussion on remaining open issues for RRM core . - Revision of CRs for RRM core requirements. - Planning of RRM performance. - Initial discussion on RRM test cases. |

# Ad hoc minutes

#### Issue 1-1: Deployment scenarios

- Proposals related to frequency range

  - Option 1 (Huawei, Mediatek): Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

  - Option 2 (Nokia, Ericsson): FR2-2 is not considered in this WI.

- Proposals related to dual connectivity

  - Options that don’t allow NR-DC

    - Option 1 (Huawei, Mediatek): Deprioritize NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

    - Option 2 (ZTE): Not support for DCI-based measurement gap skipping for NR-DC.

  - Options supporting NR-DC

    - Option 3 (Nokia): RAN4 to consider for XR enhancements only NR-DC scenario with MCG and SCG in different FRs using per-FR gaps.

    - Option 4 (Interdigital): If NR-DC gap skipping is to be supported, semi-static indication/UE assistance combined with DCI dynamic indications solutions are required.

  - Ask other working groups

    - Option 3 (vivo): Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective.

- Recommended WF

  - Proposals on FR2-2

    - Try to agree on Option 1 or 2 frequency range

  - Proposals on NR-DC

    - Please discuss the proposals

- Comments from companies

  - Moderator:

    - Option 1 (Huawei, Mediatek, QC): Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

    - Option 2 (Nokia, Ericsson): FR2-2 is not considered in this WI.

  - QC – Deprioritize

  - Apple – Fine with option 1 or 2.

  - Moderator: understanding, don’t change FR2-2 specific tables without further discussion.

  - Moderator: NR-DC

  - ZTE – We want to align same agreements as EN-DC and NE-DC, because it have the same challenge.

  - Apple – with only DCI, DC cannot be considered but with RRC solution that can be considered. Similar as ZTE

  - Vivo – this is not just RAN4. Has impact to other working groups.

  - Samsung – NR-DC in the WID is only to a RAN3 related objective. Not sure why this topic should be discussed. This topic can be deprioritized. Some companies still thinking on semi static.

- Conclusion in Ad hoc session:

  - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

#### Issue 1-3: L3 Measurements without gaps with scheduling restrictions

- Merged proposals from moderator

  - Option 1: measurement skipping apply for measurements without gaps, considering the following sub-options:

    - Option 1a: no limitation on scenarios for skipping of measurements without gaps

    - Option 1b: Skipping of measurements without gaps depend on UE capability.

    - Option 1c: Apply for 100 MHz BW

    - Option 1d: consider low mobility criteria.

    - Option 1e: Skipping applies for the following scenarios

      - Intra-frequency without gap for SSB within active BWP in FR2-1

      - For UE supporting NeedForInterruptions-r18

  - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported

    - Option 2a: Not applicable

    - Option 2b: Deprioritize discussion.

- Company comments

  - Moderator: try to achieve combined compromise

|  | L3 measurements without gaps with scheduling restrictions |  |  |
| --- | --- | --- | --- |
| L1 measurements | Apply in all scenarios | Apply in limited scenarios | Do not apply skipping |
| Apply for all L1 measurements and procedures | CMCC |  |  |
| Apply on limited scenarios |  | Apple NTT DOCOMO |  |
| Do not apply skipping |  | Apple (compromise) DOCOMO (compromise) | Qualcomm Interdigital Mediatek Ericsson ZTE Vivo Nokia Samsung |

.

      - CMCC – we can compromise with not skipping L1 measurments.

      - DOCOMO – Fine to not consider L1 measurements.

      - Apple – open to discuss all measurement without gaps. Want to consider the case that if the UE skips a measurement gap, and the UE can measurement without gaps with scheduling restriction? As baseline we have scheduling restrictions in this case

      - QC – Disagree. We need to agree that the cancelation doesn’t mean that if there was a collision of MG with something else, we deprioritize the other measurements. So the UE is not required to do the other measurements. E.g. SMTC colliging with skipped gap, than scheduling restriction of no-gap is also cancelled, if existing.

      - Agreement: If measurement gap is skipped, and is overlapping with SMTC or L1 RS used for measurements without gaps with scheduling restrictions,

        - Option 4: The UE is not required to perform other measurements (i.e. L3 measurements without gaps and L1 measurements), and the UE shall not cause scheduling restrictions during the skipped measurement gap occasion.

    - Conclusion in Ad hoc session:

      - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions.

      - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported.

      - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

#### Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Conclusion in Ad hoc session:

  - See agreement on 1-3

#### Issue 1-5: L1/L2 triggered mobility measurements and procedures

- Merged proposals form the moderator

  - Option 1: Support skipping of L1 measurements and procedures for LTM.

    - Option 1a: Apply at least for 100MHz CBW band

    - Option 1b: Apply only for measurement within gaps

  - Option 2: Not to support skipping of L1 measurements and procedures for LTM.

  - Option 3: reuse decision as for normal L1 measurements.

- Comments from companies

- Conclusion in Ad hoc session:

  - See agreement on 1-3.

#### Topic 8 (new) List of CRs for worksplit

| Clause | Topic | Dependency |
| --- | --- | --- |
| 8.xr (new) Or 9.1 | Time offset for DCI based measurement skipping Time offset for cancellation of DCI based measurement skipping | Agreement |
| L3 measurement impact |  |  |
| 9.2.6 | Intra-freq measurements with gaps | Agreement |
| 9.3.4 | Inter-freq measurements with gaps | Agreement |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements | Agreement (supported types of measurements) |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements | Agreement (supported types of measurements) |
| 8.2.2.2.19 | Interruptions due to measurements without gap carried out by UE supportingNeedForInterruptionInfoNR | Pending decision on L3 without gaps |
| 9.2.5 | Intra-freq measurements without gaps | Pending decision on L3 without gaps |
| 9.3.9 | Inter-freq measurements without gaps | Pending decision on L3 without gaps |
| 9.9 | NR measurements for positioning | Pending decision that gap-based and/or gapless positioning measurements are impacted by measurement skipping |
| 9.10 | CSI-RS based L3 measurements | Pending decision on CSI-RS L3 measurement impact |
| 9.11 | NR measurements with autonomous gaps | L3 measurement impact |
| 9.12 | Measurement for Propagation Delay Compensation | L3 measurement impact |
| L1 measurement impact |  |  |
| 8.1.2 | RLM SSB Minimum requirement | Pending decision on L1 measurements |
| 8.1.3 | RLM CSI-RS Minimum requirement | Pending decision on L1 measurements |
| 8.1.7 | RLM scheduling availability | Pending decision on L1 measurements |
| 8.5.2 | BFD, SSB-based Minimum requirement | Pending decision on L1 measurements |
| 8.5.3 | BFD, CSI-RS based Minimum requirement | Pending decision on L1 measurements |
| 8.5.5 | CBD, SSB Minimum requirement | Pending decision on L1 measurements |
| 8.5.6 | CBD, CSI-RS Minimum requirement | Pending decision on L1 measurements |
| 8.5.7 | Scheduling availability BFD | Pending decision on L1 measurements |
| 8.5.8 | Scheduling availability CBD | Pending decision on L1 measurements |
| 8.18.2 | TRP specific BFD - SSB based | Pending decision on L1 measurements |
| 8.18.3 | TRP specific BFD - CSI-RS based | Pending decision on L1 measurements |
| 8.18.5 | TRP specific CBD - SSB based | Pending decision on L1 measurements |
| 8.18.6 | TRP specific CBD - CSI-RS based | Pending decision on L1 measurements |
| 9.5.4.1 | L1-RSRP SSB | Pending decision on L1 measurements |
| 9.5.4.2 | L1-RSRP CSI-RS | Pending decision on L1 measurements |
| 9.5.6 | Scheduling availability during L1-RSRP measurement | Pending decision on L1 measurements |
| 9.8.4.1 | L1-SINR with CSI-RS based without IMR measurement requirements | Pending decision on L1 measurements |
| 9.8.4.2 | L1-SINR with SSB-based with IMR measurement requirements | Pending decision on L1 measurements |
| 9.8.4.3 | L1-SINR with CSI-RS based with IMR measurement requirements | Pending decision on L1 measurements |
| 9.8.5 | L1-SINR scheduling restrictions | Pending decision on L1 measurements |
| LTM measurement impact |  |  |
| 9.14 | Intra-frequency L1-RSRP measurements for neighbour cell | Pending decision on LTM measurements |
| 9.15 | NR inter-frequency L1 measurement | Pending decision on LTM measurements |

- Comments from companies

- Conclusion in Ad hoc session:

#### Issue 4-3: Formulas for measurement delay extension

Moderator: In order to give more clarity on the proposals of Issue 2, it is proposed to discuss based on the formulas.

- Proposals

  - Options inspired by NR-U requirements

    - Option 1 (Nokia): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

      - Max(200 ms × CSSFinter, Ceil( 8  * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

      - where Nskip is the number of  skipped MG occasions overlaing with SMTC in the measurement period.

      - Use this formula as example and follow similar approach for the other cases.

    - Option 2 (Moderator): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

      - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

      - where Nskip is the number of skipped MG occasions overlaing with SMTC in the measurement period.

      - Use this formula as example and follow similar approach for the other cases.

  - Options based on scaling factor like in concurrent gaps

    - Option 3 (Moderator): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

      - Max(200 ms, Ceil( 8  * Kgap * Kskip) × Max(MGRP , SMTC period)) × CSSFinter,

      - where Kskip = Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

      - Use this formula as example and follow similar approach for the other cases

- Comments from companies

  - MTK -no strong preference, skipped must be compensated in the measurement delay. Extending the number of occasions skipped.

  - QC – Delay is different. Scaling factor, it is different to determine in this manner. In NRU we have undeterministic way. For DCI NRU based approach takes better into account what is cancelled.

  - Vivo – Support NR-U approach. Number of samples to be extended should be multiplied by CSSF. Ex. 3 MOs on same SMTC. So it must be multiplied to the CSSF. We have 2 cases. The other one is no impact to measurement requirements.

  - Nokia: can go with NRU approach. We have one option regarding reduction of measurement delay. We should consider if we can reduce the measurement delay. The question is if it is multiplied by CSSF this is extending the delay much more.

  - Apple: Support NR-U approach. We also have a similar requirement in NRU where we determine the maximum number of skipped samples. Formulas are only agreeable if we know the maximum number of skipped occasions.

  - QC, Huawei, vivo – support Apple’s view. Important ot have an upper limit so that we don’t exstend too much the delay.

  - CMCC – how to skip is up ot network configuration. If we have upper limit. What about the case where network cancels more than that limit.

  - QC – in our view, measurement period don’t apply if the skipping is larger than the limit.

  - Huawei: if UE in good condition, not a problem not to have  requirements due to too much skipping

  - Interdigital: the problem is more about if space between available samples is not too large. Its more about how one sample becomes obsolete.

  - MTK – ok to focus on how to extend?

  - Samsung – We can use dopler to calculate coherence time for determining the maximum number of samples that can be skipped.

  - Nokia: The network is aware of the environment. So it can decide based on previous measurements if skipping is ok.

- Conclusion in Ad hoc session:

  - Agreement: Follow NR-U-like approach for designing the formulas for measurement delay extension.

#### Issue 6-1: General on UAI

- Proposals merged by the moderator

  - Proposals on general principles

    - Proposal 1: RAN4 will not specify any UE requirements dependent on XR UAI (if such is to be defined).

    - Proposal 2: RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI, in case the latter is to be supported.

  - Proposals related to measurement occasions

    - Option 1: Do not define UAI related to measurement occasions.

    - Option 2: The UE indicates a pattern of unused gaps

    - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

    - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability.

  - Proposals related to channel conditions

    - Option 1: Provide information on channel conditions

    - Option 2: do not provide information on channel conditions

- Comments from companies

- Conclusion in Ad hoc session:

#### Issue 4-1: How to capture impact of measurement skipping

- Merged proposals from moderator

  - Option 1: Discuss first the requirements without UAI consideration.

Option 2: Apply legacy measurement delay for the following condition

    - If the measurement skipping does not exceed the limit/positions indicated by UAI report

  - Option 3: High priority layers don’t extend measurement delay if skipping ratio does not exceed a limit.

- Comments from companies

- Conclusion in Ad hoc session:

#### Issue 7-1: Semi-static schemes

- Recommended WF

  - Further discussion based on recommended options

    - Option 1: Do not consider semi-static solutions

    - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution.

      - Option 2a: RRC pattern-based

      - Option 2b: MAC CE bitmap for skipping indication

      - Option 2c: RRC configuration patterns with MAC CE activation

    - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

- Comments from companies

- Conclusion in Ad hoc session:

#### Issue 7-2: Skipping indication on frequency domain

- Proposals

  - Option 1 (Interdigital, ZTE): For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

  - Proposal 2 (Nokia):

    - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

    - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

  - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

  - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

- Recommended WF

  - This issue has not been discussed yet in RAN4. Companies views is needed.

- Comments from companies

- Conclusion in Ad hoc session:

#### Issue 3-1: Interruptions with measurements without gaps

- Proposals

  - Proposal 1 (Nokia): If interruptions are needed for measurements without gaps, skipping indication cancel the interruptions as well as scheduling restrictions.

- Recommended WF

  - Please discuss the interruption requirements for measurements without gaps

- Comments from companies

- Conclusion in Ad hoc session:

#### Issue 3-2: Applicability of SMTC for measurements without gaps

- Proposals

  - Proposal 1 (Nokia): DCI skipping command applies for an entire SMTC.

- Recommended WF

  - Please discuss the proposal.

- Comments from companies

- Conclusion in Ad hoc session:

Issue 5-3: Time offset requirement for skipping of measurements without gaps

- Proposals

  - Option 1 (Nokia): For the requirements of measurements without gaps, reuse agreement on time offset requirement for measurement gap skipping

- Recommended WF

  - Pending on decision whether measurements without gaps are to be considered for measurement skipping.

- Comments from companies

- Conclusion in Ad hoc session: