3GPP TSG-RAN WG4 Meeting #114	R4-2502591

# Athens, GR, Feb. 17 - Feb. 21, 2025

Title: 	WF on RRM requirements for NR_ATG_enh

Agenda Item:	7.12.4

Source: 	CMCC

Document for:	Approval

<Topic 1> RRM core requirement

<Sub-topic 1-1> General

Issue 1-1-2: UE Antenna Type signaling

Agreement:

- To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed

- FFS for the details.

CR split

| 3.6.16 Applicability of requirements for ATG | CMCC |
| --- | --- |
| 4.2D Cell Re-selection for ATG | CMCC |
| 7.3D Timing advance for ATG | Intel |
| 7.6D(new) Maximum Receive Timing Difference for ATG | Intel |
| 8.1D Radio Link Monitoring for ATG | HW |
| FFS 8.2D(new) Interruption for ATG | Intel |
| 8.3D SCell Activation and Deactivation Delay for ATG | Apple |
| 8.5D Link Recovery Procedures for ATG | HW |
| 8.6D Active BWP switch delay for ATG | Apple |
| 8.19D Pre-configured measurement gap activation/deactivation delay for ATG | Ericsson |
| 9.1D General measurement requirement for ATG | CATT |
| 9.2D NR intra-frequency measurements for ATG | ZTE |
| 9.3D NR inter-frequency measurements for ATG | ZTE |
| 9.5D L1-RSRP measurements for Reporting for ATG | LGE |
| 9.8D L1-SINR measurements for Reporting for ATG | Ericsson |
| 9.10D CSI-RS based L3 measurements for ATG | CATT |

<Sub-topic 1-2> Timing

Issue 1-2-1: MRTD for ATG inter-band CA

Agreement:

- 33µs for the MRTD of ATG

<Sub-topic 1-3> Signalling characteristics

Issue 1-3-1: Interruption requirements

Issue 1-3-1-1 Whether Interruptions at NR SRS antenna port switching requirements need to be introduced

Agreement:

- Introduce interruption requirement at SRS antenna port switching, and legacy requirements can be reused

  - Only DL interruption is considered based on legacy interruption requirement.

Issue 1-3-1-2: Whether Interruptions at NR SRS carrier based switching requirements need to be introduced

FFS

- Proposal 1: The NR SRS carrier based switching interruption requirement can be introduced. (HW)

Issue 1-3-1-3: Is there any impact from UE antenna types for interruption requirements

Agreement:

- Reuse the current interruption requirements, no impact from UE antenna types.

- If any technical issues identified, can be further discussed.

Issue 1-3-2: SCell Activation and Deactivation Delay

Agreement:

- In SCell activation delay requirement, the definition of measurement related component (e.g, TL1-RSRP, measure) should refer to the ATG Clause.

- Inter-band SSB-less SCell activation requirements can be applied to Rel-19 ATG.

Issue 1-3-3: SCell Addition/Activation mechanism

FFS

- Proposal 1: Consider distance triggered SCell addition (CMCC, HW)

  - Proposal 1-1: To save power and avoid throughput loss caused by measurement for SCell addition and/or activation, introduce distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition. After the condition is fulfilled at UE side, UE can add and/or active the SCell automatically, and then sent the SCell addition/activation confirmation message to network.(CMCC)

  - Proposal 1-2: The distance triggered SCell addition and/or activation needs to be discussed with the UE antenna type. (HW)

- Proposal 2: RAN4 should wait for the decision of RF session on case 4 before proceed with the Scell addition/activation mechanism discussion. (Ericsson)

- Proposal 3: further optimization methods for SCell addition and activation can be considered (CATT)

  - Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

<Sub-topic 1-4> Measurement procedure

Issue 1-4-4: Deactivated SCell measurement

Agreement:

- For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed

- FFS: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

Issue 1-4-6: Scheduling Availability/Restrictions of ATG UE with the antenna array performing measurements on FR1

Agreement:

- Principle of Proposal 1 can be agreed, the specific wording can be refined in CR

  - Proposal 1: For scheduling availability of UE with the antenna array performing measurements on FR1 for intra/inter-frequency measurement without MG and CSI-RS based intra-frequency measurements: (CMCC, ZTE, CATT)

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

    - When inter-band carrier aggregation is performed:

      - For ATG UE capable of [antenna array capability] on either PCC or SCC, and support [new UE capability of Case 4], the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

      - For ATG UE capable of [antenna array capability] on either PCC or SCC, [but don’t support [new UE capability of Case 4]], the scheduling restrictions due to PCell will not apply to another serving cell in a different band.

Issue 1-4-7: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

Agreement:

- For ATG UE antenna types except Case 4

  - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement

    - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

    - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured.

- FFS for ATG Case#4 UE

Issue 1-4-8: Sharing factor between L1 measurement and RRM measurement

Agreement:

Agree on the following Proposal 1 for the intra-frequency measurement and L1-RSRP measurement, and apply the similar principle to other related requirements.

- Proposal 1:

  - The Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example):

For UE with the antenna array,

    - Klayer1_measurement=1,

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

    - Klayer1_measurement=1.5, otherwise.

For ATG UE with the antenna array,

    - P value for SSB resource to be measured is defined as

      - Ntotal / Navailable with Navailable > 0

      - Psharing factor * Ntotal / Noutside_MG with Navailable = 0

      - For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any SSB resource occasion:

      - Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

      - Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

      - Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, or not overlapped with any measurement gap occasion nor any SMTC occasion [configured] by same serving cell within the window W when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4].

      - TL1 is periodicity of the target SSB

      - Psharing factor = 3.

Issue 1-4-9: Measurement enhancement for ATG UE with antenna array

FFS

- Proposal 1:

  - If network identify neighbour cell measurement on SCC is not needed by deployment or the measurement report from PCell, then network can configure a signal which indicate UE only measure the SCell when performing the intra-frequency measurement. From requirement point of view, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed.

- Proposal 2:

  - Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.