3GPP TSG-RAN4 Meeting #114	R4-2502657

# Athens, GR, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5332 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_redcap_enh-Core) Correction to TA validation requirements for RedCap R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For inactive RedCap UE, the case that only RAN eDRX cycle is configured by the serving gNB is invalid. The case shall be that only CN eDRX cycle is configured by core network (AMF). In this case, the RSRP1 and RSRP2 measurement shall be specified based on T (determined according to RAN2) inside and outside PTW. - For the case both TeDRX-RAN and TeDRX-CN are configured and TeDRX-RAN ≤10.24s, the RSRP1 and RSRP2 measurement shall be specified based on T (determined according to RAN2) inside and outside PTW. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the invalid case that only RAN eDRX cycle is configured by the serving gNB. - For the case both TeDRX-RAN and TeDRX-CN are configured and TeDRX-RAN ≤10.24s, the RSRP1 and RSRP2 measurement are specified based on T (determined according to RAN2) inside and outside PTW. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TA validation requirements for RedCap are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.2B.3	TA validation requirements for RedCap

When cg-SDT-RSRP-ChangeThreshold TS 38.331 [2] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2 in TS 38.321 [7], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in TS 38.321 [7].

RSRP1 and RSRP2 are considered valid provided that the conditions in table5.2B.3-1 are met for FR1 when UE is not configured with any eDRX cycle by serving gNB or core network.

RSRP1 and RSRP2 are considered valid provided that the conditions in table 5.2B.3-2 are met for FR2 regardless of whether the UE is configured with eDRX cycle by serving gNB or core network (AMF).

RSRP1 and RSRP2 are considered valid provided that the conditions in table 5.2B.3-3 are met for FR1 when UE is configured:

-	only with CN eDRX cycle (TeDRX-CN) by core network (AMF), or

-	with both RAN eDRX cycle (TeDRX-RAN) by serving gNB and TeDRX-CN by the core network (AMF) and TeDRX-RAN ≤10.24s.

Table 5.2B.3-1 Valid measurement for FR1 without eDRX cycle

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640 ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640 ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640 ms, M1*TDRX)) ≤ T2’ ≤ T2 |

Table 5.2B.3-2 Valid measurement for FR2

| Measurement | FR2 |
| --- | --- |
| RSRP1 | (T1 – max{480 ms, 8*SMTC periodicity}) ≤ T1’ ≤ (T1 + max{480 ms, 8*SMTC periodicity}) |
| RSRP2 | (T2 – max{480 ms, 8*SMTC periodicity}) ≤ T2’ ≤ T2 |

Table 5.2B.3-3 Valid measurement for FR1 when RAN eDRX cycle is not configured or RAN eDRX cycle≤10.24s

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640 ms, M1* T)) ≤ T1’ ≤ (T1 + min(640 ms, M1* T)) |
| RSRP2 | (T2 – min(640 ms, M1* T)) ≤ T2’ ≤ T2 |
| NOTE 1: T is determined according to clause 7.1 in [1]. |  |

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally for the initial CG-SDT transmission, the UE shall not transmit in an CG-SDT occasion that occurs more than 640 ms after T2.

Where:

-	T1 is the time when the latest NTAwas obtained by the UE via:

-	RRCRelease with CG-SDT configuration (TS 38.331 [2]) is received, or

-	the latest TA is received if TA is received while in RRC_INACITVE state.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in clause 5.8.2.x in [7] for transmission using CG-SDT.

-	T2’ is the time when the UE has completed RSRP2.

-	TDRX is the DRX cycle length in ms.

-	M1 the scaling factor as defined in clause 4.2.2.2.

<End of Change 1>