3GPP TSG-RAN WG4 Meeting # 114											R4-2502668

Athens, GR, Feb.17 – Feb.21, 2025

Agenda item:			7.23.4

Source:	Samsung

Title:	Ad-hoc #2 minutes for NR_duplex_evo WI

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the coffee break discussion and the second Ad-hoc discussion in [114][222] NR_duplex_evo for the following issues.

Coffee break discussion in RRM (10:30 - 11:00, Wednesday):

Issue 2-1-2: Requirements for CSI-RS based L3 measurement

Second Ad-hoc discussion in RRM (RRM (18:45 - 19:30, Thursdy):

Issue 1-1-5: Scheduling restriction

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

SBFD operation impacts:

- Issue 2-1-2: Requirements for CSI-RS based L3 measurement

- Proposals

  - Proposal 1 (Nokia, CMCC):

    - For L3 CSI-RS mobility measurements, RAN4 to define the measurement requirement assuming the UE is provided with information of the SBFD configuration of neighbouring cells.

  - Proposal 2 (ZTE):

    - For the collision between CSI-RS based L3 measurement and dynamically scheduled UL transmission, determine the serving cell L3 measurement and neighbour cell L3 measurement separately.

      - For serving cell CSI-RS based L3 measurement, apply same principle as that for CSI-RS based L1 measurement.

      - For neighbour cell CSI-RS based L3 measurement, follow existing scheduling restriction to handle the collision.

  - Proposal 3 (CATT):

    - For CSI-RS based L3 inter-frequency measurements, the exiting measurement requirements can be reused.

    - For CSI-RS based L3 intra-frequency measurements, use the exiting measurement requirements as a starting point and details can be further discussed pending on RAN1/2 discussion.

  - Proposal 4 (CMCC):

    - for CSI-RS based L3 measurement, except scheduling restriction, existing requirements can be reused.

    - for CSI-RS based intra-frequency L3 measurement, whether there is scheduling restriction for TDD band can follow the discussion of scheduling restriction for CSI-RS based L1 measurement, which is pending on RAN1 reply LS.

  - Proposal 5 (E///):

    - Wait for RAN2 to conclude on whether there will be L3 neighbour cell SBFD CSI-RS configuration.

  - Proposal 6 (HW):

    - For CSI-RS based L3 measurement, existing requirements apply provided that all CSI-RS resources configured for measurement are available according to the configuration. RAN4 to inform RAN2 that information of the SBFD configuration of neighbouring cells is not needed.

  - Proposal 7 (vivo):

    - The CSI-RS L3 inter-frequency measurement can be performed as that of the legacy. Or more RAN1 input is needed for the SBFD operation where CSI-RS are distributed over SBFD and non-SBFD symbols.

    - For CSI-RS puncturing in frequency domain in SBFD symbols, the neighbour cell CSI-RS L3 intra-frequency measurement will be impacted. The related issues in 1-2-2 needs be addressed for CSI-RS L3 intra-frequency measurement, i.e., the number required for consecutive CSI-RS PRBs when the requirement is applicable; if the consecutive CSI-RS PRBs is satisfied however it is split across the two subbands.

    - The CSI-RS based L3 intra-frequency measurement could be deprioritized.

  - Proposal 8 (MTK):

    - RAN4 shall deprioritize CSI-RS based L3 measurements.

Copy discussion from Chair’s Note below

Discussion: - Discuss and agree on whether to define requirements for CSI-RS L3 measurement when SBFD is operated in any of the serving or neighbour cells. - Deprioritize the requirement: MTK, QC, Apple, E/// - Define the requirement: HW, CMCC, Samsung, ZTE, CATT, Nokia - At least Prioritize serving cell case: Nokia MTK: for the case of DUD, puncturing happens among different neighbouring cells. It is not necessary at least for DUD. HW: Not much difference on DUD and DU. Also issues to be addressed for DU. Not deprioritize. QC: The feature is not very closing to SBFD operation. Need to prioritize different issues. Apple: Not high priority issue. Not specify the requirement. E///: If to define the requirements, RAN2 effort is needed. Not understand the benefit and scenario for the combination of the features. ZTE: At least to consider the collision between the measurement and UL transmission. This is RAN4 driven work. Nokia: Cover serving L3 CSI-RS measurement. Difficult for neighbouring cells. CMCC: Support to define the requirement. No intend to define new requirement, just check the applicability of legacy requirement. Samsung: Support to define the requirement. - Discuss and agree on whether to define requirements assuming UE is aware of SBFD configuration in neighbour cells - Option 1: yes, and UE measurement would account for SBFD operation in neighbour cells, e.g. puncturing in frequency domain and different symbol types - Option 2: no, and UE measurement would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available. HW: only need applicability for option 2. There are legacy UE not supporting SFBD, network anyway needs to handle such UE. MTK: ok with applicability under certain condition. Apple: Option 2 wording is not clear. Need to make the restriction clear. E///: For option 2, it has restriction to network. Network needs to guarantee the applicable condition needs to be applied. CMCC: Option 2 is for UE requirement. No limitation on network configuration. CATT: ok with option 2. Session Chair: Further discuss offline on Option 2. - Discuss priority between CSI-RS measurement on SBFD symbols and UL transmission - For intra-frequency, measurement is performed outside gaps, - Option 1: measurement is prioritized over UL (same as legacy scheduling restriction) - Option 2: UL is prioritized over measurement - Option 3: follow same principle as L1 CSI-RS measurement - Option 4: option 1 for neighbour cell and option 2 for serving cell - For inter-frequency, measurement is performed within gaps, and is prioritized over UL.

Coffee break discussion:

Continue the discussion

  - Discuss and agree on whether to define requirements assuming UE is aware of SBFD configuration in neighbour cells

    - Option 1: yes, and UE measurement would account for SBFD operation in neighbour cells, e.g. puncturing in frequency domain and different symbol types

    - Option 2: no, and UE measurement would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available.

Based on the status, try option 2.

E///: we have the UL RB size. For DU configuration, from NW perspective. There is no issue. For DUD configuration, we still want to wait for further RF session conclusion.

Samsung: For RF in 306, the UL subband is from the allowed CBW. Then D1 and D2 have the same/similar +-1 size different.

E///: there can be some configurations which have small margin to fulfil the condition from NW with D1 and D2 are so small.

QC: without gNB -gNB communication, how can NW ensure the configuration of CSI-RS configuration.

HW: reply to QC: it can depend on the NW implementation. It is not a spec issue for SBFD.

MTK: The may concern. The UE doesn’t know the information from neighbour cell. If NW configure a small size of DL. Fine with option2.

Nokia: same understanding as NW. from UE perspective, when UE received the configuration. only applicability rule. Not mandatory to NW for the size of DL subbands.

Samsung: same as Nokia. Only applicability.

Apple: agree on what we discuss now. UE measurement the CSI-RS resource from the configuration. 
CMCC: option 2 are reasonable and compromise. Capture in WF as it is applicability rule.

E///: for DU. No issue. Fine for option 2. For DUD, still want to wait for conclusion from RF session. We should be careful for the deployment.

Agreement:

Whether to define requirements for CSI-RS L3 measurement when SBFD is operated in any of the serving or neighbour cells

For DU case:

- UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols.

FFS on DUD case

UE-to-UE CLI handling:

- Issue 1-1-5: Scheduling restriction

- Proposals

  - Proposal 1 (ZTE):

    - The number of restricted symbols is determined by the timing assumption without cell phase error.

    - For the non-default case, the Rx beam of CLI resource maybe different from data, so additional scheduling restriction besides legacy R16 CLI for FR2 is needed for PDCCH/PDSCH.

    - For the default case (i.e. assume Rx beam same as the latest PDCCH/PDSCH), same scheduling restriction as legacy R16 CLI is applied.

  - Proposal 2 (CATT):

    - UL scheduling restrictions are needed for both FR1 and FR2 irrespective of Rx beam assumptions, and UL scheduling restrictions in Rel-16 can be reused.

    - For DL receptions, when UE does not support cli-SRS-RSRP-FDM-DL or cli-RSSI-FDM-DL in FR1 and FR2, or CLI resources are not QCL-ed with TypeD to one of the latest received PDSCH or the latest monitored CORESET in FR2, scheduling restrictions are defined where the number of restricted symbols equals the number of CLI resources.

  - Proposal 3 (E///):

    - The scheduling restriction due to the L1-SRS-RSRP on how many OFDM symbols can be further discussed when the measurement timing and error being agreed.

    - The scheduling restriction due to L1-CLI-RSSI applies in OFDM symbols when the measurement resource is not type-D QCL-ed with the DL reception PDCCH/PDSCH/CSI etc in FR2.

  - Proposal 4 (HW):

    - For L1-SRS-RSRP and L1-CLI-RSSI Method #3, R16 requirements are re-used as baseline for both FR1 and FR2, and exact number of restricted symbols is FFS.

    - For L1-CLI-RSSI Method #1, scheduling restriction applies only in OFDM symbols on which the UE performs the measurements. For DL, scheduling restriction applies when the measurement resource is not type-D QCL-ed with the DL reception in FR2.

  - Proposal 5 (Samsung):

    - For scheduling restriction, the UE is not expected to receive PDSCH on SRS-RSRP measured OFDM symbols, and on Y data symbols before L1-SRS-RSRP measured OFDM symbols:

      - Y = 1 for 15kHz/30kHz for FR1,

      - Y = 2 for 60kHz for FR1

      - Y = 1 for 60kHz for FR2

      - Y = 2 for 120kHz for FR2

  - Proposal 6 (MTK):

    - The scheduling restriction symbols for L1 based UE-to-UE CLI measurement based Method#2 shall include at least twice the UL/DL transition time.

  - Proposal 7 (QC):

    - RAN4 to reuse R16 scheduling restrictions requirements for L1-SRS-RSRP measurements irrespective of the deployment scenario, i.e., inter-cell or intra-cell for both FR1 and FR2.

    - RAN4 to define scheduling restrictions only on the OFDM symbols to be measured for L1-CLI-RSSI measurements, i.e., no restrictions on the symbols before and after the measurement symbols.

    - RAN4 to discuss whether to define UE capabilities (e.g., cli-SRS-RSRP-FDM_DL and cli-RSSI-FDM-DL) for FDM DL and L1 CLI measurements.

- Recommended WF

  - Companies’ views are still diverse and addressing different aspects of scheduling restriction requirements, it is suggested to check if following are agreeable

  - For L1-SRS-RSRP,

    - For UL, scheduling restrictions always apply, for both FR1 and FR2.

    - For DL, FFS whether and how UE capabilities for FDM DL and L1 CLI measurements (similar to cli-SRS-RSRP-FDM_DL for L3 CLI) are to be defined

      - Option 1: define new UE capabilities

      - Option 2: re-use R16 UE capabilities

      - Option 3: no UE capabilities, and UE supports FDM DL and L1 CLI measurements

      - Option 4: no UE capabilities, and UE does not support FDM DL and L1 CLI measurements

    - For DL,

      - if UE does not support FDM DL and L1 CLI measurements, scheduling restrictions apply.

      - if UE supports FDM DL and L1 CLI measurements, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2.

    - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource, and Y is FFS

      - Option 1: re-use same number as R16 requirements

      - Option 2: derive the number based on the timing offset side condition

      - Option 3: Y=0

    - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition.

  - For L1-CLI-RSSI Methods #1,

    - For UL, scheduling restrictions always apply, for both FR1 and FR2.

    - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2.

    - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed.

  - For L1-CLI-RSSI Method #3,

    - If the measurement timing is same as L1-SRS-RSRP, as baseline, scheduling restriction requirements are same as L1-SRS-RSRP.

    - If the measurement timing is same as L1-CLI-RSSI Method #1, as baseline, scheduling restriction requirements are same as L1-CLI-RSSI Method #1.

    - FFS whether any adaptation is needed on top of baseline.

Agreement:

  - For L1-SRS-RSRP,

    - For UL, scheduling restrictions always apply, for both FR1 and FR2.

Adhoc discussion:

  - For L1-SRS-RSRP,

    - For DL, FFS whether and how UE capabilities for FDM DL and L1 CLI measurements (similar to cli-SRS-RSRP-FDM_DL for L3 CLI) are to be defined

      - Option 1: define new UE capabilities if NW operated as FDM by DL and SRS

      - Option 2: re-use R16 UE capabilities

      - Option 3: no UE capabilities, and UE supports FDM DL and L1 CLI measurements

      - Option 4: no UE capabilities, and UE does not support FDM DL and L1 CLI measurements

For DL, the scheduling restriction does not apply if UE is able to support FDM DL and SRS measurement if [assuming RAN1 support FDM with DL and SRS reception.]

Nokia: we check the legacy L3, UE are assumed to use searcher. Prefer Option#1, not Option 2.

Samsung: support Option 1 for new but with similar functionality. 
QC: we want to check with RAN1 to check if legacy can be used. Fine with Option #1.

E///: fine to new UE capability. Different feature. The concern is whether FDM with SRS and with DL can be supported. No dissection in RAN1.

HW: support Option #1.

HW: why RAN4 cannot define the UE capability.

MTK: has concern. It is might be some RAN1 work.

  - For L1-SRS-RSRP,

    - For DL, the scheduling restriction does not apply if UE is able to support FDM DL and SRS measurement [assuming RAN1 support FDM with DL and SRS reception.]

Only MTK is not fine with the bullet above, need to check RAN1 internally.

Adhoc discussion:

    - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource, and Y is FFS

      - Option 1: re-use same number as R16 requirements

      - Option 2: derive the number based on the timing offset side condition

      - Option 3: Y=0

    - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition.

QC, Nokia: support Option#1.

Agreement:

The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is

    - re-use same number as R16 requirements,

      - Note: this time has accounted for the UL/DL switching time.

    - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition.

SBFD operation impacts:

- Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Proposals

  - Proposal 1 (Nokia):

    - RAN4 should specify one or more conditions for which existing CSI-RS accuracy requirements should apply in case of SBFD DUD operation. Consider one or more of the following options:

      - Existing accuracy requirements shall apply if all the CSI-RS RBs are contained in one of the DL sub bands and the number of consecutive CSI-RS RB is no less than the existing threshold.

      - Existing requirement should also apply if the CSI-RS RBs are split across the two DL subbands, and the number of consecutive RBs in at least one of the DL subbands exceeds the current threshold.

      - If the CSI-RS RBs are split across the two DL subbands and neither of the two DL subbands contain the minimum required number of consecutive CSI-RS RBs, a new threshold can be introduced on the minimum number of CSI-RS RBs that shall be present in each DL subband.

  - Proposal 2 (ZTE):

    - RAN1 has already discussed the CSI processing timeline for the ‘DUD’ case and optional UE capability was introduced to scale or not scale CSI processing timeline. Such CSI processing timeline has been reflected in the L1 reporting delay within CSI framework. So no impact on UE measurement behavior for ‘DUD’ case.

    - The existing accuracy requirements can apply provided the residual CSI-RS bandwidth is not below 48 PRBs even the CSI-RS resource is punctured.

    - This is applicable to CSI-RS based L1-RSRP, L1-SINR, RLM, BFD and CBD measurements

  - Proposal 3 (CATT):

    - When one CSI-RS resource is mapped to two DL subbands, UE shall measure two DL subbands at the same time instance.

    - When the CSI-RS bandwidth is at least 48 PRBs but it is split across the two subbands, the exiting accuracy requirements apply with an additional margin. The value of margin can be further discussed.

  - Proposal 4 (CMCC):

    - for DUD, for CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of usable CSI-RS PRBs is no less than the existing threshold.

  - Proposal 5 (E///):

    - For DUD configuration, the measurement requirement needs to consider the UE capability of the CSI-RS processing time and separate requirements needs to be specified.

    - Simulation evaluation is needed for the CSI-RS resource within SBFD symbol is at least 48 PRBs but it is split across the two sub bands.

  - Proposal 6 (HW):

    - For DUD case, UE should be able to take wideband measurement across 2 DL subbands based on each CSI-RS resource occasion, i.e. measurement period for CSI-RS based L1 measurement is not changed.

    - For DUD case, for L1-RSRP/L1-SINR existing accuracy requirements apply provided that the number of consecutive CSI-RS PRBs is no less than 24 in each DL subband.

    - For DUD case, for RLM/BFD/CBD existing accuracy requirements apply provided that the number of consecutive CSI-RS PRBs is no less than 24 PRBs in one DL subband and no less than 12 in the other DL subband.

    - For DUD case, for RLM/BFD/CBD RAN4 to define relaxed accuracy requirements or update the propagation condition for the case where the number of consecutive CSI-RS PRBs is no less than 12 (but less than 24) in each DL subband.

  - Proposal 7 (CTC):

    - The measurement performance on CSI-RS RB bandwidth at least 48 PRBs across two subbands would be worse than the measurement performance on CSI-RS RB bandwidth with consecutive 48 PRBs, the existing accuracy requirements for CSI-RS based measurements could not be applied.

  - Proposal 8 (vivo):

    - In DUD configuration, when one part of downlink CSI-RS resource bandwidth (consecutive CSI-RS PRBs) satisfies the threshold for the number of consecutive PRBs of requirement and the other part does not satisfy, CSI-RS accuracy will apply for this scenario.

    - When consecutive CSI-RS PRBs of both subbands are less than the threshold of the CSI-RS accuracy requirements however the total number of consecutive PRBs across two downlink subbands satisfy that threshold, a lower bound could be defined and when the number of consecutive PRBs in each of the subband is equal or larger than this lower bound, the CSI-RS accuracy requirement will apply.

    - When UE performs CSI-RS measurement across two disjoint DL subbands, suggested to follow the principle of RAN1’s agreement and define separate UE capabilities to account for the difference of UE processing capability.

  - Proposal 9 (Samsung):

    - For CSI-RS on SBFD symbols for the DUD case, if the CSI-RS total RB bandwidth is at least X (X1+X2) PRBs but it is split across the two subbands, the existing accuracy requirements for CSI-RS based measurement can also be applied, because X1 and X2 are the same or different by only 1PRB.

    - RAN4 to support 24 PRBs of CSI-RS PRBs to cover more deployment scenarios like 20MHz channel BW.

  - Proposal 10 (MTK):

    - For SBFD-aware UE, NW shall configure the UE with the punctured CSI-RS resources by the SBFD subband operation.

    - For DUD configuration, RAN4 to conduct simulation to assess whether there is a need to update the CSI-RS based measurements based on the minimum number of CSI-RS resources.

  - Proposal 11 (QC):

    - In DUD configuration, existing delay and accuracy requirements for CSI-RS based measurements apply only if the number of consecutive CSI-RS PRBs in atleast one of the DL subband is no less than the existing thresholds, i.e., 24RBs for RLM/BFD/CBD and 48RBs for L1-RSRP/L1-SINR.

- Recommended WF

  - Check whether the following is agreeable for CSI-RS based L1 measurement in DUD case.

  - As baseline, existing measurement period for CSI-RS based L1 measurement also apply for CSI-RS in SBFD symbols.

    - FFS whether any condition on CSI-RS BW is needed.

    - FFS whether and how to account for the UE capability on CSI processing time.

  - Note: Assume CSI-RS BW in DL sub-band #1 and sub-band #2 are X1 and X2, and existing accuracy requirements are applicable when CSI-RS is no smaller than X3.

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

  - For Case 1, existing accuracy requirements apply.

  - For Case 2,

    - Option 1: existing accuracy requirements apply

    - Option 2: existing accuracy requirements apply provided X2 >= X4

    - Option 3: relaxed accuracy requirements apply

  - For Case 3,

    - Option 1: existing accuracy requirements apply

    - Option 2: existing accuracy requirements apply provided that X1 >= X4 and X2 >= X4

    - Option 3: relaxed accuracy requirements apply

  - FFS whether simulation is needed to for down selection in Case 2 and Case 3.

  - FFS whether different option is adopted for L1-RSRP/SINR and RLM/BFD/CBD.

  - FFS whether to change X3 for L1-RSRP/SINR from 48 to 24.

Adhoc discussion:

Agreement: