3GPP TSG-RAN4 Meeting #114	R4-2502870

Athens, Greece 17th – 21st February, 2025

Title: 	WF on requirements for 7MHz channel bandwidth

Agenda item:	7.9.4

Source:	Ericsson

Document for:	Approval

# Scope

Agreement:

- The 7 MHz channel bandwidth is only applicable to FDD bands that also support 5 MHz channel bandwidth.

3-1-2: NB-IoT support

Agreement:

- In-band NB-IoT can be supported in 7MHz channel BW

# System parameters – Release independence

## 2-1-1 Spectrum utilization

Agreement:

- 35 RBs will be supported.

## 2-2-1 Release independence

Agreement:

- Support of 7 MHz channel BW could be Release independent from Rel-15

  - FFS on if the maximum initial BWP size shall then be restricted to 5 MHz.

  - Support of 7MHz is optional in Rel-19 and in the previous releases

2-1-2: Channel spacing, channel raster and sync raster

Agreement:

- Introducing 7 MHz channel BW has no impact on channel spacing, channel raster nor sync raster.

# UE RF

## 2-3-3 NS_100

Agreement:

- There is no impact on A-MPR requirements for NS_100.

2-3-4: NS_12, NS_13, NS_14, NS_15

Agreement:

- For n26

  - NS_12 and NS_15 shall be supported for 7 MHz channel BW.

  - To be further discussed if NS_13 and NS_14 shall also be supported.

2-3-1: SEM

Agreement:

- Same SEM requirement for 7 MHz and 10 MHz channel BW.

2-3-2: MPR

Agreement:

- No MPR impact when introducing 7 MHz channel bandwidth.

2-3-5: Minimum Output power, Transmit Off power, Occupied bandwidth, ACLR, general spurious

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 3-20 MHz. 7 MHz shall be added to:

  - Table 6.3.1-1 (minimum output power).

  - Table 6.3.2-1(Transmit Off power).

  - Table 6.5.1-1 (occupied bandwidth).

  - Table 6.5.2.4.1-1 (ACLR).

  - Table 6.5.3.1-1 (general spurious).

2-4-2: Maximum input level

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 3-20 MHz. 7 MHz shall be added to Table7.4-1.

2-4-3: ACS, In-band blocking, Out of band blocking, Narrow Band blocking, Intermodulation, Spurious response

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 5 and 10 MHz. 7 MHz shall be added to:

  - Tables 7.5-1, 7.5-3 and 7.5-4 (ACS).

  - Table 7.6.2-1 (In-band blocking).

  - Table 7.6.3-1 (Out of band blocking).

  - Table 7.6.4-1 and evaluate Pw (Narrow band blocking).

  - Table 7.8.2-1(Intermodulation) and Table 7.7-1(Spurious response).

# BS RF

3-1-1: BS type

Agreement:

Conducted requirements shall be covered within this WI

Open issue:

FFS if BS OTA requirements should also be covered within this WI.

3-1-3: Multi-carrier and CA

Agreement:

- Muti-carrier (including MSR) and CA related requirements are considered for new added 7MHz WID but no new BCS is specified within this WI.

3-1-4: IAB and NCR

Agreement:

- 7 MHz should not be supported by IAB

- 7 MHz should not be supported by NCR

# Work effort split

According to the Work Plan, only Draft CRs for BS and UE specifications for generic requirements (TS 38.1010-1, TS 38.104, TS 37.104 and [TS 37.105]) should be provided for the RAN4#114bis meeting.

|  | Companies |
| --- | --- |
| TS 38.104 | Ericsson |
| TS 38.141-1 | Nokia |
| [TS 38.141-2] | Huawei |
| TS 38.101-1 | CR responsible: Qualcomm Sub-parts split (draft CRs only): System parameters: Nokia Tx: Huawei Rx and Annexes (if needed): Qualcomm |
| TS 38.101-4 | Qualcomm |
| TS 38.133 | ZTE |
| TS 38.307 | China Telecom |
| TS 37.104 | ZTE |
| TS 37.141 | ZTE |
| TS 38.106 (*) | Ericsson |
| TS 38.115-1 (*) | CATT |
| [TS 38.115-2] (*) | CATT |
| TS 38.174 (*) | Nokia |
| TS 38.176-1 (*) | Nokia |
| [TS 38.176-2] (*) | Huawei |
| TS 38.113 | ZTE |
| TS 37.113 | Ericsson |
| [TS 37.105] | Huawei |
| [TS 37.145-1] | Huawei |
| [TS 37.145-2] | Huawei |
| (*) The NCR and IAB specs would need to be updated to clarify 7 MHz channel BW is not supported. Note: TS in [] are pending on agreement on OTA BS requirements to be in the scope or not. |  |