3GPP TSG-RAN WG4 Meeting #114			    								               R4-2502876

Athens, Greece, February 17-21, 2025

Agenda item:			7.8.4

Source:	Apple

Title:	WF on low-low band switching

Document for:	Approval

# Way Forward

Issue 1: Scope clarification

-	Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI

-	It is not precluded that such two low bands are further combined with the additional band

-	The combination of those two low bands with other band can be further discussed in the future release

Issue 2: Switching period values

Summary of company proposals (for information):

5us: 3 (Spreadtrum, Huawei, Ericsson) 13us, assuming no LO retuning: 2 (Ericsson, Huawei) 35us: 6 (Huawei, Qualcomm, ZTE, CATT, Spreadtrum, Xiaomi) 70us: 1 (Samsung) 100us: 2 (Apple, Murata) 140us: 9 (Apple, Qualcomm, ZTE, CATT, MediaTek, Spreadtrum, Samsung, Xiaomi, Murata) 140 + FFS us: 1 (Skyworks) 200us: 1 (Google) 210us: 4 (Qualcomm, ZTE, Samsung, Xiaomi)

-	Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations

-	35us, 140us

-	RAN4 to futher discuss 13us without LO switching

-	LS will capture the currently confirmed values

-	It is not precluded to send another LS with more values in future meetings

Issue 3: Switching period capability

-	Discuss whether the switching delay capability can be per band combination

Issue 4: Time mask for switching

-	Discuss the time mask for switching based on the following options:

-	Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL)

-	Option 2: Switching period is always within the switch-from carrier

-	Option 3: Switching period is contained within the switching gap configured by the network

-	Other options are not precluded

Issue 5: Switching period location

-	Discuss the switching period location based on the following options:

-	Option 1: Switching period location is entirely up to RAN1 design

-	Option 2: Switching period location is defined relative to the slot boundary

-	Other options are not precluded

Issue 6: UE reference architecture

-	Companies are encouraged to contribute proposals on the UE reference architecture as a basis for developing RF requirements for the feature

Issue 7: Test methodology

-	Companies are encouraged to contribute proposals related to the testability of the time mask requirement

Issue 8: Optional advanced UE features

-	Further consider optional advanced UE features (other proposals are not precluded):

-	In the case of the carrier of band n5 support 4Rx antenna ports but n29 support 2Rx antenna ports, then there are 2Rx antenna ports might not be involved in the carrier switching, UE can optionally report the advanced UE capability to allow the carrier to receive in the receiver antenna port which is not involved in the carrier switching.