3GPP TSG-RAN4 Meeting #114	                                                                 R4-2502926

Athens, Greece, 17th – 21st February 2025

Agenda item:			7.12.4

Source:	CMCC

Title:	WF on UE RF requirements for ATG

Document for:	Approval

Introduction

This document captures the agreements on Rel-19 ATG UE RF requirements in RAN4#114.

Topic #1: R18 UE RF requirements maintenance

Issue 1-1: Tx requirements definition for two types of ATG UE antenna

Proposals from Apple, Keysight, CMCC: - All transmit signal quality requirements should be specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antennas(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array.

WF:

- All transmit signal quality requirements should be specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antennas(s)

- FFS for the ATG UE with the antenna array.

Issue 1-2: Rx requirements definition for two types of ATG UE antenna

Agreement during RAN4#113: - Rx antenna port requirements of the ATG UE need to be specified at the antenna connector(s) or TAB connectors. - The power should be fed to each connector during the test

WF:

- Companies are encouraged to provide wording on how to capture this agreement in the specification, especially for ATG UEs with antenna arrays.

Topic #2: R19 UE RF for inter-band CA

Issue 2-1: Assumed parameters when supporting n39 UL for CA_n3A-n39A with common antenna.

Agreement:

- Companies are encouraged to provide MSD values based on the following parameters.

| Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | [35dB] |
| n39 Tx filter rejection at 1805~1830MHz | [50dB] |

- Above parameters are only applicable for ATG UE with dedicated filter design

Issue 2-2: MSD for CA_n3A-n39A with n39 UL

Agreement:

- Companies are encouraged to provide MSD values based on the following Tables.

  - MSD analysis based on separated antenna are not precluded.

- All of these MSD values are only applicable to ATG UE.

- Further study on how to capture these MSD values to in the specification.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | ACLR2 and >ACLR2 |

Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| 23dBm | TBD |
| 24dBm | TBD |
| … | … |
| 39dBm | TBD |
| 40 dBm | TBD |

Issue 2-3: Assumed parameters for CA_n3A-n39A with n39 DL

WF:

- Use the following parameters in the approved Tdoc R4-2410641 with 20dB antenna isolation as baseline to analysis the MSD value for n39 DL.

  - Other parameters are not precluded.

| Parameters | Value |
| --- | --- |
| Antenna isolation | [20dB] |
| Tx attenuation at 1880~1920MHz for band n3 duplexer | [30dB] |
| Rx attenuation at 1710~1785MHz for band n3+n39 Rx filter | [40dB] |

Issue 2-4: MSD for CA_n3A-n39A with n39 DL

WF:

- Companies are encouraged to provide MSD values based on the following Tables.

  - Other MSD test configurations are not precluded.

- All of these MSD values are only applicable to ATG UE.

- Further study on how to capture these MSD values to in the specification.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | [1722.5] | [25] | [15] | [160 (RBstart=0)] | [1900] | [30] | MSD Table Y | [ACLR2 and >ACLR2] |

Table Y CA_n3-n39 with band n39 DL MSD requirements for different output powers

| Output power | MSD (Based on Antenna Architecture with common antenna) |
| --- | --- |
| 23dBm | TBD |
| 24dBm | TBD |
| … | … |
| 39dBm | TBD |
| 40 dBm | TBD |

Issue 2-5: whether preclude Case 4.

Issue 1-1-2: UE antenna type for inter-band CA Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

Agreement:

- ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination.

  - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation.

  - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other.

  - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

Topic #3: R19 UE RF for UL-MIMO

Issue 3-1: ULFPTx mode for UL-MIMO

- Proposals - Option 1: ULFPTx Mode is not relevant to this feature. (ZTE) - Option 2: ULFPTx mode still applies for ATG UE to support different implementation (CMCC, LGE, Ericsson) - Option 3: Depend on ATG UE implementation whether to leverage the existing ULFPTx mode. (Huawei)

WF:

- Since ULFPTx Mode is an optional feature, ULFPTx mode still applies for ATG UE with one or multiple omni-directional antennas(s)

- FFS for the ATG UE with the antenna array.

Issue 3-2: whether preclude the single antenna port related requirement

- Proposals - Option 1: no (CMCC, LGE, Ericsson, Huawei) - Option 2: not to consider the single antenna port transmission TxD for ATG UE operation. (ZTE)

WF:

- Single antenna port related requirement should not be precluded.

Topic #4: R19 UE RF for DL-MIMO

Issue 4-1: ATG UE RF requirements for UL-MIMO

Agreement:

- Since the WID does not include the UE RF part for DL-MIMO, there is no requirement impact for ATG UE DL MIMO.