3GPP TSG-RAN WG4 Meeting # 114	R4-2502942

Athens, Greece, 17th – 21st February 2025

Title:	TP TR 36.719-01-01 adding CA_41A-106A, CA_41C-106A and CA_41A-41A-106A

Source:	Ericsson, Anterix

Agenda item:	6.4.2

Document for:	Approval

# 1	Introduction

This contribution is a text proposal for TR 36.719-01-01[1] to add CA_41A-106A, CA_41C-106A and CA_41A-41A-106A.

# 2. Reference

[1] TR36.719-01-01, Rel-19 LTE Advanced CA for x bands DL (1<=x<=6) with y bands UL (y=1, 2) V0.0.2

# Text Proposal

# ---Start of changes---

### 5.4.x	CA_41-106

#### 5.4.x.1	Channel bandwidths per operating band for CA

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
| 106 | 896 MHz | – | 901 MHz | 935 MHz | – | 940 MHz | FDD |

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_41A-106A | CA_41A-106A | 41 |  |  | Yes | Yes | Yes | Yes | 23 | 0 |
|  |  | 106 | Yes | Yes |  |  |  |  |  |  |
| CA_41C-106A | CA_41A-106A | 41 | See the CA_41C Bandwidth combination set 3 in Table 5.6A.1-1 |  |  |  |  |  | 43 | 0 |
|  |  | 106 | Yes | Yes |  |  |  |  |  |  |
| CA_41A-41A-106A | CA_41A-106A | 41 | See CA_41A-41A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 43 | 0 |
|  |  | 106 | Yes | Yes |  |  |  |  |  |  |

#### 5.4.x.2	Co-existence studies

Table 5.4.x.2-1 summarizes harmonics and frequency ranges where IMD components due to  CA_41A-106A could fall.

Table 5.4.x.2-1: IMD analysis CA_41A-106A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 896 | 901 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1792 | 1802 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2688 | 2703 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3584 | 3604 | 9984 | 10760 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1794 | 1595 | 3392 | 3591 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 694 | 4091 | 4484 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4288 | 4492 | 5888 | 6281 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3588 | 3190 | 6784 | 7182 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2 | 207 | 6587 | 7174 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5184 | 5393 | 8384 | 8971 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9864 | 9083 | 1108 | 894 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11661 | 6080 | 6294 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6278 | 5686 | 2289 | 2692 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9280 | 9872 | 7680 | 8083 |

Based on Table 5.4.x.2-1 3rd harmonic issues from UL band 106 into DL band 41 are found.

Based on Table 5.4.x.2-1 the following IMD issues are found:

- 5th order IMD products generated by band 41 and band 106 uplink could have fallen into own Rx of band 41, but this will not happen since band 41 is a TDD band.

- 5th order IMD products generated by band 41 and band 106 uplink may fall into own Rx of band 106.

#### 5.4.x.3	∆TIB and ∆RIB values

∆TIB and ∆RIB values for DC_5_n41 are reused.

Table 5.3.2.3-1: IB,c

| CA_41-106 | 41 | 0.6 |
| --- | --- | --- |
|  | 106 | 0.3 |

Table 5.3.2.3-2: R IB,c

| CA_41-106 | 41 | 0.2 |
| --- | --- | --- |
|  | 106 | 0 |

#### 5.4.x.4	REFSENS requirements

MSD values for UL3/DL1 harmonics are reused from CA_7-8.

Table 5.4.x.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_41A-106A5,6 CA_41C-106A5,6 CA_41A-41A-106A5,6 | 4133 |  |  | -88 | -87.4 | -87 | -86.7 | TDD |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.4.x.4-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_41A-106A CA_41C-106A CA_41A-41A-106A | 106 | 2 | 5 |  |  |  |  | FDD |

IMD5 MSD values for band 106 is reused from CA_n20-n41.

Table 5.4.x.4-3: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | Uplink CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_41A-106A CA_41C-106A CA_41A-41A-106A | CA_41A-106A | 41 | 2656.5 | 10 | 50 | N/A | N/A | TDD | N/A |
|  |  | 106 | 898.5 | 3 | 25 | 937.5 | [10] | FDD | IMD5 |

---End of changes---