3GPP TSG-RAN WG4 Meeting #114										                    R4-2502955

Athens, Greece, 17th-21th Feb, 2025

Agenda item:			6.3.3

Source:	Skyworks Solutions Inc. Qualcomm, Huawei.

Title:	WF on ULCA_n41C MSD test point and IMD range definition

Document for:	Approval

# 1			Introduction

Contributions [1, 2, 3] provided input on mandatory MSD test point and optional fully allocated test points for CA_n40A-n41C DL and CA_n41C-n79A with CA_n41C UL. In this way forward we capture agreements on these test points and also cover the definition of the source of IMD for a fully allocated MSD test point.

# 2			Way forward input

#### 2.1		How to capture the second Full+Full test point

Background:

[1] and [3] propose to capture both the mandatory and optional second test point in the current IMD table used for the mandatory test point while [2] is proposing to capture the second optional Full+Full allocation MSD test point in the corss-band MSD table.

As a compromise it is decided to capture the two test points in the current IMD table used for the mandatory test point as long as some definition/explanation of the source of IMD is provided for the Full+Full case.

Way Forward:

The mandatory 1RB+1RB test point and Full+Full test points are captured in the current MSD table due to IMD in section 7.3A.5 of TS 38.101-1

#### 2.2		CA_n40A-n41C DL with CA_n41C UL

Background:

- The MSD value of the mandatory 1RB+1RB test point in [1] and [3] is already aligned but the RBstart values differ.

  - The RBstarts proposed in [3] is 25kHz away from the center of the victim channel while the RBstarts proposed in [1] is 340kHz away.

  - Both proposal ensures that the entire IMD spectrum fall within the 5MHz victim channel so the closer option can be retained

- The MSD value of the second optional  Full+Full test point with the same allocation and channel configuration  in [1], [3] and [4] are 38.1dB, 34.4dB and 34dB respectively so they can be averaged to 35.5dB.

Way Forward: The two MSD test points in the following table can be captured in a TP to TR in the next meeting for CA_n40A-n41C DL with CA_n41C UL

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD314 |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=52) | 2644.8 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 35.5 | TDD | IMD314,X |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

#### 2.2		CA_n41C-n79A DL with CA_n41C UL

Background:

- The MSD value of the mandatory 1RB+1RB test point in [1] and [3] is already aligned but the RBstart values differ.

  - The RBstarts proposed in [3] is 4.2MHz away from the center of the victim channel while the RBstarts proposed in [1] is 4.9MHz  away.

  - Both proposal ensures that the entire IMD spectrum fall within the victim 10MHz channel so the any option can be retained

- The MSD value of the second optional  Full+Full test point with the same allocation and channel configuration  in [1], [3] and [5] are 5.3dB, 9.7dB and 3dB respectively so they can be averaged to 6dB.

Way Forward: The two MSD test points in the following table can be captured in a TP to TR in the next meeting for CA_n41C-n79A DL with CA_n41C UL

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=89) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=93) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD414 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 6 | TDD | IMD414,X |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

#### 2.3	Source of IMD definition

Background:

First, we can observe in the agreed test points that the source of IMD should be the same for the mandatory 1RB+1RB test point and the optional Full+Full test point. It is based on the calculations of IMD collision table for UL contiguous intra-band CA table in the 2DL and 1UL with 2CC TP to TR template.

One of the possible clarification is that the IMD order captured in the table is the lowest IMD order of the two UL CCs that dominates the interfering power in the victim channel.

As an example, a fully allocated single CC/two contiguous CCs ACLR1 power is dominated by IMD3 while all the odd higher order IMDs overlap with ACLR1.

Way Forward:

Companies are encouraged to provide a clarification of the source of IMD for the test point where there is contiguous fully allocated RB in each CC based on the IMD order dominating the interference power.

Companies are encouraged to provide their view on which document(s) should capture such clarification:

- In the PRD as a rule and/or explanation to the TP to TR process.

- In the related band combination TP to TR as an addtional information to the table calculating the IMD collisions for the mandatory test point.

- In the section 7.3A.5 of TS 38.101-1 as a note or general text.

# 3			References

[1]	R4-2501577 CA_n40A-n41C and CA_n41C-n79A MSD, Qualcomm France, RAN4#114

[2] R4-2501435 Discussion on UL CA_n41C MSD test point and its capture in TS, Huawei, HiSilicon, RAN4#114

[3] R4-2500461 Additional test points for CA_n40A-n41C and CA_n41C-n79A with n41C UL, Skyworks Solutions Inc , RAN4#114

[4] R4-2419037 Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, Huawei, HiSilicon, RAN4#113

[5] R4-2419038 Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C, Huawei, HiSilicon, RAN4#113