3GPP TSG-RAN4 Meeting #114	R4-2502968

Athens, Greece 17th – 21st February, 2025

Title: 	WF on A-MPR for NR band n68

Agenda item:	6.17.1

Source:	Ericsson

Document for:	Approval

.

# 2 	Agreements

## 2.1	Background

The following contributions [1], [2] and [3] shared A-MPR simulation and measurements results. Based on those contributions and additional offline, the following agreements were reached.

## 2.2	NS_26

Agreement:

The following A-MPR regions and associated values for NS_26 were agreed:

Table : A-MPR regions for NS_26

| Channel Bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- |
|  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | ≤ 2.16 | ≥ 1 | A1 |
| 15 MHz | ≤ 4.5 | ≥ 1 | A2 |

Table: A-MPR for NS_26

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 3 | 3.5 |
|  | QPSK | 3.5 | 3.5 |
|  | 16 QAM | 4 | 4 |
|  | 64 QAM | 4.5 | 4.5 |
|  | 256 QAM | 5 | 5 |
| CP-OFDM | QPSK | 5 | 5 |
|  | 16 QAM | 5 | 5 |
|  | 64 QAM | 5 | 5 |
|  | 256 QAM | 6.5 | 6.5 |

## 2.3	NS_36

Agreement:

The following A-MPR regions and associated values for NS_36 were agreed:

Table : A-MPR regions for NS_36

| Channel Bandwidth, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- |
|  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | ≤ 1.08 | >0 | A1 |
|  | ≥ 1.08 and ≤ 2.7 | ≥ 1.8 | A2 |
| 10 MHz | ≤2.16 | >0 | A3 |
|  | ≥ 2.34 and ≥ 3.78 | ≥ 12*SCS*RB_start + 1.62 | A4 |
|  | ≥ 2.34 and ≤ 4.32 | ≥ 0.5*12*SCS*RB_start + 0.63 and < 12*SCS*RB_start + 1.62 | A5 |
|  | >4.32 | ≥ 7.2 -12*SCS*RB_start | A6 |
| 15 MHz | ≤ 4.14 | >0 | A7 |
|  | > 4.14 and ≤ 6.12 | ≥ 12*SCS*RB_start + 2.16 | A8 |
|  | > 4.14 and ≤ 7.2 | ≥ 0.5*12*SCS*RB_start + 0.63 and < 12*SCS*RB_start + 2.16 | A9 |
|  | > 7.2 | ≥ 11.0 -12*SCS*RB_start | A10 |

Table: A-MPR for NS_36

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer /Inner | Outer/ Inner | Outer/ Inner | Outer/I nner | Outer/ Inner | Outer/I nner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM | PI/2 BPSK | 8 | 4 | 13 | 10 | 4.5 | 4.5 | 12.5 | 10.5 | 5 | 5 |
|  | QPSK | 8 | 4.5 | 13 | 10 | 5.5 | 5.5 | 12.5 | 10.5 | 6 | 6 |
|  | 16 QAM | 8.5 | 4.5 | 13 | 10.5 | 7 | 6.5 | 12.5 | 11 | 7.5 | 7 |
|  | 64 QAM | 8.5 | 4.5 | 13 | 10.5 | 7.5 | 7 | 12.5 | 11 | 7.5 | 7.5 |
|  | 256 QAM | 8.5 | 5 | 13 | 10.5 | 8 | 7.5 | 12.5 | 11 | 7.5 | 7.5 |
| CP-OFDM | QPSK | 9 | 5 | 13.5 | 12 | 7 | 5.5 | 13.5 | 12 | 7.5 | 6 |
|  | 16 QAM | 9 | 5 | 13.5 | 12 | 7 | 5.5 | 13.5 | 12 | 7.5 | 6 |
|  | 64 QAM | 9 | 5 | 13.5 | 12 | 7 | 5.5 | 13.5 | 12 | 7.5 | 6 |
|  | 256 QAM | 9 | 6.5 | 13.5 | 12 | 8 | 6 | 13.5 | 12 | 7.5 | 6 |

## 2.4	High A-MPR values

Agreement:

For NS_26 and NS_36, the  A-MPR values ≥ 8 dB might be further updated based on companies’ measurements (to be shared by RAN4#114bis).

# 4	References

[1] R4-2500309, n68: Introduction of the new band and NS_26 and NS_36, Apple

[2] R4-2500423, n68 NS_26 and NS_36 A-MPR proposal, Nokia Preliminary

[3] R4-2501739, A-MPR values  for NS_36 and NS_26, Qualcomm