3GPP TSG RAN WG4 Meeting #114	R4-2502969

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.19.5

Source:	vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Title:	Updated simulation assumptions for beam prediction

Document for:	Approval

- Introduction

In this contribution, the link-level simulation assumption for AI/ML based BM case-1 is further updated based on [1]. Companies are encouraged to provide simulation results to evaluate the impact of the measurement error based on the performance metrics in section 5.

- Simulation assumptions for baseband error

Link level simulation (LLS) assumptions for evaluation of measurement error on AI/ML based BM case-1 performance are defined in Table 1-5.

Table 1 Baseline Link Level Simulation assumptions for AI/ML in beam management evaluations

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| Subcarrier spacing | 120kHz |
| Channel model | Case 1: NLOS channel - Option 1: - TDL-C, DS=100ns - Option 2: - CDL-C, DS=100ns |
|  | Case 2: AWGN channel |
| BS antenna configurations | - Option 1: - One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ as baseline. - Number of Tx beams is 32 - Option 2: - Number of TX beams: 1 - Baseband errors are independent for each TX beam Companies are encouraged to provide the codebook used in SLS |
| BS antenna element radiation pattern | Option1: Table 2 Option 2: N/A |
| BS antenna height and antenna array down-tilt angle, if needed (Option 1 above) | Option1: 25m, 110° Option 2: N/A |
| Beamforming characteristic of the BS pattern | Option 1a [corresponding to BS antenna configurations option 1]: 32 beams with grid of 4 elevation angles from [-25 to 25] and 8 azimuth angles from [-60 to 60] Option 1b [corresponding to BS antenna configurations option 1]: The codebook construction can be clarified by companies. Option 2: N/A |
| UE antenna configurations | Option 1: Panel structure: (M, N, P) = (1, 4, 2), - 2 panels (left, right) with (Mg, Ng) = (1, 2) as baseline - 1 panel as optional Number of Rx beams is up to UE Option 2: 2 Rx |
| UE antenna element radiation pattern | Option 1: Table 3 Option 2: N/A |
| UE moving speed | 3km/h |
| Reference signal | SSB or CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | Baseline: - 1 sample Optional: - 3 samples - 5 samples |
| Note 1: TXRU weights mapping for BS and UE is up to company. |  |

Table 2 BS antenna radiation pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 3 UE antenna radiation pattern model

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in dim (dB) |  |
| Antenna element radiation pattern in dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
| Note: are in local coordinate system. |  |

Table 4 Cell-specific parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 |
| SSB periodicity | ms | 20 |
| SSB bandwidth | RB | 20 |
| SSB SCS | kHz | 120 |
| RB Utilization | % | 100 |
| Data Modulation | - | QPSK |
| CP Length | - | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| Relative Delay of 1st Path (synchronous) | µs | 0 |
| SNR | dB | -3 Note: Simulation with other SNR levels are not precluded, e.g., for determining the SNR for ideal measurement. |
| Note: When analog beamforming is used in LLS, SNR value of above row represents the post beamforming SNR of the strongest Tx-Rx beam pair. For other Tx-Rx beam pairs, there are two options. Option 1: Apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS. When analog beamforming is not used in LLS, Apply error distribution based on the SNR of above row to the strongest Tx-Rx beam pair’s measurements in the SLS Option 1: Apply error distribution from the SNR of above row to all Tx-Rx beam pairs’ measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pairs’ SNR values and apply to the corresponding Tx-Rx beam pair measurements in the SLS. SNR of the strongest Tx-Rx beam pair can be fixed or not fixed. |  |  |

Table 5 CSI-RS configuration parameters

| Parameter | Value |
| --- | --- |
| CSI-RS bandwidth | 48 PRBs; |
| CSI-RS SCS | 120 kHz |
| Periodicity | 5 ms (optional); 40 ms |
| CSI-RS configuration: <X,D,N,CDM>, where: X=number of CSI-RS ports, D=density [RE/RB/port], N=number of OFDM symbols in the same slot | <32,1,4,FD-CDM2> |

- Simulation assumptions for RF error

The impact of RF errors should be considered. Following assumptions can be used to generate RF errors.

- Truncated Gaussian distribution under ±4dB RF error (μ=0,  σ=2) is used

- Dependence for each TX beam

  - Option 1: Independent for each TX beam

  - Option 2: Dependent to the strongest TX beam

    - RF error of each SSB is dependent with respect to RF error of strongest SSB.

    - Assume strongest SSB’s RF error is +X dB in one instance.

    - Other SSBs’ RF error will be conditioned with respect to +X dB.

    - For example, if RF error were uniform, other SSBs’ RF errors will be uniform in [X-Y, X+Y] dB range where X depends on company.

- Reference model

For better alignment of the performance between companies, it would be better to perform simulations based on a reference model. The model description in Fig. 1 is an example of reference model for BM-case 1 performance evaluation, while it is not the intention to preclude other reference models.

Fig. 1 Reference model description

- Performance metrics

At least the following performance characteristics are to be provided

KPI 1:

- Top K/1 without considering some margin

  - the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams", K=1,2,3,4,5

- Top K/1 with considering some margin; The margin can be implemented as:

  - Alt 1: Extend Top K/1 to Top K/N

    - the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams", K/N= 2/2, 2/4, 3/2, 3/4, 4/4, 5/2;

    - the percentage of "the Top-N genie-aided beams are two of the Top-K predicted beams", K/N= 2/2, 2/4, 3/2, 3/4, 4/4, 5/2, which means among the Top-K predicted beams, there are two beams are parts of top-N genie aided beams.

  - Alt 2: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB

KPI 2:

- 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the RSRP of the genie aided strongest beam

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ideal L1-RSRP of the same beams

Note:

90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP))

- Simulation procedures (For information)

We provide the following procedures for companies to perform simulations to evaluate the impact of measurement error in BM prediction Sceanario-1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement.

- Companies to generate ideal L1-RSRP dataset from the SLS assumption defined in TR 38.843 Table 6.3.1-1.

- Use subset samples of the ideal dataset generated from the 1st step for training models.

- Filter UEs in the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) which SNR> -3dB (use this SNR range as the starting point) to derive the ideal dataset for inference and to evaluate the performance metric without considering measurement errors.

Note: UE distribution is 100% outdoor

- Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors.

- Use the Simulation assumptions for RF error defined in Clause 3 to generate RF errors.

- Add the measurement errors generated from LLS into the ideal SLS dataset to derive the dataset with measurement errors. The measurement errors will be:

  - Case 1: Only baseband errors are included

  - Case 2: Only RF errors are included

  - Case 3: Both baseband errors and RF errors are included

- Use the dataset with measurement errors for inference and to evaluate the performance metric with considering measurement errors.

- References

- R4-2420480, Updated simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)