3GPP TSG-RAN WG4 Meeting #114	R4-2503037

Athens, Greece, 17 ‒ 21 February, 2025

Title: 	WF on UE RF requirements for MIMO enhancement

Agenda Item:	7.22.4

Source: 	Samsung

Document for:	Approval

<Topic 1> 3Tx for single carrier

<Sub-topic 1-1> Power class

<Way forward>: PC for 3Tx requirement

- For PC3 and PC2, specify requirements for FWA and handheld UE

- FFS on PC1.5 with 3Tx, will be decided in the next meeting

  - Option 1: Consider it in the next release

  - Option 2: Specify requirements in Rel-19

<Sub-topic 1-2> MPR

<Way forward>: PC2 for 3Tx

- Option 1: Use the same MPR as MPR for PC2 with 2Tx, as it’s for UL-MIMO and from simulation results

- Option 2: Use the same MPR as MPR for PC2 with 1Tx, as it’s the worst case

<Topic 2> Asymmetric DL sTRP/UL mTRP

<Sub-topic 2-1> UE RF impact

<Way forward>: RAN4 will decide between following options in the next meeting

- Option 1: No UE RF impact of asymmetric DL sTRP/UL mTRP

- Option 2: Adopt following dCR, where the companies are encouraged to propose wording improvements in the next meeting

## 6.6M	Beam correspondence for [UEs with different TCI states for UL and DL]

For a UE that supports [[future_sTRP_2TA_capability] and the IE unifiedTCI-StateType is configured as ‘separate’, and the QCL type D reference signals associated with the UL and DL TCI states are different,] the requirements of clause 6.6 apply. Side-conditions in clause 6.6 apply to each of the TCI states.