3GPP TSG-RAN WG4 #114bis	R4-2503125

Wuhan, China, April 7-11, 2025

Agenda item:	7.25.2.2		Adaptation of Common Signal/Channel Transmission

Source: 	Qualcomm Incorporated

Title: 	About RRM requirements for adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started. Several agreements have been made for SSD adaptation in RAN4 #114 in [1] as listed below. The agreements are as follows:

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

  - Further clarification on the scope of RRM impact for SSB adaption:

    - No RRM impacts for SSB adaptation requirement for PCell:

    - Frequency location is same before and after SSB adaptation.

    - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation

  - Note: Above could be further updated based on further RAN1 progress if any.

Issue 1-2-1: Applicable L1 requirements for SSB adaptation

Agreement:

- No impact of SSB adaptation:

  - RLM

  - BFD

- Define requirements if SSB adaption happens:

  - L1-RSRP for serving cell

  - L1-SINR

  - CBD

  - Whether and how to define requirements at transition is discussed in other issues.

Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details

Agreement:

- For L1-RSRP/L1-SINR/CBD

  - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB.

  - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation.

Issue 1-4-1: SCell activation requirement for SSB adaptation

Agreement:

For SCell activation:

- Define the SCell activation requirements based on the SSB periodicity after SSB adaptation.

  - For the case SSB adaptation command comes together with SCell activation command.

The requirement can be updated based on further progress in other WG

Issue 1-4-2: TCI state switching requirements for SSB adaptation

Agreement:

  - No requirements when SSB adaptation happens during TCI state switching.

In this paper we provide our views for the open issues identified in the way forward in [1] and in [2] in RAN4 #114.

# Discussion

In RAN4 #112bis it was agreed that RAN4 takes as a starting point for SSB adaptation: CONNECTED mode, SSB adaptation or SCell, SSB adaptation for NC-SSB and SSB periodicity adaptation in time domain. In RAN4 #112bis an LS has been sent to RAN1 to get further information [3]. The Reply LS has been received in [4].

RAN1 confirmed that so far it was only agreed to adapt SSB burst periodicity, Also, there is no change in frequency before and after SSB adaptation. The QCL relation for the same SSB index before and after SSB adaptation is unchanged. Finally, RAN1 confirmed that at least the case is supported that there is no change in transmitted SSB before and after SSB adaptation.

Observation 1: RAN1 confirmed that SSB adaptation means adaptation of burst periodicity. There is no change in frequency before and after SSB adaptation. The QCL relation for the same SSB index before and after SSB adaptation is unchanged. It was confirmed that there is no change in transmitted SSB before and after SSB adaptation.

Since no additional agreements have been achieved, RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged.

Proposal 1: RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged.

In addition, in RAN1 #119, no consensus was achieved to support SSB adaptation in time domain for Rel-19 NES-cacapble PCell in connected mode. Hence, RAN4 should not define requirements for PCell. This is already implicitly agreed in RAN4 since no requirements for RLM for SSB adaptation will be defined based on the agreements in RAN4 #114. For clarity, we repeat this here once more.

Proposal 2: RAN4 should not define requirements for SSB adaptation for PCell.

One discussion point in RAN4 #114 was the definition of the transition duration when SSB burst periodicity is adapted from P1  P2 as illustrated in Figure 2-1. For this transition period, it was agreed in previous RAN4 meetings that the requirements apply with a periodicity of max{P1, P2} instead of P1.

Figure 2-1: Illustration of Transition Duration

In RAN4 #114, it has been agreed in Issue 1-2-2 that the definition of the transition period is based on the periodicity P2 after SSB adaptation and applies after Tfirst_SSB. To be precise, the transition period should be defined based on the measurement period based on P2, called meas_period(P2).

Observation 2: The transition period can be defined as Tfirst_SSB + meas_period(P2). Tfirst_SSB is the time till the first SSB with the new period P2 is received. meas_period(P2) is the measurement period based on the new periodicity P2.

This is illustrated in Figure 2-1. If P1 < P2, then the transition period following requirements based on max{P1, P2} does not imply any relaxation. If P1 > P2, then the transition period following requirements based on max{P1, P2} implies a relaxation of the requirement.

The time Tfirst_SSB needs further discussion. In the agreement for Issue 1-2-2, Tfirst_SSB is defined as the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation. In RAN1 #120 it has been agreed that SSB adaptation is indicated by DCI 2-9. RAN2 may additionally introduce a new MAC CE to indicate SSB adaptation to the UE, however, this is not yet clear whether this will happen.

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Tfirst_SSB cannot be defined as the first SSB after the reception of the switch command in DCI 2-9, but additional processing time needs to be considered. Tfirst_SSB should instead be defined as the first SSB that leaves enough processing/preparation time after the reception of the switch command in DCI 2-9. In TS 38.213, the timeline for DCI 2-9 is defined as 3 ms for activation or deactivation of a current cell DTX operation or cell DRX operation for a second serving cell. It is questionable whether this timeline can be directly applied for SSB adaptation since the UE needs to be ready to process the SSB with the new periodicity. Rather than the Rel-18 3ms timeline for DCI 2-9, a timeline like the MAC CE based timeline for OD-SSB that is also under discussion in RAN4 may be applicable for DCI 2-9. Of course, it remains to be investigated by how much the timeline may be shortened since DCI 2-9 instead of MAC CE is applied.

Observation 3: Tfirst_SSB cannot be defined as the first SSB after the reception of the switch command in DCI 2-9, but additional processing time needs to be considered.

Observation 4: Rather than the Rel-18 3ms timeline for DCI 2-9, a timeline like the MAC CE based timeline for OD-SSB may be applicable for DCI 2-9. It remains to be seen by how much the timeline may be shortened since DCI 2-9 is applied.

Proposal 3: RAN4 should investigate how to define Tfirst_SSB to leave enough processing/preparation time for the UE to receive the SSB with new periodicity after the reception of the switch command in DCI 2-9.

To get further information on the impact to SMTC, a LS has been sent to RAN2 in [5]. In RAN2 #129, RAN2 has agreed on the Reply LS in [6] that is stated here:

RAN2 thanks RAN4 for the LS in R4-2416911 on SSB adaptation. Regarding the question of SMTC impacts due to SSB adaptation, RAN2 made the following agreements:

Agreements on SSB adaptation

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Our understanding of this LS is that RAN2 has agreed to introduce one or more additional SMTC configurations that include different SSB burst periodicities. This agreement is for serving cell only. For neighbor cell, however, the RAN2 agreement says that SMTC is not impacted and legacy SMTC is used. Hence, neighbor cell measurements are not impacted by SSB adaptation.

Observation 5: Based on RAN2 agreement, neighbor cell measurements are based on legacy SMTC and are not impacted by SSB adaptation.

Proposal 4: RAN4 does not need to define new requirements for L3 neighbor cell measurements.

If there are new SMTC for SSB burst adaptation for serving cell measurements, the impact to the RAN4 requirements should be relatively small. Quite often, RAN4 requirements are defined based on the SMTC periodicity TSMTC. All these requirements should be applicable for the new SMTC after the SSB adaptation as well. The main task in RAN4 is then to identify for each case a suitable transition time during which another requirement or even no requirement applies. The transition time depends on the details of the RAN2 agreement that are not yet available. For example, it still needs to be clarified by RAN2 whether always only one SMTC is active or whether multiple SMTCs need to be followed. Also, how the potential switching between active SMTC is triggered is not yet clear, i.e., whether this is done by DCI, MAC CE or even RRC reconfiguration. These details will impact the transition times and, hence, RAN4 should wait till RAN2 has made more progress on the details of the agreement.

Observation 6: The main task in RAN4 is then to identify for each case a suitable transition time during which another requirement or even no requirement applies. The transition time depends on the details of the RAN2 agreement that are not yet available.

Proposal 5: RAN4 should wait till RAN2 has made more progress on the details for introducing additional SMTC for SSB adaptation.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114. Our observations and proposals are summarized as follows:

Observation 1: RAN1 confirmed that SSB adaptation means adaptation of burst periodicity. There is no change in frequency before and after SSB adaptation. The QCL relation for the same SSB index before and after SSB adaptation is unchanged. It was confirmed that there is no change in transmitted SSB before and after SSB adaptation.

Proposal 1: RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged.

Proposal 2: RAN4 should not define requirements for SSB adaptation for PCell.

Observation 2: The transition period can be defined as Tfirst_SSB + meas_period(P2). Tfirst_SSB is the time till the first SSB with the new period P2 is received. meas_period(P2) is the measurement period based on the new periodicity P2.

Observation 3: Tfirst_SSB cannot be defined as the first SSB after the reception of the switch command in DCI 2-9, but additional processing time needs to be considered.

Observation 4: Rather than the Rel-18 3ms timeline for DCI 2-9, a timeline like the MAC CE based timeline for OD-SSB may be applicable for DCI 2-9. It remains to be seen by how much the timeline may be shortened since DCI 2-9 is applied.

Proposal 3: RAN4 should investigate how to define Tfirst_SSB to leave enough processing/preparation time for the UE to receive the SSB with new periodicity after the reception of the switch command in DCI 2-9.

Observation 5: Based on RAN2 agreement, neighbor cell measurements are based on legacy SMTC and are not impacted by SSB adaptation.

Proposal 4: RAN4 does not need to define new requirements for L3 neighbor cell measurements.

Observation 6: The main task in RAN4 is then to identify for each case a suitable transition time during which another requirement or even no requirement applies. The transition time depends on the details of the RAN2 agreement that are not yet available.

Proposal 5: RAN4 should wait till RAN2 has made more progress on the details for introducing additional SMTC for SSB adaptation.

# References

- R4-2502689, WF on RRM requirements for Netw_Energy_NR_enh_Part2, RAN4 #114, February 2025

- R4-2500538, Topic summary for [114][224] Netw_Energy_NR_enh_Part2, RAN4 #114, February 2025

- R4-2416913, LS on SSB relation in on-demand SSB and SSB adaptation on SCell, RAN4 #112bis, October 2024

- R1-2410917, Reply LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN1 #119, November 2024

- R4-2416911, LS on SSB adaptation, RAN4 #112bis, October 2024

- R2-2501484, Reply LS on SSB adaptation, RAN2 #129, February 2025