3GPP RAN4 Meeting #114-bis		R4-2503136

Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.14.2.1

Source:	Ericsson

Title:	Further discussion on XR test method

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Core

Release:	Rel-19

Document for:	Approval

# Introduction

During RAN4 #114 meeting, the following agreement were made in WF [1].

This contribution provides our views related to the remaining issues regarding XR devices performance metrics and measurement time reduction solutions.

# Discussion

## Minimum measurement distance for 50cm QZ

TR 38.870 details the method for calculating minimum range length. This range is defined as the distance from the centre of the quiet zone to the aperture of the measurement antennas/probes as shown below in the figure below.

It is the maximum of the following three limits:

-	The phase uncertainty limit: RQZ+2Drad2/ λ

-	The amplitude uncertainty limit: 3D

-	The reactive Near-Field limit: RQZ+2λ

where RQZis defined as the radius of the quiet zone, i.e., RQZ=D/2, and Dradis the diameter of the effective radiating aperture.

The minimum range length calculations for D=30cm quiet zone size TRP-TRS OTA test systems assume Drad is 30cm below 1GHz and decrease linearly from 30cm to 5cm from 1GHz to 7.125GHz, respectively.

The table below show the minimum range length for a 30cm QZ as a function of the frequency.

The diameter of the effective radiating aperture should not depend on the size of QZ, except where limited by the size of the QZ (below 1GHz). Hence, we recalculated this diameter for a 50cm QZ below 1GHz to continue the linear variation as a function of the frequency. The corresponding Drad changes linearly from 32cm to 5cm from 0.41GHz to 7.125GHz, respectively. Hence the minimum range length for 50cm QZ can be calculated using the same procedure with D=50cm, RQZ=D/2=25cm. The range distance for a 50cm QZ are shown in the table below.

Table 1: Minimum Range Length for NR FR1 TRP-TRS OTA systems with 50cm quiet zone size.

| F [GHz] | Drad [m] | RQZ+2Drad²/λ | 3D = 6RQZ | RQZ+2λ | max(RQZ+2λ,3D,RQZ+2D²/λ) |
| --- | --- | --- | --- | --- | --- |
| 0.41 | 0.32 | 0.53 | 1.5 | 1.71 | 1.71 |
| 0.6 | 0.31 | 0.64 | 1.5 | 1.25 | 1.5 |
| 0.7 | 0.31 | 0.70 | 1.5 | 1.11 | 1.5 |
| 0.8 | 0.31 | 0.75 | 1.5 | 1 | 1.5 |
| 1 | 0.30 | 0.84 | 1.5 | 0.85 | 1.5 |
| 1.2 | 0.29 | 0.92 | 1.5 | 0.75 | 1.5 |
| 1.4 | 0.28 | 0.98 | 1.5 | 0.68 | 1.5 |
| 1.6 | 0.28 | 1.09 | 1.5 | 0.63 | 1.5 |
| 1.8 | 0.27 | 1.12 | 1.5 | 0.58 | 1.5 |
| 2 | 0.26 | 1.15 | 1.5 | 0.55 | 1.5 |
| 2.2 | 0.25 | 1.17 | 1.5 | 0.52 | 1.5 |
| 2.4 | 0.24 | 1.17 | 1.5 | 0.50 | 1.5 |
| 2.6 | 0.23 | 1.17 | 1.5 | 0.48 | 1.5 |
| 2.8 | 0.23 | 1.24 | 1.5 | 0.46 | 1.5 |
| 3 | 0.22 | 1.22 | 1.5 | 0.45 | 1.5 |
| 4 | 0.18 | 1.11 | 1.5 | 0.40 | 1.5 |
| 5 | 0.14 | 0.90 | 1.5 | 0.37 | 1.5 |
| 6 | 0.10 | 0.65 | 1.5 | 0.35 | 1.5 |
| 7 | 0.06 | 0.42 | 1.5 | 0.34 | 1.5 |
| 7.125 | 0.05 | 0.37 | 1.5 | 0.33 | 1.5 |

Observation 1: The diameter of the effective radiating aperture should not depend on the size of QZ, hence the minimum range length for 50cm QZ can be calculated with D=50cm, RQZ=D/2=25cm.

Proposal 1: Use the last column of table 1 for minimum range length with 50cm QZ.

## Testing time reduction solutions for XR OTA

### Coarse measurement grids for XR device testing

#### TRP measurement

According to TS 38.161, “the TRP of the DUT is measured by sampling the radiated transmit power of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step of 15 degrees in both theta (θ) and phi (φ) axes for TRP measurement”. However, TS 38.161 is mainly focused on handheld UEs with a different shape factor and antenna environment compared to XR glasses.  Furthermore, as seen from TR 38.870, even for small size UEs that have quasi-omnidirectional antennas, TRP measurement error increases when increasing the grid step size as shown below:

XR glasses have different form factor and different position compared to handheld UEs. Thus, they can have different antennas that are more sensitive to the sampling grid step. Hence, XR glasses’ TRP measurement should not use coarse measurement grids.

Observation 2: Considering the results given in TR 38.870 and the potential different antenna patterns for XR glasses compared to hand-held UEs’, the XR glasses’ TRP should be measured without using a coarse grid measurement.

Proposal 2: For XR glasses TRP testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 15 degrees.

#### TRS measurement

According to TS 38.161, “the TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step of 30 degrees in both theta (θ) and phi (φ) axes for TRS measurement”. Given, the same arguments as in the case of TRP measurement, XR glasses’ TRS measurement should not use coarse measurement grids.

Proposal 3: For XR glasses TRS testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 30 degrees.

# Conclusions

This contribution provides our views on the remaining issues regarding XR devices performance metrics and measurement time reduction solutions. The contribution makes the following observations and proposals.

Observation 1: The diameter of the effective radiating aperture should not depend on the size of QZ, hence the minimum range length for 50cm QZ can be calculated with D=50cm, RQZ=D/2=25cm.

Proposal 1: Use the last column of table 1 for minimum range length with 50cm QZ.

Observation 2: Considering the results given in TR 38.870 and the potential different antenna patterns for XR glasses compared to hand-held UEs’, the XR glasses’ TRP should be measured without using a coarse grid measurement.

Proposal 2: For XR glasses TRP testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 15 degrees.

Proposal 3: For XR glasses TRS testing, use the same procedure as in TS38.161 without using a coarse measurement grid, i.e. test with a grid size of 30 degrees.

# References

R4-2502370, “Way Forward for [114][327] TRP_TRS_Ph3” vivo, 3GPP RAN4 #114.