# 3GPP TSG-RAN WG4 Meeting #114bis		R4-2503203

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda Item:	7.18.2

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for UE with 8Rx:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

The WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

General

Definition of MMSE-IRC receiver capability for 8Rx UE

Status in the WF [1]: Agreement: - If agreed to change the receiver definition, it also needs to be changed in Rel-18 maintenance. - If receiver definition is agreed to be changed, an LS will be sent to RAN2 for updating TS38.306. Proposals: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability

We face difficulties on how to link the existing Rel-18 8Rx UE receiver capability definition in TS38.306 with new MU-MIMO scenarios:

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

We notice that in RAN#107, there are proposals on introducing more advanced receiver (R-ML) for 8Rx UE in Rel-20. Therefore, if RAN4 agrees to change the receiver definition, option 2 that clearly specifies the receiver algorithm is preferable from us. Otherwise, we may need to further update the capability definition to differentiate MMSE-IRC and R-ML 8Rx receiver in the future.

Proposal 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:

- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

8Rx UE MMSE-IRC receiver coverage

Status in the WF [1]: Agreement: - Criteria for decision if one or two sets of PDSCH requirements are to be introduced: - Secure the span is <=2.5 dB for each receiver type by removal of outliers - If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration. - Further discuss the actual value for the [limit]. - Proposals: - Option 1: Set [limit] to 0.5dB - Option 2: Set [limit] to 1.0dB - Option 3: Set [limit] to 1.2dB - Other options not precluded.

We are open on the detailed [limit] value to decide whether one or two set of requirements should be defined. It sounds more reasonable to us to use 1.0dB which is usually used to differentiate a new feature.

However, above this detailed [limit] value discussion, we aware that the agreed criteria may lead to the situation that ‘define 1 set of requirements for some test cases and 2 sets of requirements for the others’, which will make the performance requirements very hard to read and may also confuse RAN5.

Concerning the above, we propose one more rule on the top of the agreed criteria:

- After reviewing the simulation results for all the test cases by the agreed criteria, if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases.

Proposal 2: Introduce one more rule on the top of the agreed criteria: After reviewing the simulation results for all the test cases by the agreed criteria, if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases.

Proposal 3: Use 1.0dB as the [limit] value.

PDSCH requirements with intra-cell inter-user interference

Rank 4+4

Status in the WF [1]: Proposals: - Option 1: Do not define Rank 4+4 requirements. - Option 2: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

According to the capability definition, both Baseline and simplified 8Rx MMSE-IRC Receiver should support 4 layers from the target PDSCH. In addition, there is no limitation on the rank of the co-scheduled UE. Therefore, rank 4+4 could be a practical scheduling in the field thus should be tested.

Proposal 4: Define requirements for Rank 4+4 for both baseline and simplified receiver.

Applicability rule and release independent

Test applicability

Status in the WF [1]: Test applicability for 8Rx UEs Agreement: - New requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. - TBD: additional test applicability rule. Test applicability to skip 2/4Rx requirements - Proposals - Option 1: Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed. - Option 1a: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 1b: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

For UEs supporting both 8Rx and 2/4Rx, in Rel-18, we have already introduced test applicability rules to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed as shown below.

We believe the same approach can be reused for the Rel-19 new requirement to be defined.

Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2.

Regarding the meaning of ‘corresponding’, we think if 8Rx UE has passed test for the one scenario, it can skip test for 2/4Rx with the same scenario, i.e., option 1a.

Proposal 5: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not.

Release independent

For release independent, the 8Rx UE feature is introduced in Rel-18 with MMSE-IRC as the receiver assumption. The corresponding 8Rx UE performance requirements are release independent from Rel-17 as specified in TS38.307. In addition, the UE MMSE-IRC is a mandatory feature for all Rel-17 and forward 2/4Rx UEs:

Table 5.4-1: Additional requirements of other release independent features Feature; Release independent from; Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced); Further information. UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference; Rel-15; Table B.3.3-1; Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration.. Demodulation requirements for 8Rx UE; Rel-17; Table B.3.5-1. Table B.3.5-1: UE PDSCH demodulation requirements with 8Rx Section / Clause; Description. 5.2.4.1; PDSCH demodulation requirements for 8Rx FDD. 5.2.4.2; PDSCH demodulation requirements for 8Rx TDD. 5.2A.4; PDSCH CA demodulation requirements for 8Rx. 6.2.4; CQI requirements with 8Rx.

We have already discussed the possibility to cover MMSE-IRC requirements during the performance part discussion under WI ‘NR_ENDC_ RF_FR1_enh2’. Such requirements are not introduced in Rel-18 only because of the limited time and huge workload. Therefore, we think the new requirements for 8Rx should also be release independent from Rel-17.

Proposal 6: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

Conclusion

Proposal 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:

- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 2: Introduce one more rule on the top of the agreed criteria: After reviewing the simulation results for all the test cases by the agreed criteria, if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases.

Proposal 3: Use 1.0dB as the [limit] value.

Proposal 4: Define requirements for Rank 4+4 for both baseline and simplified receiver.

Proposal 5: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 6: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

Reference

R4-2502275 Way forward for [114][324] NR_demod_Ph5_Part2_UE, Nokia, TSG-RAN WG4 Meeting #114