3GPP TSG-RAN WG4 Meeting #114bis                      R4-2503208

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 6Rx

Agenda Item:	7.1.3

Document for:	discussion

Introduction

In RAN4#114 meeting, the following conclusion are made in the Core part in [1]:

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications. Issue 2-1-2: 6-layer Support as optional feature for FWA Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following. - Introduce 6 MIMO layers support as an optional feature only for FWA.

The RAN4#114bis is the first meeting we discuss the UE demodulation performance requirements for 6Rx. The following objectives are approved in the latest WID [2]:

6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

According to the WID, RAN4 will define 6-layer performance requirements for FWA and 4-layer performance requirements for FWA and handheld UE devices.

It is also unclear how to capture ‘6-layer is only supported for FWA’ in the RAN4 spec according to the last meeting Core part agreements.

In this contribution, our views on the open issues are given.

Discussion

Test scope

According to the WID [2], all example bands for 6Rx are TDD bands:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104

Therefore, it is proposed to cover TDD 30kHz SCS with 40MHz CHBW.

Proposal 1: Cover TDD 30kHz SCS with 40MHz CHBW.

Receiver assumption

MMSE-IRC receiver is widely used for 2/4/8Rx UEs and is also used as the receiver assumption for 6-layer performance evaluation for 6Rx. Therefore, we propose to use MMSE-IRC as the receiver type for the 6Rx performance requirements.

For 8Rx, 2 types of receivers are defined in TS38.306:

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

For 6Rx UE, we do not think it is reasonable to further divide its MIMO detectors into 2 groups (e.g., 3+3). Therefore, for 6Rx UE performance requirements, we propose to define single receiver type with joint 6Rx MIMO detector.

Proposal 2: For 6Rx UE performance requirements, single receiver type with joint 6Rx MIMO detector should be assumed. And MMSE-IRC should be the receiver type.

Test coverage

In Rel-18 and Rel-19, we have defined performance requirements for 8Rx CPE/FWA/vehicle/industrial devices which can be an example for 6Rx FWA/handheld UEs. The following requirements are defined for 8Rx in TS38.101-4:

8Rx requirements for PDSCH Mapping Type A 8Rx requirements for PDSCH for PDSCH with inter-cell interference (will be introduced in Rel-19) 8Rx requirements for PDSCH for PDSCH with intra-cell inter-user interference (will be introduced in Rel-19) 8Rx requirements for CA FR1 Sustained downlink data rate requirements for FR1 single carrier and FR1 CA Periodic CQI reporting requirements under AWGN Wideband CQI reporting requirements with inter-cell interference under fading conditions (will be introduced in Rel-19)

Based on the approved objective, the scope of performance part is limited to FR1 single carrier, thus the following test coverage is proposed for 6Rx performance requirements:

6Rx requirements for PDSCH Mapping Type A 6Rx requirements for PDSCH for PDSCH with inter-cell interference 6Rx requirements for PDSCH for PDSCH with intra-cell inter-user interference Sustained downlink data rate requirements for FR1 single carrier Periodic CQI reporting requirements under AWGN Wideband CQI reporting requirements with inter-cell interference under fading conditions

Proposal 3: The following test coverage is proposed for 6Rx:

6Rx requirements for PDSCH Mapping Type A 6Rx requirements for PDSCH for PDSCH with inter-cell interference 6Rx requirements for PDSCH for PDSCH with intra-cell inter-user interference Sustained downlink data rate requirements for FR1 single carrier Periodic CQI reporting requirements under AWGN Wideband CQI reporting requirements with inter-cell interference under fading conditions

How to capture 6-layer is only supported for FWA devices in the RAN4 spec

As per the WF [1], it is now open ‘how to capture 6-layer is only supported for FWA devices’ in the RAN4 spec.

Our initial thinking is that, RAN4 can define UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling, and design proper test applicability rules in TS38.101-4 to clearly specify which requirements are applicable for which type of 6Rx UE.

Proposal 4: To capture 6-layer is only supported for FWA devices, RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification.

Conclusion

Proposal 1: Cover TDD 30kHz SCS with 40MHz CHBW.

Proposal 2: For 6Rx UE performance requirements, single receiver type with joint 6Rx MIMO detector should be assumed. And MMSE-IRC should be the receiver type.

Proposal 3: The following test coverage is proposed for 6Rx:

6Rx requirements for PDSCH Mapping Type A 6Rx requirements for PDSCH for PDSCH with inter-cell interference 6Rx requirements for PDSCH for PDSCH with intra-cell inter-user interference Sustained downlink data rate requirements for FR1 single carrier Periodic CQI reporting requirements under AWGN Wideband CQI reporting requirements with inter-cell interference under fading conditions

Proposal 4: To capture 6-layer is only supported for FWA devices, RAN4 can introduce UE declaration on the 6Rx UE type (e.g., handheld or FWA) without capability signaling and design proper test applicability in the specification.

Reference

R4-2503014, WF on 6Rx requirements, AT&T. RAN4#114, February 2025

RP-250780, Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025