3GPP TSG-RAN WG4 Meeting #114bis	R4-2503224

Wuhan, China, 07-11 Apr 2025

Title: 	On single carrier MPR Enhancement

Agenda Item:	7.1.1.2.1

Source: 	Nokia

Document for:	Approval

# 1				Introduction

In this contribution we express our views for single carier MPR enhancements.

# 2	Discussion

In last RAN4#114 meeting, several open issues are addressed, and the following issues are still open based on [1]:

Issue 1-1-3: A-MPR handling - Proposals - Option 1: - If A-MPR is specified for a given RB allocation X, then the UE performs max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs max(MPR(X), A-MPR(X)) - Option 2: - For a given additional emissions requirement and corresponding A-MPR table, if it can be confirmed that A-MPR is specified for all RB allocations X for which A-MPR(X)≥minX∈SMPR(X) where S is the set of all possible RB allocations for the given carrier, then the UE performs max(MPRenh(X), A-MPR(X)) - Option 3: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. In many cases A-MPR is defined for the channel located closest to the additional emission requirement while channels with certain guard gap default to MPR. Those channels could benefit from the power enhancement scheme while minimising the risk for implementation. - Others - WF - FFS, considering that additional emission in most cases are applied on one side of the UE CBW

In our view, in order to handle A-MPR for some channels, the RB allocation and RB region should be with respect to initial CBW, and the following MPR shall be applied by UE: max(MPRenh(X), A-MPR(X))

Proposal 1: In order to handle A-MPR for some channels, the following MPR shall be applied by UE: max(MPRenh(X), A-MPR(X)), where A-MPR(X) is determined based on a given RB allocation, and RB region with respect to initial CBW, WF and modulation etc, and MPRenh(X) is determined based on RB region/etc. within the extended UE CBW.

#### Issue 1-1-4: How to derive the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - For UE CBW with full RB allocation, the IBE of the extended part could be simplified with ΔRB =1 or ΔRB =0. FFS the specific offset value. - IBE is not used in the GB - Option 2: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions (Moderator: NRB,V and LCRB,V could be replaced by other terms, but the concept is similar) max-25-10⋅log10(NRB,V/LCRB,V), 20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,V, -57dBm+10log10SCS/15kHz-PRB - Option 3: Others - WF - FFS Extended IBE can be used in the original GB of UE CBW - FFS the extended IBE in the GB of extended CBW

Based on last RAN#4 113 WF, IBE should be used between edges of UE CBW and extended UE CBW, the main issue still its applicability in the UE CBW guard bands and UE extended CBW guard bands.

UE extended BW definition in rel-19 and the associated RF requirements should not compromise the system performance and/or impact any other UEs in the cell/network.  Since UE extended CBW can be smaller than or equal to at least one BS CBW (i.e., single CC BS CBW (no CA) or aggregated CBWs for the DL-only contiguous intra-band CA case), then UE CBW guard bands and UE extended CBW guard bands could be overlapping with possible RB allocation of the at least one BS CBW, and thus any emissions in those guard bands from one or more UEs will be causing significant interference to other UEs.

Observation 1: UE CBW guard bands and UE extended CBW guard bands could be overlapping with possible RB allocation of the at least one BS CBW, and thus any emissions in those guard bands from one or more UEs will be causing significant interference to other UEs and degrading the system performance.

In order to avoid intolerable interference for other UEs within BS CBW or aggregated CBW for the DL-only contiguous intra-band CA case (i.e., within the at least one BS CBW), IBE shall be applied in all non-allocated RBs, and guard bands for UE CBW and extended UE CBW that are non-overlapping with the guard band(s) for the at least one BS CBW (i.e., BS CBW or aggregated CBWs for the DL-only contiguous intra-band CA).

Proposal 2: IBE shall be applied in all non-allocated RBs, and guard bands for UE CBW and extended UE CBW that are non-overlapping with the guard band(s) for the at least one BS CBW (i.e., BS CBW or aggregated CBWs for the DL-only contiguous intra-band CA) to avoid interference to other UEs in the network.

In other words, UE should apply IBE on all frequency segments within UE extended CBW and/or UE CBW except frequency segment overlapping with BS guard band(s) and the allocated RBs as shown below:

Figure 1a: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are non-overlapping

Figure 1b: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are overlapping in at least one side.

Figure 1c: IBE requirement range within UE extended CBW where BS CBW guard bands and UE guard bands are overlapping in at least one side and UE extended CBW is across multiple DL intra-band contiguous CCs.

Concerning IBE used for extended CBW at each side of UE CBW, the value could be derived from the outmost allocated RB of UE CBW similar to option 1, but while extending IBE limit in the guard bands of UE CBW and extended CBW as well when they are non-overlapping with BS CBW as shown in Fig. 1a-1c.

Proposal 3: The value for IBE limit used for extended CBW at each side of UE CBW is derived from the outmost allocated RB of UE CBW similar to option 1, but while extending IBE requirements in the guard bands of UE CBW and extended CBW as well when they are non-overlapping with BS CBW as shown in Fig. 1a-1c.

## Sub-topic 1-2: Approaches to enable MPR reduction for both scenario 1 and scenario2

Scenarios discussed previous RAN4 meetings:

- Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator)

- Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators)

- Scenario 2: Narrower UE channel BW within wider BS bandwidth

Sub-topic description

#### Issue 1-2-1: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed 0.5* UE CBW is considered - Symmetrical extension is 0.5 * UE CBW on each side. - Asymmetric extension is 0.5 * UE CBW on one side. - Option 2: ≤ 0.5* UE CBW, namely flexible size at each side pending on deployment scenario, and offset of the edges between UE CBW and BS CBW, etc. - Option 3: In scenario 2 no limitations are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion. - Option 4: The CBW extension shall be aligned with existing channel bandwidth sizes. The extension should always result into a total number of RBs (“original UE channel RBs” + “RB extension”) being an element of Table 5.3.2-1 from TS 38.101-1. - Option 5: Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth - WF - FFS in next meeting - The max size of the extension at each side of UE CBW - Whether extended CBW is fixed or could be flexible

#### Issue 1-2-2: How to get the width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed value, i.e., 0.5* UE CBW is stipulated in the spec - Option 2: Flexible value signalled by the NW, the extended CBW at each side ≤ 0.5* UE CBW but not necessarily the one in Table 5.3.2-1 in TS 38.101-1 - Option 2a: Flexible value signalled by the NW, the extended CBW at each side should be the CBW specified in Table 5.3.2-1 in TS 38.101-1 - Option 3: Based on SIB1 cell specific CBW - WF - FFS in next meeting

#### Issue 1-2-4: Approaches of deriving the new inner RB region with RBs in extended CBW - Options - Option 1: Fixed value case with extended CBW - If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region. If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart_Low ≤ RBStart ≤ NRB – LCRB, and LCRB ≤ ceil(2/3 *NRB) If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart ≤ RBStart,High, and LCRB ≤ ceil(2/3 *NRB) - Option 2: Flexible value case with extended CBW at each side - Variety expressions from companies, but essentially are based on the new NRB to derive the new inner region, e.g. - N'RB=Nextended_low+NRB+Nextended_high - Option 3: Using cell specific CBW to derive the new inner region - WF - FFS in next meeting

Concerning the above issues, we support the methodology in Figure 1 (Re-produced from [2]), where the new UE inner RB region is the light green colored area which is the intersection of Extended UE CBW inner and allocation triangle of SIB1 UE CBW. New outer is the light yellow colored area which is New UE allocation triangle – new UE Inner RB region.

Proposal 4: New UE inner RB region the light green colored area which is the intersection of Extended UE CBW inner region and allocation triangle of SIB1 UE CBW. New outer is the light yellow colored area which is New UE allocation triangle – new UE Inner RB region.

- New UE inner RB region is the intersection of Extended UE CBW inner region and allocation triangle of SIB1 UE CBW.

- New UE outer is New UE allocation triangle – new UE Inner RB region.

The best is to define this new method very generic therefore in scenario 2 no limitataions are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited up to BS CBW. Scenario 1 needs more discussion.

Proposal 5: In scenario 2 no limitations are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion.

#### Issue 1-2-3: Guard band for extended CBW - Options - Option 1: if extended CBW at each side is from CBW in Table 5.3.2-1 in TS 38.101-1, the RB number is the maximum transmission bandwidth configuration corresponding to the subcarrier spacing of the component carrier and the extended CBW, GB is determined accordingly - Option 2: guard band for extended CBW is derived from the total RBs including the extended ones, which is larger than the larger guard band between: - The guard band of the CBW which is immediately lower than the bandwidth corresponding to the total number of RBs (UE RBs + extended RB). - The guard band of the CBW which is immediately higher than the bandwidth corresponding to the total number of RB (UE RBs + extended RB) - Option 3: For fixed 0.5* UE CBW case, The GB of the original CBW should be retained, and the ‘GB’ of the extension part should only be retained on the side that is far away from the original CBW. - ‘GB’ of the extension part = [(0.5* BWChannel x 1000 (kHz) – ceil(NRB/2) x SCS x 12) -SCS/2], BWChannel and NRB correspond to the original UE CBW. - Others - WF - FFS in next meeting

Concerning the UE guard band definition, UE cannot be allocated in the UE extended CBW RB outside UE maximum transmission bandwidth in the initial UE CBW and obviously cannot be allocated in the guard bands, so all frequency segments within UE extended CBW and outside UE maximum transmission bandwidth acts as UE total guard bands but with conditional IBE requirements. 
So, it is more meaningful to define the frequency segments between the edge of the UE maximum transmission BW within the initial CBW and edge of UE extended CBW as UE total guard bands, and thus UEs will have only one guard band per each side.

Proposal 6: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band.

IBE requirements should be considered in the guard band parts non-overlapping with BS guard bands.

## Sub-topic 1-3: Signaling

#### Issue 1-3-1: NW signaling - Proposals - Proposal 1: (NTT DOCOMO) - BS indication should be designed to allow selection of which ACLR/SEM/spurious emission can be relaxed to fully enable this feature. - Proposal 2: (MediaTek) - Define RRC signalling from network to UE to permit UE to apply “shifted OOBE + MPR improvement”, in order to allow mobile operators the choice of whether to allow it or not. - Proposal 3: (vivo) - Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered: - It is up to the NW to judge the gap width on both sides of the UE. - When the gap on one side is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends the signaling (e.g., BWExtension -LowerSide-Rel19, or BWExtension-HigherSide-Rel19) to indicate the extension on the corresponding side. - When the gap on both sides is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends two signaling (e.g., BWExtension -LowerSide-Rel19 + BWExtension-HigherSide-Rel19) or one combined signaling (e.g., BWExtension -BothSides-Rel19) to indicate the extension on both sides. - Proposal 4: (Skyworks) - Proposal on RB extension: - Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side RB extensions expressed as: RBextL and RBextH - Maximum RBextL and RBextH is NRB_UE/2, and thus for the BS, this is NRB/2 of the largest supported UE CBW. - Inner and outer allocation can be redefined by the UE, using the existing inner/outer allocation equations where: - NRB is replaced by NRB+RBextL+RBextH - RBstart is replaced by RBstart+RBextL - With 273RB for 100MHz CBW, the maximum RB extension value is 137, thus can be signalled with 8 bits, corresponding to 16bit total for the lower and higher side RB extensions. For 15kHz SCS cases, 50MHz has NRB/2=135, thus the same number of bits. - Note that for the 100MHz case in 30kHz SCS and 50MHz in 15kHz SCS, the RB extension would correspond to BS bandwidth cases with DL CA or multiple channels (for example band n41 100+90MHz case in the US, or 100+60MHz case in China) - several unused codes can be used as “short-cut” signalling: for example, all 1s would mean that all allocations can be turned to inner. - Others - WF: FFS whether NW signalling need to consider different requirements or just a function enabler without considering specific requirements

Proposal 7: NW should signal UE extended CBW indication by signalling at least one-sided flexible RB extension (e.g., RB0shift or RBextL) and UE extended CBW size.

Based on NW indication, UE determines the other side RB extension and RB region of the allocation, and thus the enhanced MPR with the extended UE CBW.

In main room discussion of RAN4#113, it was agreed that both scenarios 1 and 2 should be covered.

#### Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation

Agreement in main session:

- Extended CBW based approach

  - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE

  - Both scenario 1 and 2 should be covered

  - The network should ensure the co-existence be met when signaling the extended CBW.

#### Issue 1-3-2: UE capability - Proposals - Proposal 1: - If the UE reports the capability supporting MPR reduction with extended CBW, e.g. [MPR improvement with BW extension-r19], the NW decides whether to allow such feature with specific signaling. - Proposal 2: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability (Sony) - Proposal 3: Define separate UE capabilities for scenario 1 and scenario 2 (Nokia) - Proposal 4: (Skyworks) - A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability may be considered: - One where the UE is able to reassign outer/edge to inner, based on 1RB granularity of signalled RB extension. - One where the UE is only able to turn all or none of its outer/edge RBs into inner (this may be a valid option for RedCap UEs). - Others - WF - MPR reduction with extended CBW is an optional feature with UE capability FFS details of UE capability

In our view scenario 1 is not universal scenario, instead it is local scenario. Furthermore implications from interference point of view are not yet well understood. But as it was agreed to also address scenario 1 thus we propose that different UE capabilities are assigned to these scenarios, this is because it may be turned out later that UE requirements will be very different and or there are other reasons why it is convenient to refer these scenarios separately and not commonly.

Proposal 8: Define separate UE capabilities for scenario 1 and scenario 2.

# 3	Conclusion

In this contribution we have made following proposal.

Proposal 1: In order to handle A-MPR for some channels, the following MPR shall be applied by UE: max(MPRenh(X), A-MPR(X)), where A-MPR(X) is determined based on a given RB allocation, and RB region with respect to initial CBW, WF and modulation etc, and MPRenh(X) is determined based on RB region/etc. within the extended UE CBW.

Proposal 2: IBE shall be applied in all non-allocated RBs, and guard bands for UE CBW and extended UE CBW that are non-overlapping with the guard band(s) for the at least one BS CBW (i.e., BS CBW or aggregated CBWs for the DL-only contiguous intra-band CA) to avoid interference to other UEs in the network.

Proposal 3: The value for IBE limit used for extended CBW at each side of UE CBW is derived from the outmost allocated RB of UE CBW similar to option 1, but while extending IBE requirements in the guard bands of UE CBW and extended CBW as well when they are non-overlapping with BS CBW as shown in Fig. 1a-1c.

Proposal 4: New UE inner RB region the light green colored area which is the intersection of Extended UE CBW inner region and allocation triangle of SIB1 UE CBW. New outer is the light yellow colored area which is New UE allocation triangle – new UE Inner RB region.

- New UE inner RB region is the intersection of Extended UE CBW inner region and allocation triangle of SIB1 UE CBW.

- New UE outer is New UE allocation triangle – new UE Inner RB region.

Proposal 5: In scenario 2 no limitations are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion.

Proposal 6: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band.

Proposal 7: NW should signal UE extended CBW indication by signalling at least one-sided flexible RB extension (e.g., RB0shift or RBextL) and UE extended CBW size.

Proposal 8: Define separate UE capabilities for scenario 1 and scenario 2.

# 4	References

[1] R4-2503013 WF

[2] R4-2418015,Redefining allocation types for improved MPR with BS BW larger than UE CBW	Skyworks Solutions