3GPP TSG-RAN WG4 Meeting #114bis										                  R4-2503225

Wuhan, China, 7th-11th Apr, 2025

Source:	Nokia, Samsung

Title:	On 1UL/1CC HPUE MSD rules

Agenda item:		7.1.1.1.1

Document for:	Approval

# 1			Introduction

This is a proposal on 1UL/1CC HPUE MSD simplifications rules for Rel-19.

This proposal is based on WF which was not approved in Athens [3].

# 2			Discussion

#### 2.1		General aspects

It is expected that CR for 1UL/1CC HPUE MSD topic is agreed in August RAN4#116 meeting when Rel-19 is completed.

Until that in RAN4 #115 and #116 basket WI work continues without any change.

If CR for 1UL/1CC HPUE MSD topic is approved in September RAN#109 and if an exception to extend the Rel19 HPUE Baskets WIs are allowed after RAN#109, then the Rel19 HPUE Basket WIs may be revised to reflect that the new HPUE MSD rules can be applied,

NR-U is not part of this WF.

When the CR for HPUE MSD is implemented (after RAN#109)

- All specified PC2 MSD tables and PC1.5 MSD tables remain untouched in TS 38.101-1 and TS38.101-3

- For 1UL/1CC FDD, 1UL/1CC TDD or 1UL/1CC SUL PC2, MSD is specified separately for PC2 1Tx and PC2 2Tx, see Table 1.

Handling of the Rel-19 baskets during the extension after CR for HPUE MSD is implemented (after RAN#109):

- The procedure is unchanged for the default power class (i.e.PC3): MSD must be studied, and, where applicable, the default PC3 MSD is captured in the TPs for TR

- The PC2/1.5 MSD requirements is derived from table 1 for the cases that PC3 MSD > 0 dB, when PC3 MSD = 0 then HPUE MSD is at maximum as in Table 2 but needs case by case study,

- If PC2/1.5 MSD does not fit the new MSD rules (which are supposed to be rare case) or HPUE MSD is defined for the case when PC3 MSD = 0,  the exceptional PC2/1.5 MSD will be captured into current PC2/1.5 MSD tables.

#### 2.2		Definition on 1UL/1CC CC PC2/1.5 LUT

For Rel-19 inter-band CA combinations within FR1 introduced after RAN#109, the 1UL/1CC CC HPUE MSD requirements can be specified using a delta MSD “PCx” relative to the default PC3 MSD as:

PCx MSD = PC3 MSD + PCx, where,

  - x = 2 for PC2 or x=1.5 for PC1.5,

  - PCx applies to MSD due to harmonic, harmonic mixing of UL1/DLy (y>1), [and cross band isolation,]

  - PC21Tx, PC22Tx and PC1.5 are specified with the LUT of Table 1.

Table 1: 1UL/1CC/1CC PC21Tx , PC22Tx , PC1.5 MSD LUT, PC3 MSD > 0.

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

Note:

- If PC3 MSD <=2.4dB, then then LUT PC3 entry = 2.0dB

- If PC3 MSD >=2.5dB, then then LUT PC3 entry = 3.0dB.

- In case the default PC3 MSD = 0 then HPUE MSD is at maximum as in Table 2 but needs case by case study

Table 2: 1UL/1CC/1CC PC21Tx , PC22Tx , PC1.5 MSD LUT, PC3 MSD = 0.

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| 0 | < 0.6 | < 0.9 | < 2.0 |

# 3	    Conclusion

In this contribution we have proposed how to simplify 1UL/1CC/1CC HPUE MSD for Rel-19 and beyond.

# 4			References

[1] 		R4-2501579 MSD beyond PC3, Qualcomm,

[2]		R4-2500218 Simplification of HPUE 2UL DeltaMSD, Skyworks

[3]		R4-2502864 WF on MSD rules, Skyworks, Nokia