# 3GPP TSG-RAN WG4 Meeting #114bis		R4-2503231

# Wuhan, Hubei, China, 07th – 11th April 2025

Agenda item:			7.11.3

Source:	Intelsat, Gilat, Tejas Networks, Kratos, Sharp, Gatehouse Satcom,

Title:	Ku Band: Further System Parameters incorporating 15 kHz Sub-carrier Spacing and lower Uplink A Band (Priority 2 Band Combination)

Document for:	Discussion and Decisions

# Introduction

Following the approval of the Ku Band WID at the RAN#107 [1] to include 15 kHz Sub-Carrier Spacing (SCS) in the FR1 numerology, we present a number of updates in this contribution.

Furthermore, we present Band Plan and Band numbering for Alternative 2 of downlink and uplink band combinations.

In this contribution we present the following:

- Definition of the ARFCN and GSCN parameters for FR1-NTN and FR2-NTN with 15kHz SCS.

- Definition of the ARFCN and GSCN parameters for FR1-NTN and FR2-NTN for the lower Uplink A Band.

# Progress to Date

To date we have agreed the following Priority 1 and Priority 2 Band Combinations, see following Figure 1. Within these band combinations, FR1-NTN and FR2-NTN Numerologies can be implemented, independently.

Figure 1 Ku Band Priority 1 and Priority 2 Uplink and Downlink Band Combinations

In [2] the FR1-NTN and FR2-NTN satellite operating bands were approved as shown in Table 1 and Table 2, respectively.

Table 1: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127501 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |

Table 2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |

Based on these agreed bands, the Absolute Radio Frequency Channel Number (ARFCN) and Global Synchronization Frequency Channel Number (GSCN) parameters were also agreed in [2] Afterwards, in [1] the 15kHz SCS was agreed for FR1-NTN band.

# Ku Band Numerology FR1-NTN

In the following section: the Band Number; Channel Bandwidth; ARFCN and Channel Raster; and GSCN and Synchronization Raster are defined for FR1-NTN Ku Band numerology.

## Band Number.

Table 3 illustrates the satellite operating Band Numbers for FR1-NTN Ku Band numerology.  Three unique satellite operating Band Numbers are defined, namely: n248, n247, and n246.  The first two satellite operating bands are identical as what was agreed in [2].  The satellite operating band n246 has uplink between 12.7 to 13.25 GHz but the same downlink band as n248 and n247.  For ITU Region 2, the downlink band stops at 12700 MHz, and the uplink band A starts at 12700 MHz.  For ITU Region 1 and 3 the downlink band stops at 12750 MHz and the uplink band A starts at 12750 MHz.  To minimize the number of unique satellite operating bands, the superset frequency range are chosen for the Ku-band downlink and Ku-band uplink with footnotes provided to limit the frequency range based on ITU Region.  Subsequently, these frequency ranges are used to define the ARFCN and GSCN. However, the ground terminal needs intelligence to determine the actual ARFCN and GSCN range based on its ITU Region location.

Please note that in the following Table 3, satellite operating band n246 refers to the Priority 2 band combination.

Table 3: Band definitions for Ku Band using FR1-NTN numerology.

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127501,2 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127501,2 MHz | FDD |
| n246 | 127003 MHz – 13250 MHz | 10700 MHz – 127501,2 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. NOTE 2: For ITU Region 2, the downlink band stops at 12700 MHz, and the uplink band starts at 12700 MHz. NOTE 3: For ITU Region 1 and 3 the downlink band stops at 12750 MHz and the uplink band starts at 12750 MHz. |  |  |  |

Proposal 1: Select satellite operating band n246 for FR1-NTN Priority 2 band combination.

## Channel Bandwidth.

Table 4 illustrates the Channel Bandwidth for FR1-NTN Ku Band numerology.  Both the 15 kHz and 30 kHz SCS are defined for FR1-NTN Ku Band.

Please note that in the following Table 4, satellite operating band n246 refers to Priority 2 band combination.

Table 4: FR1-NTN Channel Bandwidth.

| FR1-NTN band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite operating band | SCS kHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| n248 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |
|  | 30 | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
| n247 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |
|  | 30 | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |
| n246 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |
|  | 30 | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |

Observation 1: FR1-NTN Channel Bandwidth are 10, 20, 25, 35, 50, 70, and 100 MHz with 30kHz SCS, and 10, 15, 25, 35, and 50 MHz with 15kHz SCS.

## ARFCN and Channel Raster

The Absolute Radio Frequency Channel Number (ARFCN) defines a set of RF reference frequencies used in signalling to identify the position of RF channels on the downlink and uplink frequencies.  For Ku Band, the granularity of the global frequency raster (DFGlobal) is 15kHz.  The Channel Raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink.  For 15kHz SCS, the Channel Raster (DFRaster) is defined as follows: DFRaster = 1 x DFGlobal. In this case, every AFRCN within the Ku Band are applicable for the Channel Raster and the step size of the Channel Raster is 1. Similarly, for 30kHz SCS, the Channel Raster (DFRaster) is defined as follows: DFRaster = 2 x DFGlobal. In this case, every 2nd AFRCN within the Ku Band are applicable for the Channel Raster and the step size of the Channel Raster is 2.

Table 5 illustrates the FR1-NTN ARFCN per Ku operating band.

Please note that in the following Table 5, satellite operating band n246 refers to Priority 2 band combination.

Table 5: FR1-NTN ARFCN per Ku operating band.

| Satellite operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Downlink range of NREF (First - <Step size>- Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366667 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316668 – <1> – 1333333 | 1113334 – <1> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |
| n246 | 15 | 1246668 – <1> – 1283334 | 1113334 – <1> – 1250000 |
|  | 30 | 1246668 – <2> – 1283334 | 1113334 – <2> – 1250000 |

Proposal 2: We propose the FR1-NTN ARFCN entries per Ku operating band with Channel Raster of 15 kHz and 30 kHz as illustrated in Table 5.

## GSCN and Synchronization Raster

The GSCN indicates the frequency position of the Synchronization Signal Block (SSB) that can be used by the UE on the downlink for system acquisition. Table 6 illustrates the FR1-NTN GSCN entries for the different satellite operating bands. The SSB block pattern for FR1-NTN Ku-band are Case A for 15 kHz and Case C for 30 kHz.

Please note that in the following Table 6, satellite operating band n246 refers to Priority 2 band combination.

Table 6: FR1-NTN Applicable GSCN entries per operating band.

| Satellite operating band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n246 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |

Proposal 3 : We propose the FR1-NTN GSCN entries per Ku operating band with step size of 1 as illustrated in Table 6.

# Ku Band Numerology FR2-NTN

In the following section: the Band Number; Channel Bandwidth ; ARFCN and Channel Raster; and GSCN and Synchronization Raster are defined for FR2-NTN Ku Band numerology.

## Band Number.

Table 7 illustrates the satellite operating Band Numbers for FR2-NTN Ku Band numerology. Three unique satellite operating Band Numbers are defined, namely: n509, n508, and n507. The first two band are identical as what was agreed in [2].  The satellite operating band n507 has uplink between 12.7 to 13.25 GHz but the same downlink band as n509 and n508.  Like FR1-NTN, to minimize the number of unique satellite operating bands, the superset frequency range are chosen for the Ku-band downlink and Ku-band uplink with footnotes provided to limit the frequency range based on ITU Region.

Please note that in the following Table 7, satellite operating band n507 refers to Priority 2 band combination.

Table 7: Band definitions for Ku Band using FR2-NTN numerology.

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127501,2 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127501,2 MHz | FDD |
| n507 | 127003 MHz – 13250 MHz | 10700 MHz – 127501,2 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. NOTE 2: For ITU Region 2, the downlink band stops at 12700 MHz, and the uplink band starts at 12700 MHz. NOTE 3: For ITU Region 1 and 3 the downlink band stops at 12750 MHz and the uplink band starts at 12750 MHz. |  |  |  |

Proposal 4: Select satellite operating band n507 for FR2-NTN Priority 2 band combination.

## Channel Bandwidth.

Table 8 illustrates the Channel Bandwidth for FR2-NTN Ku Band numerology.  The 120 kHz SCS is currently the only subcarrier spacing defined for FR2-NTN Ku Band.

Please note that in the following Table 8, satellite operating band n507 refers to Priority 2 band combination.

Table 8: FR2-NTN Channel Bandwidth.

| FR2-NTN band / SCS / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite operating band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n509 | 120 | Yes | Yes | Yes | Yes |
| n508 | 120 | Yes | Yes | Yes | Yes |
| n507 | 120 | Yes | Yes | Yes | Yes |

Observation 2: FR2-NTN Channel Bandwidth are 50, 100, 200 and 400 MHz with 120kHz SCS.

## ARFCN and Channel Raster

The ARFCN defines a set of RF reference frequencies used in signalling to identify the position of RF channels on the downlink and uplink frequencies. For Ku Band, the granularity of the global frequency raster (DFGlobal) is 15kHz. For 120 kHz SCS, the Channel Raster (DFRaster) is defined as follows: DFRaster = 8 x DFGlobal. In this case, every 8th  AFRCN within the Ku Band are applicable for the Channel Raster and the step size of the Channel Raster is 8.

Table 9 illustrates the FR2-NTN ARFCN per Ku operating band.

Please note that in the following Table 9, satellite operating band n507 refers to Priority 2 band combination.

Table 9: FR2-NTN ARFCN per downlink Ku operating band.

| Satellite operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Downlink range of NREF (First - <Step size>- Last) |
| --- | --- | --- | --- |
| n509 | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1250000 |
| n508 | 120 | 1316672 – <8> – 1333328 | 1113336 – <8> – 1250000 |
| n507 | 120 | 1246672 – <8> – 1283328 | 1113336 – <8> – 1250000 |

Proposal 5: We  propose the FR2-NTN ARFCN entries per Ku operating band with Channel Raster of 120 kHz and step size of 8 as illustrated in Table 9.

## GSCN and Synchronization Raster

The GSCN indicates the frequency position of the Synchronization Signal Block (SSB) that can be used by the UE on the downlink for system acquisition.  Table 10 illustrates the FR2-NTN GSCN entries for the different Ku Band numbers.  The SSB block pattern for FR2-NTN Ku-band is Case D.

Please note that in the following Table 10, satellite operating band n507 refers to Priority 2 band combination.

Table 10: FR2-NTN Applicable SS raster entries per operating band.

| Satellite operating band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 kHz | Case D | 12864 – <12> – 14256 |
| n508 | 120 kHz | Case D | 12864 – <12> – 14256 |
| n507 | 120 kHz | Case D | 12864 – <12> – 14256 |

Proposal 6: We propose the FR2-NTN GSCN entries per Ku operating band with step size of 12 as illustrated in Table 10.

# Conclusion and Proposals

In this contribution, we have defined for FR1-NTN and FR2-NTN Ku Band numerology, the Band Number; Channel Bandwidth ; Absolute Radio Frequency Channel Number (ARFCN) and Channel Raster; and Global Synchronization Frequency Channel Number (GSCN) and synchronization raster.

From our detailed calculations we have the following observations:

Observation 1: FR1-NTN Channel Bandwidth are 10, 20, 25, 35, 50, 70, and 100 MHz with 30kHz SCS, and 10, 15, 25, 35, and 50 MHz with 15kHz SCS.

Observation 2: FR2-NTN Channel Bandwidth are 50, 100, 200 and 400 MHz with 120kHz SCS.

The proposals that can be drawn from our calculations are as follows:

Proposal 1: Select satellite operating band n246 for FR1-NTN Priority 2 band combination.

Proposal 2: We propose the FR1-NTN ARFCN entries per Ku operating band with Channel Raster of 15 kHz and 30 kHz as illustrated in Table 5.

| Satellite operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Downlink range of NREF (First - <Step size>- Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366667 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316668 – <1> – 1333333 | 1113334 – <1> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |
| n246 | 15 | 1246668 – <1> – 1283334 | 1113334 – <1> – 1250000 |
|  | 30 | 1246668 – <2> – 1283334 | 1113334 – <2> – 1250000 |

Proposal 3: We propose the FR1-NTN GSCN entries per Ku operating band with step size of 1 as illustrated in Table 6.

| Satellite operating band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n246 | 15 kHz | Case A | 12848 – <1> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |

Proposal 4: Select satellite operating band n507 for FR2-NTN Priority 2 band combination.

Proposal 5: We  propose the FR2-NTN ARFCN entries per Ku operating band with Channel Raster of 120 kHz and step size of 8 as illustrated in Table 9.

| Satellite operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Downlink range of NREF (First - <Step size>- Last) |
| --- | --- | --- | --- |
| n509 | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1250000 |
| n508 | 120 | 1316672 – <8> – 1333328 | 1113336 – <8> – 1250000 |
| n507 | 120 | 1246672 – <8> – 1283328 | 1113336 – <8> – 1250000 |

Proposal 6: We  propose the FR2-NTN GSCN entries per Ku operating band with step size of 12 as illustrated in Table 10.

| Satellite operating band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 kHz | Case D | 12864 – <12> – 14256 |
| n508 | 120 kHz | Case D | 12864 – <12> – 14256 |
| n507 | 120 kHz | Case D | 12864 – <12> – 14256 |

# References

[1] “Revised WID: Introduction of Ku Bands for NR NTN”, RP-250799, 3GPP TSG RAN meeting #107, Incheon, Korea, March 12th -14th, 2025.

[2]  “Way Forward for [114][313] NR_NTN_Ku_Band_General”, 3GPP TSG-RAN WG4 Meeting #114, draft R4-2502290