# 3GPP TSG-RAN WG4 Meeting #114-bis                                                          R4-2503266

Wuhan, China, 7-11 April, 2025

Title: 	REFSENS for NTN bands supporting 3MHz CBW

Source: 	Huawei, HiSilicon

Agenda item:	7.10.3.2

Document for:	Discussion

# Introduction

RAN4 113 meeting the WF[1] was agreed, and the REFSENS requirements are remained as FFS in this document, those open issues will be addressed.

# Discussion

Issue 2-2-1: Reference sensitivity - Agreement - For band n254, [2.2] dB improvement compared with 5 MHz CBW; - For band n256, [1.7] dB improvement compared with 5 MHz CBW; - FFS for band n255

Among these NTN bands, n256 has the lowest UL/DL separation frequency (60 MHz), which could cause degradation on REFSENS level due to IMD2, if the duplexer and the RF filter do not have sufficient isolation. We performed REFSENS evaluations with a simple architecture that is shown in Figure 1.

Figure 1 n256 Tx/Rx architecture

In the upper part of Table 1 the summary of the parameters that are used for the evaluation is given. It should be mentioned that the isolations data of the duplexer and the filter are obtained from commercially available datasheets. On the lower part of the Table 1 the evaluations show that for an SNR of -1dB and even with an implementation margin of 2.5dB, a REFSENS of -101.73dB is feasible. In other words, a 2.2 dB improvement compared to 5 MHz CBW is achievable.

Proposal 1: For n256, a 2.2 dB REFSENS improvement compared to 5 MHz CBW is achievable, based on our evaluation.

Proposal 2: Among {n254, n255, n256}, n256 has the lowest UL/DL separation frequency (60 MHz). Therefore, 2.2 dB REFSENS improvement for 3MHz CBW compared to 5 MHz CBW is achievable for n254, n255.

Table 1 n256 3MHz REFSENS evaluation

# Conclusions

Proposal 1: For n256, a 2.2 dB REFSENS improvement compared to 5 MHz CBW is achievable, based on our evaluation.

Proposal 2: Among {n254, n255, n256}, n256 has the lowest UL/DL separation frequency (60 MHz). Therefore, 2.2 dB REFSENS improvement for 3MHz CBW compared to 5 MHz CBW is achievable for n254, n255.

# Reference

[1]	“R4-2502282, WF for [114][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi.”