# 3GPP TSG-RAN WG4 Meeting #114-bis                                                          R4-2503277

Wuhan, China, 7-11 April, 2025

Title: 	Remaining discussions on UE capability for type 4 UEs

Source: 	Huawei, HiSilicon

Agenda item:	7.4.1.2

Document for:	Discussion

# Introduction

RAN4 114 meeting the WF [1] was agreed, in which some open points are left without a conclusion. In this document we are going to address them.

# Discussion

UE switching among Type 1, Type 2 and Type 4a/4b for NR CA:

To allow a full exploitation of the network, similar to Rel-18, there should be a mechanism to allow a UE type switch between type 1, 2 and 4.

In Rel-18 specifications the following is captured:

38.101-1 i8 section 7.10A.2: For a UE capable of intraBandNR-CA-non-collocated-r18 for the following CA band combinations in Table 7.10A.2-2, the Power imbalance requirements are applicable with 2Rx antenna ports for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

The text can be reformulated as follows:

when maxMIMO-Layers=2 :

·         If UE reports intraBandNR-CA-non-collocated-r18 and network does NOT issue nonCollocatedTypeNR-CA-r18 è  type 2 (default type)

·         If UE reports intraBandNR-CA-non-collocated-r18 and network issues nonCollocatedTypeNR-CA-r18 è   type 1

when maxMIMO-Layers=4 : è type 1 (default type)

On the other hand, for Rel-19, in WF[2] it was agreed to consider, the same approach as Rel-18 when maxMIMO-Layers=2. There are two other scenarios in Rel-19, too. When maxMIMO-Layers = 4 and when maxMIMO-Layers= 8. Regarding the case where maxMIMO-Layers= 8, there is no ambiguity as type 1 is the only type that can support up to 8 layers/CC, hence and it can be used by the network to indicate that type 1 UE should be used. However, when maxMIMO-Layers = 4 there could be some ambiguities, since both type 1 and 4 can support 4 layers per CC. To respect the same approach as Rel-18 we can have a UE type switch based a present or absent network IE, as follows:

when maxMIMO-Layers=4:

·         If UE reports [intraBandNR-CA-xxxnon-collocated-r19] and network does NOT issue [nonCollocatedTypexxxNR-CA-r19] è  type 4 (default type)

·         If UE reports [intraBandNR-CA-xxxnon-collocated-r19] and network issues  [nonCollocatedTypexxxNR-CA-r19] è   type 1

It should be mentioned that using this mechanism when a Rel-19 UE operates with a Rel-18 BS, it does not receive the Rel-19 [nonCollocatedTypexxxNR-CA-r19] and it operates as a type 4 device. There won’t be any performance degradation as type 4 UE has better power imbalance and MTTD/MRTD performance than type 1.

Proposal 1 : For intra-band NR CA, when networks issues maxMIMO-Layers= 8, the UE is configured as type 1.

Proposal 2 : For intra-band NR CA, to follow the same logic as Rel-18, the Rel-19 signalling can be addressed as :

when maxMIMO-Layers=4:

·         If UE reports [intraBandNR-CA-xxxnon-collocated-r19] and network does NOT issue [nonCollocatedTypexxxNR-CA-r19] è  type 4 (default type)

·         If UE reports [intraBandNR-CA-xxxnon-collocated-r19] and network issues [nonCollocatedTypexxxNR-CA-r19] è   type 1

Observation 1: When a Rel-19 UE operates with a Rel-18 BS, it does not receive the Rel-19 [nonCollocatedTypexxxNR-CA-r19] and it operates as a type 4 device. There won’t be any performance degradation as type 4 UE as better power imbalance and MTTD/MRTD performance than type 1.

Another approach can be the bit wise approach to indicate in the RRC message via 1 bit whether the UE should be configured with type 1 or 4. This approach is not the one that was used in Rel-18, moreover if the network does not include the bit in the RRC message (which is possible since the feature is optional) then we should specify a default behaviour for the UE in the absence of the network IE, and take the Rel-18 approach.

Observation 2: The bitwise approach (0: type 1, 1: type 4) is different than Rel-18 approach. Moreover if the network does not include the bit in the RRC message (which is possible since the feature is optional), then we should specify a default behaviour for the UE in the absence of the network IE, which means falling back to Rel-18 approach.

UE switching among Type 1, Type 2 and Type 4a/4b for EN-DC:

To allow a full exploitation of the network, similar to Rel-18, there should be a mechanism to allow a UE type switch between type 1, 2 and 4.

In Rel-18 specifications the following is captured:

38.101-3 i8 section 7.10B.3: If the UE indicates interBandMRDC-WithOverlapDL-Bands-r16 but does not indicate requirementTypeIndication-r18 or a UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and IE nonCollocatedTypeMRDC-r18 is not provided when maxMIMO-Layers with value less than or equal to 2, the Rx requirements for two Rx ports are applicable for each band in EN-DC operating mode for the following EN-DC band combinations in Table 7.10B.3-2. Table 7.10B.3-2: TDD-TDD EN-DC combinations EN-DC combination. DC_42_n771. DC_42_n781. NOTE 1: Note 9 in table 5.5B.4.1-1 is applicable..

The text can be reformulated as follows:

·         If UE reports interBandMRDC-WithOverlapDL-Bands-r16 but does not indicate requirementTypeIndication-r18 è  type 2 (default type) (to ensure the Rel-16 UE compatibility)

· when maxMIMO-Layers=2 :

·     If UE reports interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 but the network does NOT issue nonCollocatedTypeMRDC-r18 è  type 2 (default type)

·         If UE reports interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and the network issues nonCollocatedTypeMRDC-r18 è   type 1

· when maxMIMO-Layers=4 : è type 1 (default type)

using similar arguments as the NR CA section, one can deduce the following proposals:

Proposal 3: For inter-band EN-DC, when networks issues maxMIMO-Layers= 6 or 8, the UE is configured as type 1.

Proposal 4: For inter-band EN-DC, to follow the same logic as Rel-18, the Rel-19 signalling can be addressed as:

when maxMIMO-Layers=4:

·         If UE reports [interBandMRDC-xxxxWithOverlapDL-Bands-r19] and network does NOT issue [nonCollocatedTypexxxxMRDC-r19] è  type 4 (default type)

·         If UE reports [interBandMRDC-xxxxWithOverlapDL-Bands-r19] and network issues [nonCollocatedTypexxxxMRDC-r19] è  type 1

In general, it is better to leave the details of the signaling design to RAN2 and send an LS to RAN2 about it.

Proposal 5 : Send an LS to RAN2, with RAN4 requirements about the signallings.

# Conclusions

Proposal 1 : For intra-band NR CA, when networks issues maxMIMO-Layers= 8, the UE is configured as type 1.

Proposal 2 : For intra-band NR CA, to follow the same logic as Rel-18, the Rel-19 signalling can be addressed as :

when maxMIMO-Layers=4:

·         If UE reports [intraBandNR-CA-xxxnon-collocated-r19] and network does NOT issue [nonCollocatedTypexxxNR-CA-r19] è  type 4 (default type)

·         If UE reports [intraBandNR-CA-xxxnon-collocated-r19] and network issues [nonCollocatedTypexxxNR-CA-r19] è   type 1

Observation 1: When a Rel-19 UE operates with a Rel-18 BS, it does not receive the Rel-19 [nonCollocatedTypexxxNR-CA-r19] and it operates as a type 4 device. There won’t be any performance degradation as type 4 UE as better power imbalance and MTTD/MRTD performance than type 1.

Observation 2: The bitwise approach (0: type 1, 1: type 4) is different approach than Rel-18 approach, moreover if the network does not include the bit in the RRC message (which is possible since the feature is optional) then we should specify a default behavior for the UE in the absence of the network IE, which means falling back to Rel-18 approach.

Proposal 3: For inter-band EN-DC, when networks issues maxMIMO-Layers= 6 or 8, the UE is configured as type 1.

Proposal 4: For inter-band EN-DC, to follow the same logic as Rel-18, the Rel-19 signalling can be addressed as:

when maxMIMO-Layers=4:

·         If UE reports [interBandMRDC-xxxxWithOverlapDL-Bands-r19] and network does NOT issue [nonCollocatedTypexxxxMRDC-r19] è  type 4 (default type)

·         If UE reports [interBandMRDC-xxxxWithOverlapDL-Bands-r19] and network issues [nonCollocatedTypexxxxMRDC-r19] è  type 1

Proposal 5 : Send an LS to RAN2, with RAN4 requirements about the signallings.

# Reference

[1]	“R4-2502924, WF on UE requirements for non-collocated scenario, KDDI.”

[2]	“R4-2420399, WF on non-collocated intra-band EN-DC and NR-CA, KDDI.”