# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503301

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Xiaomi

Title: 	Discussion on simulation assumptions and results for LP-WUS/WUR

Agenda Item:	7.23.4.1

Document for:	Discussion

# 1Introduction

In RAN4#114 meeting, the set of LP-SS binary sequences for RAN4 simulation alignment purpose were agree in [1].

In this contribution, our link level simulation results are provided.

- Discussion

- Simulation assumptions

The simulation assumptions are given in Table 1 and 2:

Table 1: General parameters

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2.6 GHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| LP-SS block BW | 132 subcarriers for SCS=30kHz |
| BS transmit antennas for LP-SS blocks | 1 tx |
| UE receive antennas | 1 rx |
| Subcarrier spacing | 30 kHz |
| Number of LP-SS blocks per SS burst set, K | 1 |
| SINR | -3dB |
| Propagation conditions | TDL-C 300ns |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| LP-SS (OOK based LP-WUR receiver) |  | OOK-4 with M = 1,2,4 | when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1. |
| LP-SS pattern |  | The agreed LP-SS binary sequence in RAN4#114 | The agreed LP-SS binary sequence in RAN4#114 |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | 10 ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: T= 5us for M=1, L=6 T= 2us for M=2, L=8 and M=2, L=16 T= 1us for M=4, L=16 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | Set 1: SNR = -2.7 Set 2: SNR = -2.4 | Set 1: SNR = -11.7 Set 2: SNR = -8.4 |
| Ês/Iot | dB | -3 | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | FR1: 3 km/h |  |

- Simulation results

- Based on the above captured assumption, the simulation results are summarized below:

Table 3: simulation results

| {M,L} | LP-SS Binary Sequence | RSRP Measurement Accuracy [dB] (max(5%,95%))vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | channel | Interference Source | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X = 2 |  | X = 3 |  | X = 4 |  | X = 5 |  |  |  |  |  |
|  |  | mean | std | mean | std | mean | std | mean | std |  |  |  |  |
| {1, 6} | Serving cell:101010 Interference Cell:010101 |  | 2.73 |  | 2.09 |  | 1.93 |  | 1.84 | -3 | AWGN | LP-SS sync | Set1 |
| {2, 8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.03 |  | 0.90 |  | 0.82 |  | 0.77 | -3 | AWGN | LP-SS sync | Set1 |
| {2, 16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.76 |  | 0.61 |  | 0.53 |  | 0.48 | -3 | AWGN | LP-SS sync | Set1 |
| {4, 16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 1.12 |  | 0.94 |  | 0.86 |  | 0.76 | -3 | AWGN | LP-SS sync | Set1 |
| {1, 6} | Serving cell:101010 Interference Cell:010101 |  | 3.38 |  | 3.03 |  | 2.81 |  | 2.71 | -3 | TDL-C | LP-SS sync | Set1 |
| {2, 8} | Serving cell:01011001 Interference Cell:01100101 |  | 2.19 |  | 1.86 |  | 1.69 |  | 1.53 | -3 | TDL-C | LP-SS sync | Set1 |
| {2, 16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 1.9 |  | 1.76 |  | 1.52 |  | 1.38 | -3 | TDL-C | LP-SS sync | Set1 |
| {4, 16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 2.62 |  | 2.21 |  | 2.03 |  | 2.00 | -3 | TDL-C | LP-SS sync | Set1 |
| {1, 6} | Serving cell:101010 Interference Cell:010101 |  | 2.67 |  | 2.41 |  | 2.27 |  | 2.17 | -3 | AWGN | LP-SS sync | Set2 |
| {2, 8} | Serving cell:01011001 Interference Cell:01100101 |  | 1.24 |  | 1.12 |  | 1.05 |  | 0.99 | -3 | AWGN | LP-SS sync | Set2 |
| {2, 16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 0.66 |  | 0.56 |  | 0.51 |  | 0.47 | -3 | AWGN | LP-SS sync | Set2 |
| {4, 16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 0.84 |  | 0.66 |  | 0.58 |  | 0.50 | -3 | AWGN | LP-SS sync | Set2 |
| {1, 6} | Serving cell:101010 Interference Cell:010101 |  | 3.46 |  | 3.14 |  | 2.70 |  | 2.6 | -3 | TDL-C | LP-SS sync | Set2 |
| {2, 8} | Serving cell:01011001 Interference Cell:01100101 |  | 2.26 |  | 2.00 |  | 1.77 |  | 1.61 | -3 | TDL-C | LP-SS sync | Set2 |
| {2, 16} | Serving cell:1001010110011001 Interference Cell:1001100101100101 |  | 1.55 |  | 1.47 |  | 1.26 |  | 1.19 | -3 | TDL-C | LP-SS sync | Set2 |
| {4, 16} | Serving cell:0110100110101010 Interference Cell:0110101010011010 |  | 2.15 |  | 2.02 |  | 1.68 |  | 1.56 | -3 | TDL-C | LP-SS sync | Set2 |

In previous meeting, RAN4 achieved the following agreement on accuracy to derive the number of samples.

Issue 2-1-4: Accuracy baseline for simulation Agreement: Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only: - For FR1: - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session - Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

The simulation results summarized above do not utilize the overlaid sequence on LP-SS. Also, the the ideal RSRP used in the simulation is derived from the serving cell signal after fading channel. Based on stimulation results, we could make following observations:

- To achieve 3.5 dB absolute baseband accuracy, measurement with at least 2 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

- To achieve 2.5 dB absolute baseband accuracy, measurement with 5 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

Observation 1: To achieve 3.5 dB absolute baseband accuracy, measurement with 2 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

Observation 2: To achieve 2.5 dB absolute baseband accuracy, measurement with more than 5 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

Proposal 1: RAN4 to use 3 samples for LR measurement requirements with the target accuracy of ±3.5 dB.

- Conclusion

Observation 1: To achieve 3.5 dB absolute baseband accuracy, measurement with 2 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

Observation 2: To achieve 2.5 dB absolute baseband accuracy, measurement with more than 5 LP-SS samples is needed at SNR = -3dB in TDL-C channel.

Proposal 1: RAN4 to use 3 samples for LR measurement requirements with the target accuracy of ±3.5 dB.

- Reference

- R4-2502600, WF for RRM core requirements for LP-WUS/WUR, Vivo