3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503310

Wuhan, China, April 7th – 11th, 2025

Source: 	Xiaomi

Title: 	Discussion on event triggered L1 measurement reporting for mobility

Agenda Item:	7.27.2.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Event-triggered L1 reporting

- Discussion

Reporting capability

- Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///) - Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia) - Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK) - Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

There are some discussions about the UE capability of supported number of events. Generally, we are fine to define capability for supported number of events for L1 based reporting for LTM. Since CSI-RS based LTM is also considered in L1 event triggered reporting, the relation between L3 reporting and L1 reporting, and relation between CSI-RS based L1 measurement/reporting and SSB based L1 measurement/reporting will have impact on the capability definition. it’s also FFS how to count frequency layer. Therefore, we prefer to discuss the detail of UE capability for in later stage when design is clearer.

Proposal 1: RAN4 to define UE capability for Event Triggering and Reporting Criteria for LTM.

Measurement delay

- Issue 2-2: Event triggered reporting delay for L1 reporting Agreement: - Except for periodic MR: - Option 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - Option 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - The measurement period on the LTM candidate cells is based on the Rel-18 requirement, with the assumption of Treport is 0. - For periodic MR: - Option 1: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR. - Option 2: For the first report after condition is met, the event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period corresponding to the Cell(s) which triggers the event on the cells corresponding to the event. - RAN4 define delay requirement for the first report after condition is met. After that, UE follow the network configuration on the reporting interval. FFS how to capture this agreement into the CR.

If the measurement periods for different beams are the same, the legacy requirement can be re-used. However, if the RS periodicity is different, we prefer that the maximum measurement period between current beam and candidate beam will be used as measurement delay.

Proposal 2: If multiple beams with different measurement period need to be measured, the maximum measurement period will be considered as measurement delay.

L1 result in report

In legacy L3 event triggered reporting, according to 38.331, if L3 measurement result satisfies the event triggering condition, UE will initiate the message reporting procedure and submit the MeasurementReport message to lower layers for transmission. It means that the L3 measurement result which triggers reporting will be included in the L3 report.

For L1 event triggered reporting, different from L3, L1 measurement is much faster, it’s possible that UE can obtain several new L1 results between the time UE trigger L1 reporting and the time when UL resource arrives. Since this is L1 report, the procedure is better specified in physical layer. We prefer to clarify this issue in RAN4.

Option 1: UE ignores new L1 result after L1 report is triggered

For this case, UE can obtain new L1 result (RS3) after L1 reporting is triggered. For example, there are two sub-cases:

Case 1: L1-RSRP at RS after L1 reporting still maintain the L1 reporting decision

Case 2: L1-RSRP at RS after L1 reporting will change the L1 reporting decision

For both cases, UE will ignore the L1-RSRP of RS(e.g. RS3) which is after L1 event triggered no matter whether it will change the triggering decision or not. The L1-RSRP measurement result(RS2) which triggers reporting will be included in the L1 report.

Fig.1 UE disregards subsequent L1 measurements after event triggering

Option 2: UE consider new L1 result after L1 report is triggered

Fig.2 UE continuously monitors measurements until UL transmission

For this case, UE will consider the impact of new L1 result after L1 triggering, the L1-RSRP result(RS3) which is closet to first UL will be put in the report.

RAN4 should mandate that L1 event-triggered reports must contain the exact L1-RSRP measurement instance (RS2) that initiated the reporting procedure. This ensures:

- Deterministic correspondence between triggering condition and reported value

- Elimination of temporal ambiguity in network decision-making

- Backward compatibility with existing event evaluation mechanisms

- Simplified UE implementation by avoiding post-trigger measurement arbitration

Proposal 3: The L1-RSRP measurement that initially satisfied triggering criteria shall be included in the report, regardless of post-trigger measurement variations.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: RAN4 to define UE capability for Event Triggering and Reporting Criteria for LTM.

Proposal 2: If multiple beams with different measurement period need to be measured, the maximum measurement period will be considered as measurement delay.

Proposal 3: The L1-RSRP measurement that initially satisfied triggering criteria shall be included in the report, regardless of post-trigger measurement variations.

- Reference