3GPP TSG-RAN WG4 Meeting # 114bis	 R4-2503311

Wuhan, China, April 7th – 11th, 2025

Source: 	Xiaomi

Title: 	Discussion on CSI-RS based L1 measurement for mobility

Agenda Item:	7.27.2.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Known cell condition - Definition of intra-frequency and inter-frequency - Supported measurement types - Pre-requisite of CSI-RS based L1 measurement

- Discussion

2.1 Known cell condition

Issue 3-1-4: Side condition for CSI-RS based L1 measurement for neighbor cell <Candidate solutions> - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (MTK) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Ericsson, Nokia, Saumsung)

We suggest to apply the same side condition for both SSB and CSI-RS based L1 measurement for neighbour cell.

Proposal 1: Use legacy side condition SINR = -3dB for CSI-RS based L1 measurement for neighbour cell.

2.2 Definition of intra-frequency and inter-frequency

In last meeting, there is some discussion about how to define intra-frequency and inter-frequency measurement for CSI-RS L1-RSRP.

Issue 4-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Generally, we are fine with both options. We understand the intention of option 2. However, we prefer the unified framework as legacy. The detail requirement will anyway be defined in each scenarios, the result is similar.

Proposal 2: Prefer to define intra – frequency by unified framework as legacy, support option 1.

2.3 Gap and RTD assumption

- Issue 4-7: RTD assumption Candidate options: - Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details - Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. - Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details - Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP.

For measurement with gap, SSB based L1 measurement can be applied. Considering the complexity of performing CSI-RS based measurement, we suggest to prioritize CSI-RS based L1-RSRP measurement inside active BWP.

For RTD >CP, it will require multiple FFTs. Since CSI-RS bandwidth may be much larger than SSB, therefore it requires multiple large FFTs. We also prefer not to consider this scenario.

Proposal 3: Not support L1 measurement with gap and RTD>CP for CSI-RS based L1-RSRP measurement.

2.4 Pre-requisite of CSI-RS based L1 measurement

Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

The decision to skip or retain Step 2 (SSB-based L1 measurements) in the measurement procedure for CSI-RS-based LTM candidate cell measurements depends on a trade-off between potential benefits and drawbacks.

When SSB based L1 measurement is performed before step 3:

- Possible Benefit:

  - Reduce UE implementation complexity by skipping RX beam sweeping in step 3.

  - Reduce CSI-RS resource number configured for measurement.

- Drawback:

  - Extended L1 measurement time for neighbour cell. Since SSB sharing between different cells may be needed. Periodicity of SSB may be longer than CSI-RS resource.

  - Limit the NW configuration flexibility.

Since there are both benefits and drawback to keep step 2. Therefore, we support option 2, it’s up to UE capability reporting.

Proposal 4: Whether step 2 can be skipped or not can be based on UE capability reporting.

2.5 Definition of inside/outside active BWP

- Issue 4-5: Definition of “inside/outside active BWP” Candidate options: - Option 1: (Apple) The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP. - Option 2: (CATT) RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies. It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’. - Option 3: (vivo) From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

In legacy intra-f CSI-RS based L3 measurement, the definition is as below:

#### 9.10.2.2 Requirements applicability The measurement of the associated SSB follows the same requirements as SSB based measurements defined in clause 9.2. The requirements in clause 9.10.2 apply, provided: - Only one intra-frequency CSI-RS layer per serving cell is configured, and - The BW of the CSI-RS on the intra-frequency neighbor cell is within the active BWP of the UE, and

The similar definition can be re-used, i.e. the BW of the CSI-RS is within active BWP of the UE.

Proposal 5: The CSI-RS of candidate cell should be considered inside the active BWP if the BW of the CSI-RS is within active BWP of the UE.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: Use legacy side condition SINR = -3dB for CSI-RS based L1 measurement for neighbour cell.

Proposal 2: Prefer to define intra – frequency by unified framework as legacy, support option 1.

Proposal 3: Not support L1 measurement with gap and RTD>CP for CSI-RS based L1-RSRP measurement.

Proposal 4: Whether step 2 can be skipped or not can be based on UE capability reporting.

Proposal 5: The CSI-RS of candidate cell should be considered inside the active BWP if the BW of the CSI-RS is within active BWP of the UE.

- Reference