3GPP TSG-RAN WG4 Meeting # 114bis	R4-2503312

Wuhan, China, April 7th – 11th, 2025

Source: 	Xiaomi

Title: 	Discussion on UE-initiated/event-driven beam management on Rel-19 MIMO phase 5

Agenda Item:	7.22.3.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the RRM requirement of UE-initiated/event-driven beam management for Rel-19 MIMO phase 5.

- Discussion

2.1 Reporting delay

Issue 1-4: Components of Event triggered measurement reporting delay? <Way forward > - Option 1: (Apple, vivo, Samsung, E///, MTK) - The event triggered measurement reporting delay is TL1-RSRP_measure_period + Tfirst UL channel - FFS on whether other components are needed. - FFS the definition of Tfirst UL channel - Option 2: (CMCC, Xiaomi) - The event triggered measurement reporting delay shall be less than Tmeasure, excluding a delay which is caused by no UL resources being available for UE to send the measurement report on and L3 filtering. - Option 3: (Nokia) - The UE shall be able to send the first PUCCH latest at the PUCCH occasion that is available after the maximum L1 event evaluation delay (TL1-RSRP_measure_period).

There is no much difference between option 1 and option 2. Option 1 tries to specify the detail component and consider the impact of UL resource waiting delay. however, it’s not clear what’s the definition of Tfirst UL channel and it’s also related to issue 1-7. Different UE implementation will have impact on the starting point of Tfirst UL channel. RAN4 needs to discuss which RS will be used as reference for the starting point of Tfirst UL channel. If UE behavior is clear, both options are fine to us.

Suppose that UE periodically performs L1-RSRP at RS1, RS2, …RS3. UE detect channel conditions drops at RS2 which is lower than the threshold and L1 reporting will be triggered at RS2. UE will obtain new L1 result at RS3 before PUCCH.

There are two possible ways to define the starting point of Tfirst UL channel.

Option 1: L1-RSRP RS which trigger L1 reporting will be used as starting point for Tfirst UL channel, as shown in Fig.1.

Fig.1 RS triggering reporting as starting point for Tfirst UL channel

There are two possibilities. After L1 reporting is triggered(RS2), new L1 result at RS3 can be higher than threshold or lower than threshold. If UE doesn’t consider the impact of new L1 result(RS3), the starting point of Tfirst UL channel will be the RS which trigger L1 reporting(RS2).

Observation 1: If UE doesn’t consider the impact of new L1 result after L1 reporting is triggered, the definition of Tfirst UL channel will be the time interval from L1-RSRP RS which trigger L1 reporting to first PUCCH.

Case 2: UE consider new L1 result after L1 report is triggered

The total reporting delay is defined as:

TL1-RSRP_measure_period + TL1-RSRP measure after triggering + Tfirst UL channel.

The total delay will include 3 parts:

- TL1-RSRP_measure_period, which is L1-RSRP measurement period.

- TL1-RSRP measure after triggering, which will be the time interval from L1-RSRP RS which triggers L1 reporting to L1-RSRP RS which is nearest to first PUCCH.

- Tfirst UL channel will be the time interval from L1-RSRP RS nearest to first PUCCH to first PUCCH.

For this case, UE will consider the impact of new L1 result after L1 triggering, the starting point of Tfirst UL channel will be the latest RS which is closest to first PUCCH. As shown in Fig.2, suppose that RS3 are closest to PUCCH, the starting point of Tfirst UL channel will be from RS3 but not RS2. The time interval from RS2 to RS3 will also need to be considered in total report delay.

Fig.2 RS nearest to PUCCH as starting point for Tfirst UL channel

Observation 2: If UE consider the impact of new L1 result after L1 report is triggered, starting point of Tfirst UL channel will be L1-RSRP RS which is nearest to first PUCCH. The time interval from L1-RSRP RS which triggers L1 reporting to L1-RSRP RS which is nearest to first PUCCH shall also be considered.

Besides, there are also L1 results, e.g. RS4, RS4, between PUCCH and PUSCH, as shown in Fig.3. If UE consider the impact of new L1 results after triggering, there are also several possibilities and the situation is too complex.

Fig.3

Observation 3: If UE consider the impact of new L1 results, there are new L1 results, e.g. RS4, RS4, between PUCCH and PUSCH. There are also several possibilities and the situation is too complex.

From our point of view, for simplicity, it’s better to re-use legacy L3 reporting method, i.e. only consider the result for RS which triggers the L1 reporting.

Proposal 1: For L1 reporting delay, didn’t consider the impact of L1 result after L1 reporting is triggered.

Proposal 2: Fine with both Option 1 and Option 2. For Option 1, the definition of Tfirst UL channel will be the time interval from L1-RSRP RS which trigger L1 reporting to first PUCCH.

For this issue, RAN1 also has some similar discussion. For option -1, the PUCCH is used as anchor and RS close to PUCCH will be used as reference for evaluation. the evaluation time is defined from the ending point, i.e. PUCCH, and then forward. For some other options, the evaluation time is defined from the starting point, i.e. channel changes at air, and afterward, similar as what we have defined before.

Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured. - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s). - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement. - Note: Other options are not precluded.

Observation 4: RAN1 is also discussing the evaluation time and will have impact on RAN4 delay requirement.

2.2 Measurement time

Issue 1-3: Same or different periodicities of current beam and new beam(s)? <Way forward > - Option 1: (Qualcomm, Apple, CMCC, Samsung, vivo) - The same periodicities of current beam and new beam(s) to define the requirements, until further RAN1 agreements on different periodicities. - According to RAN1 LS (R1-2410914) - Option 2: (Huawei) - From RAN4 perspective, the periodicity of current beam and new beam(s) can be the same or different which is up to network configurations. Send LS to RAN1 - Option 3: (vivo) - RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

RAN1 agreements are as below:

RAN1#119 Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. RAN1#120 Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.

From RAN1 agreement, for event-2, for single CC case, the periodicity of the current beam RS should be the same as that of the new beam RS(s). For cross-cc case, the current beam RS and new beam RS(s) are in the same CC, therefore, the periodicity of current beam and new beam are the same either.

Proposal 3: For event-2, apply the same measurement periodicities of current beam and new beam(s) to define the requirements.

Issue 1-6: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with configurable M? <Way forward > - Option 1: (Qualcomm, Apple, Ericsson) - Only define requirements for the number of events is met at once - Option 2: (China Telecom, ZTE, Huawei, Samsung, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable M - Option 2a: (ZTE) - Event1&Event 2: Case 2: multiple samples M and timer X should be considered - Event1&Event 7: FFS - Option 2b (Samsung) - For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event. - Option 2c (MediaTek) - For multiple events is met by counter: - Tevaluate_L1_event = TL1-RSRP_measure * C + Tuncertainty given that Tevaluate_L1_event ≤ Ttime_window - TL1-RSRP_measure: L1-RSRP measurement delay for either SSB or CSI-RS - C: event instance counter is configured by NW.

From our understanding, the issue is similar as whether to consider TTT in legacy event triggered L3 reporting. for L3, the minimum configurable value of TTT can be 0, therefore, the requirement is defined without considering the impact of TTT. Similar, for L1 event triggered reporting, the minimum configurable value of M can be adopted, e.g. M=1. We don’t think that there is case that there is no time window. It just means that the minimum window length is 1. We support to define a unified requirement with M=1 for all cases.

Proposal 4: Define a unified requirement with M=1 to cover all cases.

2.5 Reduction of beam application delay

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception. - Other alternatives are not precluded.

In last meeting, RAN1 agrees to further study the reduction of beam application delay, the majority options will skip extra SSB reception for synchronization. This part may have impact on the MAC CE based TCI activation delay if target TCI state is not in the active TCI state list. RAN4 may need to consider the impact according to RAN1 progress.

Proposal 5: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: If UE doesn’t consider the impact of new L1 result after L1 reporting is triggered, the definition of Tfirst UL channel will be the time interval from L1-RSRP RS which trigger L1 reporting to first PUCCH.

Observation 2: If UE consider the impact of new L1 result after L1 report is triggered, starting point of Tfirst UL channel will be L1-RSRP RS which is nearest to first PUCCH. The time interval from L1-RSRP RS which triggers L1 reporting to L1-RSRP RS which is nearest to first PUCCH shall also be considered.

Observation 3: If UE consider the impact of new L1 results, there are new L1 results, e.g. RS4, RS4, between PUCCH and PUSCH. There are also several possibilities and the situation is too complex.

Proposal 1: For L1 reporting delay, didn’t consider the impact of L1 result after L1 reporting is triggered.

Proposal 2: Fine with both Option 1 and Option 2. For Option 1, the definition of Tfirst UL channel will be the time interval from L1-RSRP RS which trigger L1 reporting to first PUCCH.

Observation 4: RAN1 is also discussing the evaluation time and will have impact on RAN4 delay requirement.

Proposal 3: For event-2, apply the same measurement periodicities of current beam and new beam(s) to define the requirements.

Proposal 4: Define a unified requirement with M=1 to cover all cases.

Proposal 5: Reduction of beam application delay may have impact on TCI state activation delay, RAN4 shall wait for more RAN1 progress.

- Reference

[1] R1-2410913, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management, ZTE