3GPP TSG-RAN WG4 Meeting #114bis				                              R4-2503323

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.10.3.4

Title: 	Discussion on RRM requirements for less than 5MHz NTN

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for enhanced requirements and test methodology for NR and IoT NTN was agreed and one of objectives for less than 5MHz for NTN is self-contained as below.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

And in the last RAN4 meeting, the agreements for less than 5MHz for NTN was achieved in [2]. Therefore, in this discussion we provided further considerations on the open issues for less than 5MHz NTN.

- Discussion

Up to RAN4#113, the main open issues for these RRM requirements are [2]

- requirements for SSB time index detection

Issue 1-1-2: Measurement(SSB index detection requirements) Agreements: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by [3 or 4] and [3] for intra-frequency and inter-frequency cases respectively. Notes: the final value can be confirmed in the next meeting and the corresponding value will be updated in the draftCR.

Based on the previous simulation results for TN less than 5MHz [5], the minimum measurement samples needed for a successful SSB index detection for TN less than 5MHz was agreed as blow[6]:

SSB Index reading and MIB reading Agreement: - SSB index identification delay - Unknown intra-frequency cell (Es/Iot≥-6 dB): [3+4] samples - Unknown inter-frequency cell (Es/Iot≥-4 dB): [3+3] samples

And as we agreed that the necessary requirements for NTN less than 5MHz can leverage from the conclusion from TN less than 5Mhz, especially regarding no any further simulation results available we can suggest that:

Issue 1-2-1: Baseline requirement which can be taken as the start point for NTN less than 5MHz requirements Agreements: - RRM requirement for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz requirement

Proposal 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by [4] and [3] more samples for intra-frequency and inter-frequency cases respectively.

- Conclusion

In this contribution, serval issues related to the RRM requirements under less than 5MHz NTN in Rel19 are discussed. The observations and proposals can be summarized as:

Proposal 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by [4] and [3] more samples for intra-frequency and inter-frequency cases respectively.

- References

- RP-240857

- R4-2502589

- TS38.133 v18.7.0

- TS38.211 v18.3.0

- R4-2321344

- R4-2321629