3GPP TSG-RAN WG4 Meeting #114bis									R4-2503333

Wuhan, China, 7th – 11th April, 2025

Agenda item:	7.1.1.3.4

Source: 	Xiaomi

Title: 	Discussion on SRS IL imbalance for 6Rx UE

Document for:	Discussion

Introduction

In RAN4#114 meeting, RAN4 discussed the issue of power imbalance for SRS insertion loss for 6Rx UE, and the related WF was approved in [1]. This contribution discusses the remaining issues and provides our proposals.

Sub-topic 3-1: General considerations for SRS IL imbalance issue Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1. Issue 3-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1. Sub-topic 3-2: SRS IL imbalance issue solutions Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: RAN4 to take the candidate solution framework below as baseline for further discussion. Candidate solution framework: - Rel-19 UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on RAN4 specification impacts. - No RAN1 specification impact. - FFS if UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others

Discussion

In Rel-18, RAN4 has had extensively discussion on whether to introduce reporting mechanism for SRS IL compensation in 8Rx topic. After the whole release debate, no progress was made, since the proposals from companies are quite diverse and strong concern on the benefit of introducing such reporting. In Rel-19, the diverse proposals of SRS IL compensation were resubmission in 6Rx topic.

From implementation perspective, the UE can perform self-compensation of SRS IL up to its configured maximum output power which it works in most of cases. And in far-cell scenario, the SRS transmission to obtain accurate CSI for DL-MIMO may not be beneficial, thus, it is questionable to do the compensation for SRS IL. In addition, according to the long-debate in RAN4 and RAN1, it is questionable that the potential reporting solution for SRS IL imbalance reporting would not help improving accuracy of the downlink channel estimation at network side. And from OTA perspective, it is not the critical factor to impact SRS Tx power imbalance at the network side because different antenna radiation pattern and/or channel condition may also impact even if the SRS IL is well compensated. Based on above consideration, we support RAN4 not to continue the discussion on how to solve the SRS IL imbalance issue, and up to UE self-compensate the power imbalance of SRS IL.

Proposal 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

Proposal 2: In Rel-19, the UE performs self-compensation of SRS IL up to its configured maximum output power.

Conclusion

In this contribution, we discussed the issue of power imbalance for SRS insertion loss for 6Rx UE and provides our proposals:

Proposal 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

Proposal 2: In Rel-19, the UE performs self-compensation of SRS IL up to its configured maximum output power.

- Reference

[1]	R4-2503014	WF on 6Rx requirements		AT&T