# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503334

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.11.3

Source: 	Xiaomi

Title: 	System parameters for Ku band

Document for:	Discussion

1		Introduction

For Rel-19 NTN Ku Band introduction, the suitable numerologies e.g., FR1 like or FR2 like are key issue. During previous RAN4 discussion, companies realized both FR1 numerologies and FR2 numerologies have pros and cons in terms of spectrum usage efficiency, UL timing synchronization, phase noise, beam hopping granularity and link budget UL/DL.  It’s hard to make decision on numerologies selection based on above decision criteria. Both FR1 and FR2 numerologies are feasible for the deployment.

In RAN4#113, RAN4 failed to meet the RAN-P demand to make down-selection between FR1 like or FR2 like numerologies due to the diverse demand from NTN operators for the deployed bandwidths with the range from 30MHz to 250MHz which are across FR1 channel bandwidth set and FR2 channel bandwidth set. RAN4 make following recommendation to RAN-P in WF [2], with corresponding LS to RAN1/RAN2 cc RAN-P in [3] for the potential impact to RAN1/RAN2:

# WF for NTN Ku band System parameters - Sub-topic 3-9: FR1 FR2 down selection Overall observation is that both FR1 numerology and FR2 numerology are feasible. There are use cases that favor either FR1 numerology or FR2 numerology Some of the disadvantages of numerology combinations can be mitigated with implementation choices WF on numerology: Agree to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. RAN4 will further discuss SCS options.

In RAN#106, the situation maintained same as RAN4 discussion with different use cases and different demand from satellite industry on Ku band usage which captured in WF [4]. It’s agreed to revise the WID including both numerologies with two sets of bands meanwhile also pending on the feedback from RAN1/RAN2 with check point in RAN#107:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

In RAN#107, WF on check point were agreed that:

- Two sets of bands with FR1 numerologies or FR2 numerologies confirmed based on the response LS from RAN1/RAN2

- In additional to 30kHz SCS, 15kHz SCS was added into WID which target for GSO and low mobility scenario

The update WID with major changes as following:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only. ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The objective of the performance part is to update as appropriate for Ku band: - RRM performance requirements and test cases - UE demodulation and CSI reporting requirements - For 15 kHz subcarrier spacing, the requirements are only applicable for GSO and low mobility scenarios. - Satellite access node demodulation requirements - Satellite access node conformance tests

On the band definition, in last RAN4 meeting, following bands were introduced:

- Sub-topic 1-2: Definition of extended FR1-NTN and FR2-NTN frequency ranges Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN (Note 1); 410 MHz – 14500 MHz. FR2-NTN (Note 2); 10700 MHz – 30000 MHz. NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.. - Sub-topic 1-3: Ku bands definition Table 5.2.3-1: Satellite operating bands in FR2-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n509; 14000 MHz - 14500 MHz; 10700 MHz – 127504 MHz; FDD. n508; 13750 MHz - 14000 MHz; 10700 MHz – 127504 MHz; FDD. NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT... Table 5.2.3-2: Satellite operating bands in FR1-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n248; 14000 MHz - 14500 MHz; 10700 MHz – 127501 MHz; FDD. n247; 13750 MHz - 14000 MHz; 10700 MHz – 127501 MHz; FDD. NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT...

In this contribution, we discussed the system parameters for both FR1 and FR2 numerologies with two sets of bands following RAN-P decision.

2		Discussion

Band definition and frequency range

In existing RAN1/RAN2 specifications, there are many places refer to the reference as FR1 and FR2-1 bands, or TDD-FR1 bands, TDD-FR2-1 bands and TDD-FR2-2.  In order to avoid the impact to other WGs (RAN1 and RAN2), RAN4 shall introduce Ku bands in existing frequency range designation instead of introducing new frequency ranges. In order to cover Ku bands without introducing new frequency ranges, the frequency range of existing FR1-NTN and FR2-NTN need to be extended with overlapping frequency ranges. To avoid confusion and impact to other WGs on these overlapping frequency between two frequency range designation, additional notes need to be added to clarify NTN bands with band numbering below or equal to 256 treated as FR1 band, and NTN bands with band numbering between 257 to 512 as FR2 bands.

Proposal 1: Clarify NTN bands with band numbering below or equal to 256 treated as FR1 band, and NTN bands with band numbering between 257 to 512 as FR2 bands. (Detailed proposed changes as following)

Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range with band numbering below or equal to 256 are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range with band numbering from 257 to 512 are regarded as a FR2-1 band when references from other specifications. |  |

CHBW, SCS and spectrum utilization

According to last RAN-P decision, 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing were introduced in additional to 30kHz SCS for FR1-NTN Ku bands.

Proposal 2: On detailed channel bandwidth sets including spectrum utilization, guard band reusing existing values from TN specifications for following channel bandwidths under 15kHz SCS

- FR1 NTN Ku Band(s)- 15kHz: 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths

Sync raster and channel raster

In additional to 30kHz channel raster, 15kHz channel raster also need to be included into FR1-NTN Ku bands.

Proposal 3: For FR1 NTN Ku Band(s), 15kHz channel raster needs to be added in additional to 30kHz channel raster with NR-ARFCN as following:

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316668 – <2> – 1333333 | 1113334 – <2> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

On sync raster, 15kHz SSB also needs to be introduced as RAN-P agreed same numerology shall be applied for SSB and data.

With both 15kHz SSB and 30kHz SSB introduced on the same band(s), this will further increase UE initial cell search complexity with more candidate SSB patterns and sync raster entries.

Considering minimum channel bandwidth on FR1-NTN Ku band(s) for 15kHz SCS is 10MHz. To reduce UE initial cell search complexity, we propose to introduce 15kHz SSB with sync raster step size as 4.

Proposal 4: For FR1 NTN Ku Band(s), 15kHz SSB is applied for FR1 NTN Ku Bands  with GSCN step size as 4 .

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12848 – <4> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15kHz | Case A | 12848 – <4> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |

3		Conclusion

In this contribution, views on system parameters for Ku band were provided.

Proposal 1: Clarify NTN bands with band numbering below or equal to 256 treated as FR1 band, and NTN bands with band numbering between 257 to 512 as FR2 bands. (Detailed proposed changes as following)

Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range with band numbering below or equal to 256 are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range with band numbering from 257 to 512 are regarded as a FR2-1 band when references from other specifications. |  |

Proposal 2: On detailed channel bandwidth sets including spectrum utilization, guard band reusing existing values from TN specifications for following channel bandwidths under 15kHz SCS

- FR1 NTN Ku Band(s)- 15kHz: 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths

Proposal 3: For FR1 NTN Ku Band(s), 15kHz channel raster needs to be added in additional to 30kHz channel raster with NR-ARFCN as following:

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 15 | 1316668 – <2> – 1333333 | 1113334 – <2> – 1250000 |
|  | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

Proposal 4: For FR1 NTN Ku Band(s), 15kHz SSB is applied for FR1 NTN Ku Bands  with GSCN step size as 4 .

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 15 kHz | Case A | 12848 – <4> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 15kHz | Case A | 12848 – <4> – 14268 |
|  | 30 kHz | Case C | 12850 – <1> – 14266 |

Reference

[1] RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

[2] R4-2419909, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

[3] R4-2419902, LS on Ku band numerology, Eutelsat, TO: RAN1, RAN2, CC: RAN-P

[4] RP-243301, Way Forward on Ku band numerology, Intelsat, Thales, Eutelsat Group, European Space Agency, FirstNet, Bosch, Novamint, SyncTechno Inc, Fraunhofer HHI, Fraunhofer IIS, BMW, Gilat, CHTTL, SES, Airbus, EDF, Sharp, Hispasat, Lockheed Martin, UIC

[5] R4-2502299, Way Forward for [114][313] NR_NTN_Ku_Band_General, Intelsat